3GPP TSG RAN Meeting #99	RP-230682

Rotterdam, Netherlands, March 20-23, 2023	(revision of RP-yyxxxx)

Agenda Item:	9.2.4

Source: 		T-Mobile USA

Title: 	Motivation for New WID on Introduction of 6 and 7 MHz channel bandwidths

Document for:	Discussion

- Introduction

In September 2019, Sprint, AT&T and Dish Network proposed the creation of NR band n26, including a 7 MHz channel bandwidth for n26 [1]. The request to include a new 7 MHz channel bandwidth received a lot of push-back from vendors who felt that adding any new channel bandwidths would result in a flood of requests for new channel bandwidths. As a result, 7 MHz was dropped from the approved version of the WID [2].

After further discussions in RAN, a new Study Item was created: “Study on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths.” [3]

The Study Item considered several techniques for handling irregular channel bandwidths, and subsequent work is being proposed in Work Item to follow up the Study Item [4]. While we support that new Work Item, the SI TR shows that these techniques would not be efficient for 6 and 7 MHz channel bandwidths due to the need for separate SSBs in offset time slots [5]. As a result, we are proposing a parallel follow-on Work Item that creates 6 and 7 MHz channel bandwidths [6].

- Discussion

The Study Item “Study on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths” (FS_NR_eff_BW_util) studied several techniques for efficiently utilizing spectrum that is not allocated in increments of 5 MHz. While the overlapping carrier techniques are practical for channel bandwidths above 8 MHz, they are not efficient for 6 and 7 MHz channel bandwidths because a single SSB cannot handle the offset carrier and two SSBs will not fit in the same slot as described in the SI TR 38.844. From TR 38.844:

Table 6.2.1.2-3: Summary of how many SSB/CORESET#0 needed for different channel bandwidths.

| Channel (MHz) | Number of SSB/CORESET#0 | Time multiplexing for 2 SSBs is needed |
| --- | --- | --- |
| 6 | 2 | yes |
| 7 | 2 | yes |
| 11 | 1 or 2 | no |
| 12 | 1 or 2 | no |
| 13 | 1 or 2 | no |

38.844 also contains the following note in reference to the use of time multiplexed SSBs:

NOTE:	It may create the challenge of aligning SSBs in the time and frequency domain so that they do not overlap thus complicating the gNB scheduling, i.e. for this approach complexity increases to coordinate the overlapping SSBs for different UEs.

38.844 also says:

For scenarios with less than 10 MHz, second SSB is not excluded but not recommended due to significant additional overhead (duplicated SSB transmission as well as other radio resources such as PDCCH, CSI-RS, PDSCH (for SIB), CSI for Tracking, etc.). However, second SSB could enable or improve the legacy UEs' use of the further PRBs provided by the additional RF carrier in spectrum scenarios with less than 10 MHz.

If multiple time slots were used for SSBs for 6 and 7 MHz channel BWs, the additional overhead would nearly offset the benefit of the external bandwidth above 5 MHz. Since these are critical coverage bands, it is essential to efficiently utilize the spectrum. It is also not clear if time offset SSBs would result in additional problems that would further delay a solution. Also, the PRB blanking technique might be useful for capacity bands, but because of the danger of blocking caused by a neighbour network in the blanked PRBs, it would not be appropriate

Therefore, as a follow on to the FS_NR_eff_BW_util SI, it is important that in addition to a Work Item to standardize the techniques studied in the SI, 6 MHz channel bandwidth should be introduced for n12 and n85, and 7 MHz channel bandwidth should be introduced for n5 and n26 [6]. Wider irregular channel bandwidths can take advantage of one or more of the techniques studied in the Study Item and proposed in the other follow-on Work Item [4].

The 6 and 7 MHz WI is not intended to be a basket WI, but example bands are included. Since n5 is a subset of n26 and n12 is a subset of n85, 7 MHz for n5 and n26 and 6 MHz for n85 and n12 are included.

Proposal: Approve two follow-on Work Items for the Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidth: one for adding 6 and 7 MHz channel bandwidths [6], and one for specifying the requirements for the techniques studied in the Study Item [4].

- Conclusion

Proposal: Approve two follow-on Work Items for the Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidth: one for adding 6 and 7 MHz channel bandwidths [6], and one for specifying the requirements for the techniques studied in the Study Item [4].

# 4.	References

- RP-191873, “New WID: Introduction of NR Band n26,” Sprint, AT&T, Dish Network

- RP-192273, “New WID on Introduction of NR Band n26,” Sprint, AT&T, Dish Network

- RP-222618, “Revised SID: Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths,” T-Mobile, Ericsson

- RP-230494, “New WID: Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths,” Ericsson

- TR 38.844, “Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidth”

- RP-230683, “New WID for 6 and 7 MHz channel bandwidths.” T-Mobile USA