3GPP TSG RAN Meeting #99	RP-230795

Rotterdam, Netherlands, 20-23 March, 2023

Agenda Item:	9.2.3

Source:	Huawei (Rapporteur)

Title:	Results of offline discussions on ambient IoT

Document for:	Discussion and Decision

Design targets

Proposal 6-1 (offline consensus): Agree to set at least the design targets below in Ambient IoT in the RAN SI.

- Device power consumption

- Device complexity

- Coverage

- Data rate

- Maximum message size (or maximum ‘TB’ size)

- Latency

- Positioning accuracy

- Connection/device density

- Device speed (FFS absolute or relative or both)

Deployment scenarios

Proposal 3.5 (offline consensus): The following 5 deployment scenarios are studied and captured in the TR:

- Note1: all the detailed values (with or without square brackets) in the tables are to be further discussed

- Note2: the location of assisting or intermediate node, where applicable for the corresponding topologies, would need to be defined additionally.

Deployment 1: Device indoors, base station indoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico- cell BS |
|  | Connectivity topology | Topology (1),[(2)],[(3)] |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Deployment 2: Device indoors, base station outdoors

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory indoor sensor indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2) |
|  | Spectrum | Licensed FDD, or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 3: Device indoors, UE based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| indoor inventory [indoor sensor] indoor positioning indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD, Licensed TDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | [Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

Deployment 4: Device outdoors, Outdoor base station

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory outdoor sensor outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS |
|  | Connectivity topology | Topology (1),(2),(3) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device [A or B] or C |

Deployment 5: Device outdoors, UE based reader

| Applicable representative use cases | Characteristics | Description |
| --- | --- | --- |
| outdoor inventory [outdoor sensor] outdoor positioning outdoor command | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Licensed TDD |
|  | Coexistence with existing 3GPP technologies | [Legacy UE or new UE] |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or [C] |

## Wednesday 08.00 – 09.00

Design targets

Proposal 6-3a-v2 (Wed offline consensus): Device design target for power consumption during transmitting/receiving is:

- [Device A ≤ 10 μW] or [Device A ≤ 1 μW]

- Device A ≪ Device B < Device C, or Device A ≤ Device B < Device C

- Device C ≤ 1 mW to ≤ 10 mW

Proposed working assumption (Wed offline consensus): Device complexity design target is:

- Device A: Comparable to UHF RFID ISO18000-6C (EPC C1G2)

- Device A < Device B < Device C

- Device C: Orders-of-magnitude lower than NB-IoT

Proposal 6-3d-v2 (Wed offline consensus): User experienced data rate design target is, at least for the uplink:

- Maximum  not less than: 5 kbps

- Minimum  not less than 0.1 kbps

Deployment scenarios

Proposal: Add device characteristics into the deployment scenario table, with possible values of ‘Device A’, ‘Device B’, ‘Device C’.

Proposal: All rows in the deployment scenario tables can be indicated with more than one value.

Connectivity topologies

Proposal: Do not include Topology (5) in the study.

- Note: it is understood this topology can be realized via combinations of other topologies