3GPP TSG RAN WG1 Meeting #100bis-e	R1-2001557

E-meeting, April 20 – April 30, 2020

Agenda Item:	7.2.4.7

Source:	Huawei, HiSilicon

Title:	Remaining details of NR Uu control for LTE sidelink

Document for:	Discussion and Decision

## Introduction

During RAN1#100-e meeting, the following agreements were achieved regarding NR Uu control of LTE sidelink shown after inclusion of the revision marks in the chairman’s notes:

Agreements:

         If UE is configured to monitor DCI 3_0, the sizes of DCI 3_1 and DCI 3_0 are is aligned by zero padding.

         If UE is not configured to monitor DCI 3_0, the mechanism for size alignment between DCI 3_0 and Uu DCI is reused for size alignment between DCI 3_1 and Uu DCI

This contribution discusses the size alignment issue together with the agreements achieved for NR Uu control of NR SL in mode 1.

## Discussion on the size alignment

The following agreements were achieved in RAN1#99 for AI mode 1 [1], also shown after inclusion of the revision marks in the chairman’s notes:

Agreements:

- Existing DCI size budget is maintained when the UE is configured with SL

- (working assumption): The size of the new DCI format and the size of one of the existing NR DCI formats are aligned.

Agreements:

- Use a separate PDCCH monitoring configuration (as configured in Rel-15) for DCI scheduling SL

  - The per-CC and across-CC blind decoding budget and the maximum number of non-overlapped CCEs for channel estimation are not increased.

  - The per-CC and across-CC maximum number of search spaces is not increased.

- The per-CC and across-CC maximum number of CORESETs is not increased

  - When in the same slot, there is both PDCCH monitoring for Uu and PDCCH monitoring for SL for the same CC, the search space(s) for SL is configured to be the same or a subset of those for Uu for the same CC or vice versa

    - A UE does not expect to receive in the same PDCCH monitoring occasion and the same scheduling cell PDCCH carrying a DL grant and PDCCH carrying a SL DG.

      - FFS SL grant is limited to having SL HARQ FB enabled or not.

      - FFS for CG type-2 activation/deactivation

According to this agreement, on a given scheduling cell and a given PDCCH monitoring occasion, either PDCCH carrying a DL grant or PDCCH carrying a SL DG is configured. It is understood the intention for this agreement was not to align DCI format 3_0 to other DCI format size in NR Uu, and leave it up to network configuration to ensure the blind decoding not to exceed the maximum number of monitored PDCCH candidates defined per slot for a DL BWP for a serving cell.

Taking the RAN1#100-e agreement that the method for alignment of 3_0 to Uu is reused for 3_1 alignment to Uu, it is therefore also agreed that the gNB will be assumed to manage the blind decoding budget relating to format 3_1.

Thus, when UE is not configured to monitor DCI format 3_0, the blind decoding budget is managed by network through proper configuration, so UE does not expect to receive the same scheduling cell PDCCH carrying a DL grant and PDCCH carrying DCI format 3_1 in the same PDCCH monitoring occasion.

Proposal 1: Since size alignment of format 3_0 to a Uu DCI size is agreed to be not specified, size alignment of format 3_1 to a Uu DCI (when 3_0 is not configured) has also been agreed to be not specified.

- This means a UE does not expect to receive in the same PDCCH monitoring occasion and the same scheduling cell PDCCH carrying a DL grant and PDCCH carrying DCI format 3_1.

# Conclusions

This contribution analyzed the agreements made for NR control of NR SL and LTE SL, which leads to the following proposal:

Proposal 1: Since size alignment of format 3_0 to a Uu DCI size is agreed to be not specified, size alignment of format 3_1 to a Uu DCI (when 3_0 is not configured) has also been agreed to be not specified.

- This means a UE does not expect to receive in the same PDCCH monitoring occasion and the same scheduling cell PDCCH carrying a DL grant and PDCCH carrying DCI format 3_1.

# References

Chair notes in RAN1#99, Reno, USA, November 18-22, 2019.