3GPP TSG RAN WG1 #100bis                                                     R1- 2001824

e-Meeting, April 20th – 30th, 2020

Source: MediaTek Inc.

Title: Remaining issues on UL full power transmission

Agenda Item: 7.2.6.4

Document for:  Discussion

# Introduction

In this contribution, we discuss remaining issues of UL full power transmission based on V16.1.0 specifications [1]-[3].

# Scenario of ULFPTx and ULFPTxModes setting

The ULFPTx and ULFPTxModes are mentioned in specification [1]-[3] for supporting UL full power transmission. However, it is not clear in the specifications if a coherent UE can be configured ULFPTx and ULFPTxModes or not. In other words, if the on-going UE feature (FG16-5) is tied to a non-coherent/partial-coherent UE?

TS 38.213 specifies new power scaling rule for nonCoherent or partialAndNonCoherent (as shown below). TS 38.214 specifies the new features for ULFPTx mode. We think it is important that RAN1 can clarify if the ULFPTx mode (and all associated new features) can apply to coherent UE or not. Some editorial changes might be required after the clarification.

TS 38.213:

## 7.1	Physical uplink shared channel

For a PUSCH transmission on active UL BWP , as described in Clause 12, of carrier  of serving cell , a UE first calculates a linear value  of the transmit power , with parameters as defined in Clause 7.1.1. For a PUSCH transmission scheduled by a DCI format or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook',

-	if ULFPTx in PUSCH-Config is provided and codebookSubset in PUSCH-Config is set to nonCoherent or partialAndNonCoherent, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where:

-	if ULFPTxModes in PUSCH-Config is set to Mode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port', s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource

<Remaining parts are omitted>

Proposal 1: clarify if ULFPTx and ULFPTxModes can be configured to coherent UE or not.

# References

[1] 3GPP TS 38.212 V16.1.0

[2] 3GPP TS 38.213 V16.1.0

[3] 3GPP TS 38.214 V16.1.0