3GPP TSG-RAN WG1 Meeting #100bis	Tdoc R1-2002280

e-Meeting, April 20th – 30th 2020

Agenda Item:	7.2.11.12

Source:	Ericsson

Title:	UE features for TEI

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss UE features for TEI and the discussion is based on the features as listed in R1-2001484[1]. The following proposals for feature groups were added in this category:

| Features | Index | Feature group |
| --- | --- | --- |
| 14. NR TEI | 14-1 | Multiple LTE-CRS rate matching patterns |
|  | [14-1a] | Multiple LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier |
|  | 14-2 | PDSCH Type B mapping of length 9 and 10 OFDM symbols |
|  | 14-3 | One slot periodic TRS configuration for FR1 |
|  | 14-4 | SRS Tx switch with allowing downgrading configuration |
|  | 14-5 | Half-duplex UE behaviour in TDD CA |
|  | [14-5a] | Half-duplex UE behaviour in TDD CA with different SCS |
|  | 14-6 | New RACH configuration for FR1 TDD |
|  | [14-7] | New capability for beamSwitchTiming values of 224 and 336 |

# 2	Discussion

In the following, Ericsson’s comments can be found on the proposed feature groups.

14-1	Multiple LTE-CRS rate matching patterns

- The FSS proposal to add “Up to 3 LTE-CRS non-overlapping rate matching patterns within a NR carrier” is ok

[14-1a]	Multiple LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier

- There is no need for this FG since 14-1 al

- ready captured 2 to 6 patterns and at most 3 non-overlapping as captured in the basic functionality (if “Up to 3 LTE-CRS non-overlapping rate matching patterns within a NR carrier” is added in 14-1)

14-2	PDSCH Type B mapping of length 9 and 10 OFDM symbols

- FDD/TDD differentiation should be “No”

  - Note that the main use case and motivation is for TDD and 30 kHz subcarrier spacing in which case the UE is not aware that DSS is in operation. So FDD/TDD differentiation should be No, and “DSS” in the note should be interpreted as for information only. For example, it must be ensured that the RAN1 interpretation is that this feature is not restricted to be dependent that LTE-CRS rate matching pattern is configured (i.e. FDD,15 kHz).

- The signalling should be per UE

- The feature group should be mandatory with capability signalling

14-5	Half-duplex UE behaviour in TDD CA

and

[14-5a]	Half-duplex UE behaviour in TDD CA with different SCS

- If we consider separating the UE capability for same and different SCS, then each capability shall be a “Per UE” capability. If the capability 14-5 applies to “Per Band Combination”, then there’s no need to add 14-5a.

[14-7]	New capability for beamSwitchTiming values of 224 and 336

- Ericsson is supportive of the new feature.

- The following component should be added

3) 	Support for scheduling aperiodic CSI-RS for beam management with an offset smaller than the beam switching threshold.

# 3	Conclusion

In this contribution we discussed the UE features for TEI.

# References

R1-2001484, RAN1 UE features list for Rel-16 NR after RAN1#100-E, AT&T, NTT DOCOMO, INC.