3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002604

E-meeting, April 20 – 30, 2020

Agenda Item:	6.2.5.1

Source:	Huawei, HiSilicon

Title:	Rel-16 UE features for LTE-MTC

Document for:	Discussion and Decision

Introduction

In this contribution, our views on UE features for LTE-MTC [1] is provided.

Discussion

UE-group wake-up signal

The motivation for introducing group resource alternation is to achieve fairness among UEs. Without it, some UE will always have higher false alarm probability and thus higher power consumption, which is against the motivation of introduction of wake up signals. Thus we think it should not be a separate feature group of UE group WUS.

Proposal 1: FG 1-2 (Group resource alternation) is a component in feature 1-1.

Transmission in preconfigured UL resources (PUR)

Serving cell RSRP for TA validation is critical for PUR procedures. Without serving cell RSRP for TA validation, it’s not accurate enough for UE to validate TA. And with a valid but inaccurate TA, there will be interference. It will cause a waste of uplink resource and thus increase UE’s power consumption. Therefore, it should be mandatory for PUR feature group.

Proposal 2: FG 1-7 (Serving cell RSRP for TA validation) is a component of FG 1-3/1-4/1-5/1-6.

L1 ACK can effectively reduce UE power consumption which is aligned with the objective of PUR. If L1 ACK is not supported, the network can only send higher layer feedback (i.e. RRC signalling) and UE needs to send ACK accordingly. Thus it will consume more DL and UL resource and increase UE’s power consumption.

Proposal 3: FG 1-9 (L1 ACK) is a component of FG 1-3/1-4/1-5/1-6.

Multi-TB scheduling for unicast

As it has been concluded in RAN1#99 that scheduling gaps is to be realized by resource reservation [1], the “scheduling gaps” item in feature 1-20 is not needed.

RAN1 #99 Conclusion For unicast, aim to realize the scheduling gaps using the Rel-16 LTE-MTC WI feature for improved LTE-MTC resource reservation

Proposal 4: The FG 1-20 (scheduling gaps) is not needed.

Resource reservation

The main purpose of resource reservation for downlink is to avoid collision with SSB. For uplink it is to avoid collision with URLLC. For SSB and URLLC transmission, typically only one slot or several symbols are used in a subframe. Slot level or symbol level resource reservation is more resource efficient than subframe level.

Proposal 5：Separate indication to slot/symbol level resource reservation in feature groups from 1-23 to 1-26 is not supported.

Conclusions

In this contribution, our views on UE features for LTE-MTC is provided. The following proposals are made.

Proposal 1: FG 1-2 (Group resource alternation) is a component in feature 1-1.

Proposal 2: FG 1-7 (Serving cell RSRP for TA validation) is a component of FG 1-3/1-4/1-5/1-6.

Proposal 3: FG 1-9 (L1 ACK) is a component of FG 1-3/1-4/1-5/1-6.

Proposal 6: The FG 1-20 (scheduling gaps) is not needed.

Proposal 5：Separate indication to slot/symbol level resource reservation in feature groups from 1-23 to 1-26 is not supported.

- References

R1-2001485, RAN1 UE features list for Rel-16 LTE after RAN1#100-E, AT&T, NTT DOCoMO, E-Meeting, Feb. 24th – Mar. 6th, 2020.