3GPP TSG-RAN WG1 Meeting #100b-e	R1-2002624

e-Meeting, April 20th – April 30th , 2020

Agenda Item:	7.2.11.8

Source:	Ericsson

Title:	View on UE feature description for NR positioning

Document for:	Discussion, Decision

# Introduction

This contribution presents our comments on the e-mail discussion [100e-NR-Rel-16-UEFeatures] email discussion/approval of Rel-16 NR UE feature, on the topic of NR positioning. The comments are based on the version 2, revision 2 of the document “RAN1 UE features list for Rel-16 NR after RAN1#100-E” produced during the e-mail discussion.

# Rel16 UE features for NR positioning

## Clarification of component support

During the UE feature discussion, the discussion leader clarified that each feature group mapped to a capability, so that a UE reporting support for feature group supports all components in the group.

For NR positioning, there are several components labeled “support of ”. for example in FG 13-d1 (Transmission of SRS for positioning) component 5 is “Support of SSB for serving cell DL pathloss estimation and OLPC for SRS for positioning”. A clarification is required as to what is the intended result in signaling:

- Option 1: The support of the component is part of the feature group capability, e.g. a UE signaling support for the feature group does support each component.

- Option 2: the support of the component is itself a capability

In our view, option 1 should be the valid option.  If the support of a component is a capability, the component should be a separate feature group. Therefore, we propose that all components that start with “support” are either rephrased in an affirmative statement (e.g. for the example above “SSB for serving cell DL pathloss estimation and OLPC for SRS for positioning is supported” , or moved to a separate feature group to become a capability.

Components starting with “support” are either rephrased in an affirmative statement or moved to a separate feature group to become a capability.

## Capabilities for RRM measurements

The motivation for the 13a-1 and 13-a2 Feature group is unclear. There is even a note to mention that there is no new UE capability.  We do not see the need for these feature groups.

Remove feature groups 13a-1 and 13a-2

## PRS processing capability

Component 2 of feature group 13c-1 details the PRS processing capability. There is a list of notes that should be moved to the appropriate column (notes) as they don’t belong in the component column.

Move the notes of component 2 in 13c-1 to the note section.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Components starting with “support” are either rephrased in an affirmative statement or moved to a separate feature group to become a capability.

Proposal 2	Remove feature groups 13a-1 and 13a-2

Proposal 3	Move the notes of component 2 in 13c-1 to the note section.

# References

RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, Shenzhen, China, March, 2019