3GPP TSG RAN WG1 #100bis		R1-2002947

e-Meeting, April 20th – 30th, 2020

Source:	Moderator (OPPO)

Title:	Text Proposal 1 in [100b-e-NR-eMIMO-multiTRP-03]

Agenda Item:	7.2.6.2

Document for:	Discussion and Decision

- Introduction

Rel-16 enhancement on MIMO WID includes objectives of enhancing multi-TRP/Panel transmission with ideal and non-ideal backhaul. During the work of rel-16, designs for multiple-PDCCH based and single-PDCCH based multi-TRP/Panel transmission were discussed and specified. This document provides the TP for #b-9 listed in [21] for TS 38.212

- #b-9: PT-RS transmission in single-DCI based M-TRP

- Text Proposal

Reason for changes:

It was agreed that 2-port PT-RS is supported for single-DCI based multi-TRP transmission. When a PDSCH with multiple layers is scheduled with two TCI states, a subset of the layers is transmitted from one of the two TRPs.  If two PTRS ports are configured, each PTRS port is transmitted from one of the two TRPs.  In this case, the amount of PTRS power allocation for a PTRS port would depend on the number of PDSCH layers scheduled from the same TRP.  However, there is no description on power boosting for 2-port PT-RS transmission in current specification.

Summary of changes:

Add the specification that, if two PT-RS ports are transmitted, the power boosting ratio for each PT-RS port depends on the number of PDSCH layers transmitted from the same TRP, not the total number of PDSCH layers.

Specs/Sections impacted:

TS 38.214 V16.1.0 /4.1

Consequences if not approved:

The UE behavior on power boosting of 2-port PT-RS is ambiguous.

The text proposal for TS 38.214 is:

## 4.1 Power allocation for downlink ---- Unchanged text are omitted ---- When the UE is scheduled with one or two PT-RS ports associated with the PDSCH, - if the UE is configured with the higher layer parameter epre-Ratio, the ratio of PT-RS EPRE to PDSCH EPRE per layer per RE for each PT-RS port () is given by Table 4.1-2 according to the epre-Ratio, the PT-RS scaling factor specified in clause 7.4.1.2.2 of [4, TS 38.211] is given by. - otherwise, the UE shall assume epre-Ratio is set to state '0' in Table 4.1-2 if not configured. Table 4.1-2: PT-RS EPRE to PDSCH EPRE per layer per RE () epre-Ratio; The number of PDSCH layers with DM-RS associated to the PT-RS port. 1; 2; 3; 4; 5; 6. 0; 0; 3; 4.77; 6; 7; 7.78. 1; 0; 0; 0; 0; 0; 0. 2; reserved. 3; reserved. ---- Unchanged text are omitted ----

- Reference

- R1-2001563	Remaining issues on multi-TRP in R16	Huawei, HiSilicon

- R1-2001596	Maintenance of multi-TRP enhancements	ZTE

- R1-2001678	Discussion on remaining issues on multi TRP operation	vivo

- R1-2001726	Text proposals for enhancements on multi-TRP and panel Transmission	 OPPO

- R1-2001822	Enhancements on Multi-TRP/Panel Transmission	MediaTek Inc.

- R1-2001913	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

- R1-2001971	Remaining issues on multi-TRP/panel transmission	Lenovo, Motorola Mobility

- R1-2002004	Corrections to multi-TRP	Intel Corporation

- R1-2002052	TP on Multi-TRP/Panel Transmission	Futurewei

- R1-2002090	Remaining issues on multi-TRP/panel transmission	CATT

- R1-2002139	On Rel.16 multi-TRP/panel transmission	Samsung

- R1-2002212	Remaining issues on multi-TRP/panel transmission	CMCC

- R1-2002270	Discussion on remaining issues for multi-TRP operation	Spreadtrum Communications

- R1-2002294	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

- R1-2002337	Remaining issues for Multi-TRP enhancement	Apple

- R1-2002364	Remaining issues on multi-TRP transmission	NEC

- R1-2002378	Clarification on Multi-TRP URLLC Scheme 4	Convida Wireless

- R1-2002448	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC

- R1-2002472	Remaining issues on Multi-TRP/Panel Transmission	Ericsson

- R1-2002551	Multi-TRP Enhancements	Qualcomm Incorporated

- R1-2002730         FL summary #2 for Multi-TRP/Panel Transmission