3GPP TSG RAN WG1 #100bis						R1-2003007

e-Meeting, April 20th – 30th, 2020

______________________________________________________________________Agenda item: 7.2.4.5

Source: Moderator (LG Electronics)

Title: Text proposal from Email discussion thread #1 for AI 7.2.4.5 Physical layer procedures for sidelink

Document for: Discussion and decision

- Introduction and proposal

RAN1 made the agreements copied in Appendix in [100b-e-NR-5G_V2X_NRSL-PHY-Procedure-01] Email discussion/approval regarding handling TX and RX of multiple PSFCHs. This contribution includes the text proposal endorsed. It is proposed to adopt the text proposal in Section 2 for the following reasons

- Reason for change: RAN1 made the agreements to complete the UE procedure for simultaneous PSFCH transmissions/receptions and PSFCH transmit power determination. The text proposal is to implement these agreements.

- Summary of change: Prioritization between PSFCH transmission and reception and determination of PSFCH transmit power are extended to cover the case where multiple PSFCH transmissions and receptions are scheduled for a UE.

- Consequences if not approved: The specification cannot support the case where multiple PSFCH transmissions and receptions are scheduled.

- Text proposal

  - Text proposal for TS 38.213

===========================<Start of change #1>=======================

### 16.2.3	PSFCH

When Nsche,TX PSFCH transmissions are scheduled for a UE and Nmax,PSFCH is the maximum number of transmissions of PSFCH supported by the UE, the A UE determines the number of transmitted PSFCHs N and a power PPSFCH,k(i) for a PSFCH transmission k on a resource pool in PSFCH transmission occasion i for 1≤k≤N as

-  If p0-DL-PSFCH is provided,

PPSFCH,k(i)=minPCMAX,PO,PSFCH+10log102μ+αPFSCH⋅PL [dBm]

Where

-	P'PSFCH=PO,PSFCH+10log102μ+αPFSCH⋅PL [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PO,PSFCH is a value of p0-DL-PSFCH if provided; else, PPSFCH(i)=PCMAX

-	αPFSCH is a value of alpha-DL-PSFCH, if provided; else, αPFSCH=1

-	PL=PLb,f,c(qd) as described in Clause 7.1.1

-	1≤k≤N where N is a number of PSFCH transmissions in PSFCH transmission occasion i as described in Clause 16.2.4.2

-  if Nsche,TX≤Nmax,PSFCH,

-  if P'PSFCH+10log10Nsche,TX is smaller than or equal to PCMAX  determined for Nsche,TX PSFCH transmissions according to [8-1, TS 38.101-1],

-  N=Nsche,TX and

-  PPSFCH,k(i)=P'PSFCH [dBm]

-  else

-  UE determines by its implementation N PSFCH transmissions with ascending order of the priority as described in Clause 16.2.4.2 such that N≥X where X is [TBD] and

-  PPSFCH,k(i)=minPCMAX-10log10(N),P'PSFCH [dBm]

where PCMAX	is defined in [8-1, TS 38.101-1] determined for the N simultaneous PSFCH transmissions

-  else

-  UE selects by its implementation Nmax,PSFCH PSFCH transmissions with ascending order of the priority as described in Clause 16.2.4.2

- if P'PSFCH+10log10Nmax,PSFCH is smaller than or equal to PCMAX determined for Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1],

-  N=Nmax,PSFCH and

-  PPSFCH,k(i)=P'PSFCH [dBm]

-  else

-  UE determines by its implementation N PSFCH transmissions with ascending order of the priority as described in Clause 16.2.4.2 such that N≥X where X is [TBD] and

-  PPSFCH,k(i)=minPCMAX-10log10(N),P'PSFCH [dBm]

where PCMAX	is defined in [8-1, TS 38.101-1] determined for the  N simultaneous PSFCH transmissions

=========================<End of change #1>==========================

===========================<Start of change #1>=======================

#### 16.2.4.2	Simultaneous PSFCH transmission/reception

If a UE

-	would transmit a first Nsche,TX PSFCHs and receive a second Nsche,RXPSFCHs in a PSFCH slot, and

-	a transmissions of the first Nsche,TX PSFCHs would overlap in time with a receptions of the second Nsche,RX PSFCHs

the UE transmits or receives only the set of PSFCHs with the higher smallest priority field value as determined by a first set of SCI format 0_1 and a second set of SCI format 0_1 [5, TS 38.212] that are respectively associated with the first Nsche,TX PSFCH and the second Nsche,RX PSFCH.

If a UE would transmit M Nsche,TX PSFCHs in a PSFCH transmission occasion, the UE transmits minM,NN PSFCHs corresponding to the smallest minM,NN priority field values indicated in all SCI formats 0_1 associated with the PSFCH transmission occasion, where N is the number of PSFCH transmissions determined in Clause 16.2.3.

=========================<End of change #2>==========================

Appendix: Agreements made in the email discussion [100b-e-NR-5G_V2X_NRSL-PHY-Procedure-01]

Agreements:

When the UE supports up to Nmax,psfch simultaneous PSFCH transmissions in a PSFCH TX occasion and UE have Nreq PSFCHs to be transmitted in a given PSFCH TX occasion, the UE selects N PSFCHs for actual transmission with ascending order of the priority in a PSFCH TX occasion as follows:

- Case 1: When Nreq<=Nmax,psfch and PO,PSFCH is (pre-)configured,

  - Case 1-1: N=Nreq if the sum of PO,PSFCH+10log102μ+αPFSCH⋅PL for the Nreq PSFCHs is smaller than or equal to PCMAX determined for the Nreq PSFCH transmissions.

  - Case 1-2: Otherwise, N is up to UE implementation under N >= X >= 1.

- Case 2: When Nreq>Nmax,psfch and PO,PSFCH is (pre-)configured, the UE firstly selects Nmax,psfch PSFCHs with ascending order of the priority.

  - Case 2-1: N=Nmax,psfch if the sum of PO,PSFCH+10log102μ+αPFSCH⋅PL for the Nmax,psfch PSFCHs is smaller than or equal to PCMAX determined for the Nmax,psfch PSFCH transmissions.

  - Case 2-2: Otherwise, N is up to UE implementation under N >= X >= 1.

- Down select X in RAN1#101-e

  - Alt 1: X = max {1, the largest value which doesn’t lead to the power limited case}

  - Alt 2: X= 1

  - Other alternatives are not precluded.

Agreements:

For the prioritization between PSFCH TX and PSFCH RX,

- When the UE is required to transmit more than one PSFCH, the highest priority of the associated PSCCH/PSSCH is used for prioritization of the PSFCH transmission.

- When the UE is required to receive more than one PSFCH, the highest priority of the associated PSCCH/PSSCH is used for prioritization of the PSFCH reception.