3GPP TSG RAN WG1 #100bis-e	R1-2003109

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.4.2.1

Source:	Moderator (Ericsson)

Title:	Text proposal for TS 38.213 related to the agreements in [100b-e-NR-5G_V2X_NRSL-Mode-1-03]

Document for:	Endorsement

# 1	Introduction

This document contains a text proposal for supporting Type-1 codebooks for SL HARQ-ACK reporting to the gNB.

Following the existing agreements, the TP takes TS 38.213 v15.9.0 as the starting point. All changes are made with respect to the procedures for DL HARQ-ACK reporting in that version of the specification.

Among others, the following agreements from RAN1#100-bis-e are considered:

Agreements: - Higher layer signaling is used to configure the values of the PSFCH to PUCCH gap (NOTE: this is referred to as sl-FeedbackToUL-ACK in the following) - The field PSFCH-to-HARQ_feedback timing indicator: - Selects one of the configured values of the PSFCH to PUCCH gap, except in the case that, together with PUCCH resource indicator, it indicates that no PUCCH resource is provided. - FFS Presence in DCI format 3_0 and size (0-3 bits). Conclusion: - In preparing the TP for SL HARQ-ACK reporting to the gNB using type-1 codebook: - The following parameters and the corresponding parts of the Rel-15 specification are not used: - Parameters related to transmission of more than 1 TB - harq-ACK-SpatialBundlingPUCCH - maxNrofCodeWordsScheduledByDCI - Parameters related to CBG transmission - PDSCH-CodeBlockGroupTransmission - maxCodeBlockGroupsPerTransportBlock - Parameters related to slot aggregation - pdsch-AggregatoinFactor - Set of row indices R and pdsch-TimeDomainAllocationList - firstActiveDownlinkBWP-Id - The following functionality from the Rel-15 specification is not supported - HARQ-ACK for SPS PDSCH release - Capability to receive more than one unicast PDSCH per slot - Reporting of UCI (e.g., SR, CSI) NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way. Conclusion: - In preparing the TP for SL HARQ-ACK reporting to the gNB using type-1 codebook, at least the following changes are made to the Rel-15 specification: - A single cell is considered - DCI format 3_0 is used instead of formats 1_0 and 1_1. - SL- RNTI and SL-CS-RNTI replace C-RNTI and CS-RNTI, respectively. - sl-FeedbackToUL-ACK replaces dl-DataToUL-ACK - “PSFCH-to-HARQ_feedback timing indicator” replaces “PDSCH-to-HARQ_feedback timing indicator” - sl-PSFCH-ToPUCCH is used to determine PSFCH-to-PUCCH gap for SL CG type-1 - SL BWP replaces DL BWP - The ratio 2^(u_SL-u_UL) replaces the ratio 2^(u_DL-u_UL) - SL resource pool configuration is used to determine whether a slot belong to the SL pool or not. - The period of PSFCH resources is used: - to determine the number of PSFCH reception occasions for one PUCCH transmission - to determine whether a given SL slot has PSFCH resources or not - to determine the number of HARQ-ACK bits associated with each PSFCH reception occasion. NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way

The motivation of the TP is as follows:

- Reasons for change: introduction of SL HARQ-ACK reporting to the gNB using type-1 codebook.

- Summary of changes: the Rel-15 procedure for DL HARQ-ACK reporting is reused with adaptations for SL HARQ-ACK reporting to the gNB using type-1 codebook.

- Specs/Sections impacted: TS 38.213 Section 16.5

- Consequences if not approved: SL HARQ-ACK reporting to the gNB using type-1 codebook is not supported.

R1-2003111 contains a related TP for the introduction of type-2 codebook for SL HARQ-ACK reporting on uplink.

# 2	Text Proposal for TS 38.213

---------------------------------- Start of Text Proposal ---------------------------------

## 16.5	UE procedure for reporting HARQ-ACK on uplink

<Unchanged parts omitted>

A UE does not expect to be indicated to transmit HARQ-ACK information for more than one SL configured grants in a same PUCCH.

In the following, the CRC for DCI format 3_0 is scrambled with a SL-RNTI, or a SL-CS-RNTI.

### 16.5.1	Type-1 HARQ-ACK codebook determination

This subclause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

A UE reports HARQ-ACK information for PSSCH transmission(s) with corresponding PSFCH reception(s) only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PSFCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 3_0 scheduling the PSSCH transmission(s), or by a value of PSFCH-to-HARQ feedback timing indicator field in a corresponding activating DCI format 3_0 for a SL configured grant type-2, or by a value of sl-PSFCH-ToPUCCH for a SL configured grant type-1. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PSFCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 3_0 or by a value of sl-PSFCH-ToPUCCH for a SL configured grant type-1.

The UE reports HARQ-ACK information for PSSCH transmission(s) with corresponding PSFCH reception(s) in slot  only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot , where  is a number of slots indicated by the PSFCH-to-HARQ_feedback timing indicator field in a corresponding DCI format. If the UE reports HARQ-ACK information for the PSSCH transmission(s) with corresponding PSFCH reception(s) in a slot other than slot , the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

If a UE reports HARQ-ACK information in a PUCCH only for

-	PSFCH reception(s) associated with PSSCH transmission(s) scheduled by a DCI format 3_0 with counter SAI field value of 1, or

-	PSFCH reception(s) associated with a PSSCH transmission(s) scheduled by a SL configured grant

within the MA occasions for candidate PSSCH transmission(s) with corresponding PSFCH reception(s) as determined in Subclause 16.5.1.1, the UE determines a HARQ-ACK codebook only for the PSFCH reception(s) associated with PSSCH transmission(s) scheduled by DCI format 3_0 or only for the PSFCH reception(s) associated with PSSCH transmission(s) scheduled by a SL configured grant according to corresponding MA occasion(s), where the value of counter DAI in DCI format 3_0 is according to Table 9.1.3-1; otherwise, the procedures in Subclause 16.5.1.1 for a HARQ-ACK codebook determination apply.

#### 16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell , a SL BWP, and an active UL BWP, as described in Subclause 12, the UE determines a set of MA occasions for candidate PSSCH transmission(s) with corresponding PSFCH reception(s) for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot . The determination is based:

a)	on a set of slot timing values  associated with the SL BWP.  is provided by sl-FeedbackToUL-ACK for DCI format 3_0

b)	on the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration  provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively

c)	a set of configured SL pool bitmaps

d)	The value of a period of PSFCH transmission occasion resources for the SL pool provided in periodPSFCHresource.

For the set of slot timing values , the UE determines a set of MA occasions for candidate PSSCH transmission(s) with corresponding PSFCH reception(s) according to the following pseudo-code.

Set  - index of occasion for candidate PSSCH transmission(s) with corresponding PSFCH reception(s)

Set MA=∅

Set  to the cardinality of set

Set k =0 – index of slot timing values , in descending order of the slot timing values, in set

Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the SL pool

while

if mod nU-K1,k+1,max2μUL-μSL,1=0

Set nS=0 – index of a SL slot within an UL slot

while nS<max2μSL-μUL,1

if slot  starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell

nS=nS+1;

else

if slot nU-K1,k⋅2μSL-μUL+nS belongs to the SL pool and contains PSFCH resources as indicated at least by one of the SL pool bitmaps and periodPSFCHresource, where  is the k-th slot timing value in set

Set nF=0 – index of a SL slot within an PSFCH period

while nF<NPSFCH

MA=MA∪j;

j=j+1;

nF=nF+1;

end while

end if

nS=nS+1;

end if

end while

end if

;

end while

A UE determines  HARQ-ACK information bits, for a total number of  HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a PSFCH, due to the UE not detecting a corresponding DCI format 3_0, the UE generates a NACK value for the PSFCH. The cardinality of the set MA defines a total number M of occasions for PSFCH reception corresponding to the HARQ-ACK information bits.

= HARQ-ACK information bit for candidate PSSCH transmission(s) with corresponding PSFCH reception(s) with index j for  0 ≤j<M as described in clause 16.5.

If OACK≤11, the UE determines a number of HARQ-ACK information bits  for obtaining a transmission power for a PUCCH, as described in Subclause 7.2.1, as nHARQ-ACK=Nmreceived where

-	Nmreceived is the number of HARQ-ACK information bits determined for corresponding PSSCH transmission(s) with corresponding PSFCH reception(s) in PSFCH reception occasion .

---------------------------------- End of Text Proposal ---------------------------------