3GPP TSG RAN WG1 #101		R1-2003390

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Enhancement for Scheduling/HARQ

Agenda Item:	7.2.5.4

Document for:	Discussion and Decision

- Introduction

In RAN1#100bis-e meeting, enhancement for Scheduling/HARQ are discussed. The following agreements are achieved [1]:

- If a UE is scheduled with a first PDSCH and a second PDSCH which is starting later than the first PDSCH on a given serving cell, the corresponding PUCCHs carrying HARQ-ACK with different priorities can overlap in time.

  - FFS: For supporting this feature, a new FG, separate from FG 12-1, will be introduced.

  - FFS: The PUCCH associated with the second PDSCH cannot be scheduled for transmission at or earlier than PUCCH associated with the first PDSCH.

In this contribution, we share our view on enhancement for Scheduling/HARQ.

- Remaining issues on two overlapping PUCCHs

In Rel-15, for HARQ-ACK feedback with one priority, out-of-order HARQ may occur, i.e. gNB can indicate out of order HARQ-ACK timing within a slot. The starting symbol of PUCCH resource corresponding to the second PDSCH transmission is earlier than the start symbol of PUCCH resource corresponding to the first PDSCH. However, in Rel-15 UE will multiplex the two HARQ-ACK bits into one PUCCH resource within the slot such that there is no out-of-order issue for the actual PUCCH transmission. UE processes both PDSCH and feeds back HARQ-ACK using one PUCCH.

In Rel-16, when a UE is scheduled with a first PDSCH and a second PDSCH which is starting later than the first PDSCH on a given serving cell, the corresponding PUCCHs carrying HARQ-ACK with different priorities can overlap in time. In this case, PUCCH carrying HARQ-ACK with the low priority would be dropped as long as cancelation timeline is met. Therefore, UE only transmits PUCCH carrying HARQ-ACK with high priority. From this point, there is no additional UE complexity and no new UE feature is needed.

Considering the long discussion on this issue, we can also support to introduce a new UE feature for compromising as second preference. In our opinion, two overlapped PUCCHs corresponding to different priority starting at the same symbols should be deemed as in order HARQ-ACK. Thus, we correct the description of UE feature as following:

- The starting symbol of PUCCH associated with the second PDSCH is scheduled for transmission earlier than the starting symbol of PUCCH associated with the first PDSCH

Proposal 1:

- 1st  preference: separate UE feature is not needed.

  - The starting symbol of PUCCH associated with the second PDSCH can be scheduled for transmission at or earlier than the starting symbol of PUCCH associated with the first PDSCH.

- 2nd preference: a new FG, separate from FG 12-1 can be introduced. UE feature definition as following:

  - The starting symbol of PUCCH associated with the second PDSCH can be scheduled for transmission earlier than the starting symbol of PUCCH associated with the first PDSCH.

- Conclusion

In the contribution, we have some investigations on enhancement to scheduling/HARQ, and propose that,

Proposal 1:

- 1st  preference: separate UE feature is not needed.

  - The starting symbol of PUCCH associated with the second PDSCH can be scheduled for transmission at or earlier than the starting symbol of PUCCH associated with the first PDSCH.

- 2nd preference: a new FG, separate from FG 12-1 can be introduced. UE feature definition as following:

  - The starting symbol of PUCCH associated with the second PDSCH is scheduled for transmission earlier than the starting symbol of PUCCH associated with the first PDSCH.

References

[1] Chairman notes RAN1 #100bis e-meeting