3GPP TSG RAN WG1 Meeting #101-e                                      R1-2003473

e-Meeting, May 25th – June 5th, 2020

Source:         ZTE

Title:            Maintenance of UL Reference Signals for NR Positioning

Agenda item:    7.2.8.2

Document for:	Discussion and Decision

Introduction

In this contribution, we present our view on text changes of UL reference signals for NR positioning.

Text proposal

The current Table 7.3.1.1.2-24 in TS 38.212 uses SRS-PosResourceSet and aperiodicSRS-ResourceTriggerList  instead of SRS-PosResourceSet-r16 and aperiodicSRS-ResourceTriggerList-r16, which are the correct name defined by RAN2. To align the parameter name, we propose to replace SRS-PosResourceSet and aperiodicSRS-ResourceTriggerList with SRS-PosResourceSet-r16 and aperiodicSRS-ResourceTriggerList-r16.

Further, for positioning, higher layer parameter aperiodicSRS-ResourceTrigger does not exist in SRS-PosResourceSet-r16 based on the endorsed 38.331[2] because it is merged into aperiodicSRS-ResourceTriggerList-r16. So we suggest to remove the related description.

Based on the above analysis, we have the text changes in appendix.

Proposal 1: Adopt the TP in Appendix.

Conclusions

In this contribution, we have identified one unaligned parameter name in Table 7.3.1.1.2-24 in TS 38.212. We have the following proposal.

Proposal 1: Adopt the TP in Appendix.

References

- 3GPP TS 38.212 V16.1.0 (2020-05), Multiplexing and channel coding (Release 16)

- 3GPP TS 38.331 V16.0.0 (2020-04), Radio Resource Control (RRC) protocol specification (Release 16)

- Appendix: TP

=====================TS 38.212 clause 7.3.1.1.2 unchanged parts omitted=========================

Table 7.3.1.1.2-24: SRS request

| Value of SRS request field | Triggered aperiodic SRS resource set(s) for DCI format 0_1, 0_2, 1_1, 1_2, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' | Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA' |
| --- | --- | --- |
| 00 | No aperiodic SRS resource set triggered | No aperiodic SRS resource set triggered |
| 01 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 SRS resource set(s) configured by SRS-PosResourceSet-16 with an entry in aperiodicSRS-ResourceTriggerList-r16 set to 1 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 1st set of serving cells configured by higher layers, or SRS resource set(s) configured by SRS-PosResourceSet-r16 and resourceType in SRS-PosResourceSet-r16 set to 'aperiodic' for a 1st set of serving cells configured by higher layers |
| 10 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 or an entry in aperiodicSRS-ResourceTriggerList set to 2 SRS resource set(s) configured by SRS-PosResourceSet-r16 with an entry in aperiodicSRS-ResourceTriggerList-r16 set to 2 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 2nd set of serving cells configured by higher layers, or SRS resource set(s) configured by SRS-PosResourceSet-r16 and resourceType in SRS-PosResourceSet-r16 set to 'aperiodic' for a 2nd set of serving cells configured by higher layers |
| 11 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 or an entry in aperiodicSRS-ResourceTriggerList set to 3 SRS resource set(s) configured by SRS-PosResourceSet-16 with an entry in aperiodicSRS-ResourceTriggerList-r16 set to 3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 3rd set of serving cells configured by higher layers, or SRS resource set(s) configured by SRS-PosResourceSet-r16 and resourceType in SRS-PosResourceSet-r16 set to 'aperiodic' for a 3rd set of serving cells configured by higher layers |

==================================unchanged parts omitted===============================