3GPP TSG RAN WG1 Meeting #101-e                                                                              R1-2003516

E-meeting, May 25 - June 05, 2020

Agenda Item:	7.2.2.2.5

Source:	Huawei, HiSilicon

Title:	Maintenance on the wideband operation procedures

Document for:	Discussion and Decision

Introduction

In the paper, we discuss the remaining issues on the wideband operation for shared spectrum operation after RAN1#100bis-e. The issues includes PDCCH monitoring in multiple monitoring locations, RB set/intra cell guard band configurations. In section 4, PUSCH mapping on the intra cell guard band is discussed. The corresponding TPs are in the appendix.

PDCCH monitoring

In NR Rel-15, gNB configures the number of monitored PDCCH candidates (nrofCandidates and nrofCandidates-SFI) in the IE of SearchSpace for each search space. In RAN1#101bis-e [1] , following agreement was achieved when multiple monitoring locations are configured for a search space.

Agreement:

The number of PDCCH candidates per aggregation level configured by nrofCandidates or nrofCandidates-SFI within a SearchSpace IE applies to each of RB sets configured by freqMonitorLocations-r16.

- nrofCandidates-SFI is 1 for a search space configured with freqMonitorLocations-r16

-	an indication by dci-Format2-0 to monitor one or two PDCCH candidates, or to monitor one PDCCH candidate per RB set if the UE is provided freqMonitorLocation-r16 for the search space set, for DCI format 2_0 and a corresponding CCE aggregation level

In latest CR for TS38.213 section 10.1 [2], the agreement was captured as the highlighted text for DCI format 2_0 as following.

-	a number of PDCCH candidates  per CCE aggregation level  by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively

….

ms,nCI=0,⋯,Ms,nCIL-1, where Ms,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s for a serving cell corresponding to nCI;

for any CSS, Ms,maxL=Ms,0L;

for a USS, Ms,maxL is the maximum of Ms,nCIL over all configured nCI values for a CCE aggregation level L of search space set s ;

However, it is not reflected for UE specific search space.  For example, the following text in the TS 38.213 v16.1.0 [4] only covers the case where freqMonitorLocations-r16 is not configured.

Proposal 1: The agreement on nrofCandidates when freqMonitorLocations-r16 is configured should be captured in TS 38.213 as TP#1 in the appendix.

When counting on the number of allocated PDCCH candidates and non-overlapped CCE, the UE subtracts the number of PDCCH candidates and non-overlapped CCEs per search space in ascending order of search space ID. In Rel-15, UE would drop the whole search space when the number of allocated PDCCH candidate or non-overlapped CCE exceed maximum number. If a search space has multiple monitoring locations, UE will subtract PDCCH candidate and non-overlapped CCE per monitoring location in ascending order of RB set where monitoring location is. UE drops the PDCCH blind detection in the rest of monitoring location(s) and search space(s).

Proposal 2: UE can drop PDCCH BD per monitoring location in a search space. The corresponding text proposal is in TP#1 in the appendix.

UL BWP configuration

In RAN1#99 [5], there was an agreement to ensure that a BWP consists one or multiple full RB set(s) from the perspective of channel access.

Agreement:

- The RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 include a mechanism to indicate that no intra-carrier guard-bands are configured

  - Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP

- For a carrier with intra-carrier guard bands, the UE does not expect that the dedicated BWP is configured to include parts of a RB set.

In RAN1#100bis-e [1], it was agreed that no intra cell guard in UL carrier was achieved by configuring nrofCRBs-r16 to 0 from the perspective of uplink resource allocation.

Agreement:

For an UL carrier without intra-cell guard bands when the parameter useInterlacePUCCH-PUCCH is configured in any of BWP-UplinkCommon and BWP-UplinkDedicated:

- The UL carrier can be configured with NRB-set,UL≥1NRB-set,UL≥1non-overlapping RB set(s)

- For each RB set except for RB set 0, the starting CRB index is given by startCRB-r16

  - For RB set 0, the starting CRB index is given by Ngrid,ULstart,μNgrid,ULstart,μ

- The UE expects nrofCRBs-r16 set to 0 for all GBs between two adjacent RB sets within the UL carrier.

- The UE expects N RBs contained in each interlace of each RB set, wherein 10 <= N <= 11.

  - For 30 kHz SCS, the number of RBs within any RB set is between 50 and 55, and for 15 kHz SCS, the number of RBs within any RB set is between 100 and 110

- Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP (from RAN1#99 agreement)

- Note: It’s up to gNB’s configuration to fulfill RAN4 requirement with  e.g., on maximum transmission bandwidth configuration, spectral emission mask, and so on.

- Note: In order to reuse existing PUCCH/PUSCH resource allocation mechanisms, this proposal applies to all supported carrier bandwidths except 10 MHz

- FFS: Whether BWP can be configured to be partially overlapping with a RB set

When a UE is configured with a UL BWP consisting of part of UL RB sets within the carrier bandwidth, the number of available RB in the BWP may exceed the maximum number of available RB of a carrier with same bandwidth, which may result leakage out of BWP exceeding the limit. Considering little usage of such configuration, a UL BWP should include all RB sets in a carrier if UL intra cell guard band is configured with 0 RB.

Proposal 3: a UL BWP should include all RB sets in a carrier if UL intra cell guard band is configured with 0 RB.

Uplink resource mapping on the intra-cell guard band

In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and a set of up to N RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any.

In the section 6.1.2.2.3 of TS38.214 v16.1.0 [4], resource mapping on intra cell guard bands between two indicated adjacent RB sets were supported for uplink resource allocation type 0/1 implicitly and for resource allocation type 2 explicitly as copied in the text box below.

However, the current text highlighted in yellow results in zero frequency resource allocation, considering the mutual exclusion between intra-cell guard bands and RB set according to the definition. The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets, and intersection of resource blocks of the indicated interlaces and intra-cell guard bands defined in Clause 7 between the indicated RB sets if scheduled.

Proposal 4: The intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets will result zero frequency resource allocation. The text should be reworded as TP#2.

For uplink resource allocation type 0, the granularity of resource allocation is one RBG. The size of RBG depends on number of PRB in a BWP, which is defined in the Table 6.1.2.2.1-1 in TS38.214. One RBG includes at most 16 PRB.  On the other side, the configuration of intra cell guard band is in the unit of RB, which usually does not align with the RBG boundaries. Based on NR Rel-15 mechanism, at most 15 PRB might be wasted in an RB set as illustrated in the Figure 2. It would be beneficial for UE to rate match RB indicated by FDRA while overlapping with intra cell guard band between two RB set where PUSCH is not scheduled on one of the RB set.

Proposal 5: For resource allocation type 0, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated RBG(s) and union of RB set(s) overlapping with the indicated RBG(s) and intra cell guard bands between the adjacent RB sets overlapping with the indicated RBG(s), if any. The corresponding text proposal is in TP#2 in the appendix.

Figure 2 Resource allocation type 0 in RB set

Conclusions

Based on the discussion, we have following observations and conclusions.

Proposal 1: The agreement on nrofCandidates when freqMonitorLocations-r16 is configured should be captured in TS 38.213 as TP#1 in the appendix.

Proposal 2: UE can drop PDCCH BD per monitoring location in a search space. The corresponding text proposal is in TP#1 in the appendix.

Proposal 3: a UL BWP should include all RB sets in a carrier if UL intra cell guard band is configured with 0 RB.

Proposal 4: The intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets will result zero frequency resource allocation. The text should be reworded as TP#2.

Proposal 5: For resource allocation type 0, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated RBG(s) and union of RB set(s) overlapping with the indicated RBG(s) and intra cell guard bands between the adjacent RB sets overlapping with the indicated RBG(s), if any. The corresponding text proposal is in TP#2 in the appendix.

References

Chairman’s Note RAN1#100bis-e

R1-2003180, “Corrections on shared spectrum channel access”, Samsung

3GPP TS38.213 v16.1.0

3GPP TS38.214 v 16.1.0

Chairman’s Note of RAN1#99

- Appendix:

- TP#1: TS38.213 v16.1.0

- 10.1	UE procedure for determining physical downlink control channel assignment

*** Unchanged text is omitted ***

-	a number of PDCCH candidates  per CCE aggregation level  by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively; or a number of PDCCH candidates Ms(L) per CCE aggregation level L per RB set if the UE is provided freqMonitorLocation-r16 for the search space set.

*** Unchanged text is omitted ***

For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI of the search space set and per RB set if any in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCIare given by

where

for any CSS, Yp,ns,fμ=0;

for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ-1modD, Yp,-1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537;

i=0,⋯,L-1;

NCCE,p is the number of CCEs, numbered from 0 to NCCE,p-1, in CORESET p  and, if any, per RB set;

nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, nCI=0;

ms,nCI=0,⋯,Ms,nCIL-1, where Ms,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s and per RB set if any for a serving cell corresponding to nCI;

for any CSS, Ms,maxL=Ms,0L;

for a USS, Ms,maxL is the maximum of Ms,nCIL over all configured nCI values for a CCE aggregation level L of search space set s ;

the RNTI value used for nRNTI is the C-RNTI.

A UE does not expect to be provided freqMonitorLocations-r16 for a search space set s in a serving cell if intraCellGuardBandDL-r16 indicates that no intra-cell guard-bands are configured for the serving cell.

*** Unchanged text is omitted ***

For all search space sets within a slot , denote by  a set of CSS sets with cardinality of  and by  a set of USS sets with cardinality of . The location of USS sets , , in  is according to an ascending order of the search space set index.

Denote by , , the number of counted PDCCH candidates for monitoring for CSS set  and by , , the number of counted PDCCH candidates for monitoring for USS set . Denote by MSussj L, 0≤j<Juss, , the number of counted PDCCH candidates for each monitoring location for USS set Sussj, if  freqMonitorLocations-r16 is configured.

For the CSS sets, a UE monitors  PDCCH candidates requiring a total of  non-overlapping CCEs in a slot.

The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration  in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided value 0 for first CORESETs, and is provided value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring.

Denote by  the set of non-overlapping CCEs for search space set  and by  the cardinality of  where the non-overlapping CCEs for search space set  are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets , .

Set

Set

Set

while  AND

allocate  PDCCH candidates for monitoring to USS set

;

;

;

end while

if freqMonitorLocations-r16 is configured, there are KMLmonitoring locations in frequency domain in the search space set Sussj. Denote by VCCESussj the set of non-overlapping CCEs in each monitoring location of search space set Sussj and by CVCCESussj the cardinality of VCCESussj where the non-overlapping CCEs in each monitoring location of search space set Sussj are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets Sussj, 0≤k≤j .

Set  ;

Set ;

Set ;

While LMSussj L≤MPDCCHuss AND CVCCESussj≤CPDCCHuss

Set l=0;

While l+1∙LMSussj L≤MPDCCHuss  AND  l+1∙CVCCESussj≤CPDCCHuss AND l<KML

k=k+1;

end while

allocate k∙LMSussj L PDCCH candidates for monitoring to USS set

MPDCCHuss=MPDCCHuss-k∙LMSussj L;

CPDCCHuss=CPDCCHuss-k∙CVCCESussj;

j=j+1;

end while

*** Unchanged text is omitted ***

- TP#2: TS38.214v16.0.0

##### 6.1.2.2.1	Uplink resource allocation type 0

In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured in pusch-Config and the size of the bandwidth part as defined in Table 6.1.2.2.1-1. For operation with shared spectrum channel access mechansim, The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated RBGs and the union of RB sets overlapping with the indicated RBGs and intra-cell guard bands defined in Clause 7 between the adjacent RB sets overlapping with the indicated RBGs, if any.

*** Unchanged text is omitted ***

##### 6.1.2.2.3	Uplink resource allocation type 2

In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and a set of up to N RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the union of indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any.

*** Unchanged text is omitted ***