3GPP TSG RAN WG1 Meeting #101                                         R1-2003628

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	Remaining Issues on Multi-beam Operation Enhancement

Agenda Item:	7.2.6.3

Document for:	Discussion and Decision

- Introduction

In previous meetings, majority of issues on multi-beam operation were resolved. There are some remaining details which need to be refined or updated in the specification. We provide our TPs on multi-beam operation related issues in this contribution.

- TPs for specifications

2.1  TPs for 38.213

In RAN1 #99 meeting, the following agreement for determination of RS resource index qd for path-loss measurement of PUSCH scheduled by DCI format 0_1 without a SRI field was reached.

Agreement in RAN1#99 When enablePLRSupdateForPUSCHSRS is configured, if a grant-based or grant-free PUSCH transmission is scheduled/activated by DCI format 0_1 that does not include a SRI field, the RS resource index qd corresponding to the PUSCH-PathlossReferenceRS-Id mapped with sri-PUSCH-PowerControlId = 0 is used for path-loss measurement of PUSCH transmission. In this case, UE expects to be configured with sri-PUSCH-PowerControl

In this case, the prerequisite for the UE to use the RS resource index qd  corresponding to the PUSCH-PathlossReferenceRS-Id mapped with sri-PUSCH-PowerControlId = 0 is that UE is configured with parameter sri-PUSCH-PowerControl. If this parameter is not configured, it is ambiguous for UE to determine the pathloss reference signal for power control.

Proposal 1: Capture RAN1#99 agreement that UE expects to be configured with sri-PUSCH-PowerControl to determine RS resource index qd which will be used for path-loss measurement of PUSCH transmission. When enablePLRSupdateForPUSCHSRS is configured and if a grant-based or grant-free PUSCH transmission is scheduled/activated by DCI format 0_1 that does not include a SRI field.

TS38.213: 7.1.1 UE behaviour -----Start TP----- - For a PUSCH transmission scheduled by a DCI format that does not include a SRI field, or for a PUSCH transmission configured by ConfiguredGrantConfig and activated, as described in Clause 10.2, by a DCI format that does not include a SRI field, a RS resource index is determined from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0. The UE expects to be configured with sri-PUSCH-PowerControl. -----End TP-----

In section 6 of TS38.213 it is specified that periodic CSI-RS can be explicitly or implicitly provided as the reference signal of Beam Failure Detection (BFD). SS/PBCH blocks are not included in the BFD reference signal.

TS38.213: 6 Link recovery procedures A UE can be provided, for each BWP of a serving cell, a set of periodic CSI-RS resource configuration indexes by failureDetectionResources or beamFailureDetectionResourceList …….. If the UE is not provided by failureDetectionResources or beamFailureDetectionResourceList for a BWP of the serving cell, the UE determines the set to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH.

At the same time, in section 6 of TS38.213 it is said that in RAN1 when UE assess the quality of the beam, both CSI-RS and SS/PBCH blocks can be used.

TS38.213: 6 Link recovery procedures The physical layer in the UE assesses the radio link quality according to the set of resource configurations against the threshold Qout,LR. For the set , the UE assesses the radio link quality only according to periodic CSI-RS resource configurations, or SS/PBCH blocks on the PCell or the PSCell, that are quasi co-located, as described in [6, TS 38.214]

These two paragraphs are contradictory to each other as to whether SS/PBCH block are allowed to be used for beam failure detection. A clarification is needed. Depending on the outcome, one of these two paragraphs needs to be updated.

Proposal 2: Clarify whether SS/PBCH blocks can be provided for BFD.

- Conclusion

In this contribution, the following proposals are given:

Proposal 1: Capture RAN1#99 agreement that UE expects to be configured with sri-PUSCH-PowerControl to determine RS resource index qd which will be used for path-loss measurement of PUSCH transmission. When enablePLRSupdateForPUSCHSRS is configured and if a grant-based or grant-free PUSCH transmission is scheduled/activated by DCI format 0_1 that does not include a SRI field.

Proposal 2: Clarify whether SS/PBCH blocks can be provided for BFD.

- References

- 3GPP RAN1, TS 38.213 V16.1.0