3GPP TSG RAN WG1 Meeting #101		                     	   	           R1-2003640

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	IIoT use cases and scenarios for evaluation of NR Positioning Enhancements

Agenda item:	8.2.1

Document for:	Discussion and Decision

Introduction

At RAN#86 meeting, the study item on NR Positioning Enhancements was approved [1]. From RAN1’s perspective, the SI includes the following objectives:

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

  - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

NOTE 1:	Sidelink is not part of this objective.

NOTE 2:	Involve RAN4 for validating assumptions for the systems evaluations where appropriate.

NOTE 3:	The commercial use cases and requirements are applicable to a limited geographic area.

As stated above, the SI will define the IIoT use cases with the associated performance requirements for identifying the performance gap and the simulation scenarios for the IIoT use cases with associated parameters.

This contribution discusses IIoT use cases and scenarios for evaluation of NR Positioning Enhancements. Section 2 discusses IIoT use cases and related requirements for positioning enhancements. Section 3 discusses IIoT InF scenarios for the evaluation of positioning performance. Section 4 discusses evaluation parameters for IIoT scenarios. Section 5 summarizes the proposals with conclusions.

IIoT use cases and related requirements

The following 3 scenarios are defined for NR positioning study in Rel-16 in TR 38.855 [3]

-	Scenario 1. Indoor Office for FR1 and FR2 (Open office)

-	Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

-	Scenario 3. UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)

The performance targets for RAT dependent solutions for commercial use cases in Rel-16 are also defined in TR 38.855 [3]

-	Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios

-	Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios

-	End to end latency < 1s

For Rel-17, as indicated in the SI objective, additional scenarios (e.g. IIoT) will be defined for evaluating the achievable positioning accuracy and latency with the Rel-16 positioning solutions and identifying any performance gaps, as well as the enhancements to Rel-16 positioning techniques.

For IIoT positioning requirements, eight IIoT use cases are defined in Table 8.1.7 in TR 22.804 [4] with each of them having different performance requirements in terms of horizontal accuracy, latency, and availability, as shown in below table.

Table 8.1.7   Positioning Service Performance Requirements (from TS 22.804)

| Use case reference | Scenario | Horizontal accuracy | Availability | Heading | Latency for position estimation of UE | UE Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Factories of the Future 6.5 | Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | 99.9% | < 0,54 rad | < 1 s | N/A |
| Factories of the Future 6.7 | Mobile control panels with safety functions ( non-danger zones) | < 5 m | 90% | N/A | < 5 s- | N/A |
| Factories of the future 10.8 | Augmented reality in smart factories | < 1 m | 99% | < 0,17 rad | < 15 ms | < 10 km/h |
| Factories of the Future 13.3 | Process automation – plant asset management | < 1 m | 90% | N/A | < 2 s | < 30 km/h |
| Factories of the Future15.5 | Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of autonomous driving systems) ) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | 99.9% | N/A | 10 ms | < 30 km/h |
| Factories of the Future15.6 | Inbound logistics for manufacturing (for storage of goods) | < 20 cm | 99% | N/A | < 1 s | < 30 km/h |
| Factories of the Future18.19 | Flexible, modular assembly area in smart factories (for autonomous vehicles (only for monitoring proposes)) | < 50 cm | 99% | N/A | 1 s | < 30 km/h |
| Factories of the Future18.20 | Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1m (relative positioning) | 99% | N/A | 1 s | < 30km/h |

The positioning requirements defined in TR 22.804 [4] for most of the IIoT use cases are very challenging and thus it may not be practical to meet all of them in Rel-17. Note also that the requirements defined in TR 22.804 [4] are service requirements which are not meant to be fulfilled with RAT dependent positioning mechanisms alone but rather in combination with RAT independent positioning methods. In addition, it was agreed the in the SI, “Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case.” To evaluate whether the enhancements to Rel-16 positioning techniques meet the requirements, we would need first to define the target performance for Rel-17 in order to identify the performance gap.

The SID also provides the exemplary performance targets in the justification of the SI [1]: “NR Positioning in Rel-17 should evaluate and specify enhancements and solutions to meet the following exemplary performance targets:

(a) For general commercial use cases (e.g., TS 22.261):

- sub-meter level position accuracy (< 1 m)

(b) For IIoT Use Cases (e.g., 22.804):

- position accuracy < 0.2 m

The target latency requirement is < 100 ms; for some IIoT use cases, latency in the order of 10 ms is desired.”

Therefore, we may need first to define (or confirm) the target positioning requirements for Rel-17 for selected IIoT use cases, or to define the target positioning requirements for Rel-17 (e.g., using the exemplary performance targets as in the SID) without referring to specific IIoT use cases.

For the eight IIoT use cases are defined in TR 22.804 [4], the use cases of mobile control panels with safety functions (Factories of the Future 6.5 and 6.7) have very stringent requirements in terms of latency and service availability, and the use cases of the flexible, modular assembly area in smart factories(Factories of the Future18.19 and 18.20)  focuses on ultra-reliable wireless communication for a variety communication services adhering to negotiated and guaranteed QoS parameters. Therefore, positioning accuracy or latency is not the main requirements for the use cases of mobile control panels with safety functions, as well as the use cases of the flexible, modular assembly area in smart factories. The process automation – plant asset management (Factories of the Future 13.3) and inbound logistics for manufacturing (Factories of the Future15.5 and 15.6) should be selected as the target IIoT use cases. The positioning requirements for the use case of process automation – plant asset management are mainly based on the typical scenarios where IoT devices (e.g., sensors) are giving insight into process or environmental conditions or inventory of material, asset management or maintenance. For the use case of inbound logistics for manufacturing, its positioning requirements come from two scenarios; the first scenario is that driving trajectories of products for a supply-chain company moves products between multiple different independent manufacturing, distribution, and retail centres. And the second scenario is that storage of goods for tracking and inventory control purposes in local factory inventory management system.

It should be noted that only horizontal accuracy is defined in the Table 8.1.7, however, vertical accuracy is included in the SID [1] as the one of objectives of positioning enhancements, moreover, there are also the practical requirements for vertical positioning accuracy in IIoT scenarios. For example, in the scenario of multi-layer placement of goods in the logistics warehouse, vertical positioning is required for the tracking of goods in different layers of goods shelves. Rel-16 Positioning focuses on the study and evaluation of horizontal accuracy, so it will bring great challenges for Rel-17 Pos SI if vertical accuracy also needs to be evaluated, such as the base stations need to be multi-layer placement to carry out vertical positioning solutions.

In order to define one unified target positioning requirements for Rel-17, it is proposed the requirements of the three scenarios should be merged with appropriate modifications, including using the proper requirements among the three selected scenarios as the target positioning requirements and adding vertical accuracy requirement.

Proposal 1:  Select three IIoT use cases (Factories of the Future 13.3, 15.5 and 15.6) from Table 8.1.7 in TR 22.804 as the target IIoT use cases, and the target positioning requirements in Rel-17 should be defined based on the positioning requirements of  the selected IIoT use cases as follows:

| Use case reference | Scenario | Horizontal accuracy | Availability | Heading | Latency for position estimation of UE | UE Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Factories of the Future 13.3 | Process automation – plant asset management | < 1 m | 90% | N/A | < 2 s | < 30 km/h |
| Factories of the Future15.5 | Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of autonomous driving systems) ) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | 99.9% | N/A | 10 ms | < 30 km/h |
| Factories of the Future15.6 | Inbound logistics for manufacturing (for storage of goods) | < 20 cm | 99% | N/A | < 1 s | < 30 km/h |

Proposal 2:  Based on the positioning requirements of the three selected IIoT use cases, one unified target positioning requirements for Rel-17 is defined as follows:

| Requirements | Scenario | Horizontal accuracy | Vertical accuracy | Availability | Latency for position estimation of UE | UE Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Target positioning requirements in Rel-17 | - Process automation – plant asset management - Inbound logistics | < [20] cm | < [1]m | 90% | 100 ms | < 30 km/h |

Note: The final positioning requirements in Rel-17 may be modified based on the simulation evaluation.

As per the SID [1], besides positioning accuracy and latency, network efficiency and device efficiency aspects should also be studied in the study item. In order to improve network efficiency, the scalability and RS overhead should be studied. And for device efficiency, power consumption and complexity of devices should be also investigated.

For assessing scalability of positioning solutions, latency of positioning procedure should be studied as a function of number of devices to be positioned.  Average power consumption of devices should also be studied as a function of configured time and frequency resources for positioning. Such assessment would give a clear picture of scalability of a positioning solution while also providing metrics for power consumption of devices.

Proposal 3: For assessing scalability of positioning solutions, latency of positioning procedure should be studied as a function of number of devices to be positioned.

Proposal 4: Average power consumption of devices should be studied as a function of configured time and frequency resources for positioning.

IIoT InF scenarios

For the evaluation of positioning performance for the target IIoT scenarios, there is a need to define the details of the evaluation parameters, including the channel parameters for the IIoT scenarios based on TR 38.901[2], where the following 5 Indoor Factory (InF) scenarios are defined:

- InF-SL:	Indoor Factory with Sparse clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-DL:	Indoor Factory with Dense clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-SH:	Indoor Factory with Sparse clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-DH:	Indoor Factory with Dense clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-HH: Indoor Factory with High Tx and High Rx (both elevated above the clutter)

For the first four scenarios (i.e., InF-SL, InF-DL, InF-SH and InF-DH), large scale calibration has been completed in Rel-16 stage, including the calibration of coupling loss, geometry, delay spread, ASD, ZSD, ASA and ZSA [5]. A lot of scenarios parameters have been specified for calibration in 38.901 [2], such as BS height, clutter density, clutter height and clutter size, as shown in the Table 1 in below. Therefore, it is recommended to select one or more scenarios from the first four scenarios which had been calibrated in Rel-16 for positioning evaluation of Rel-17, so as to reduce the workload of determination of scenario parameters and calibration of simulations results.

Table 1   Scenario parameters for the indoor factory scenarios (InF-SL, InF-DL, InF-SH, InF-DH )

| Indoor Factory |  | Sparse clutter |  | Dense clutter |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Clutter density | 20% | Clutter density | 60% |
|  |  | Clutter height | 2m | Clutter height | 6m |
|  |  | Clutter size | 10m | Clutter size | 2m |
| Low BS height | 1.5 m | InF-SL (120 x 60m) |  | InF-DL (300 x 150m) |  |
| High BS height | 8 m | InF-SH (300 x 150m) |  | InF-DH (120 x 60m) |  |
| UE antenna height =1.5 m ; Room height =10m |  |  |  |  |  |

Above four InF scenarios in Table 1 can be considered for evaluation of NR positioning enhancements. In general, high BS height and sparse clutter will increase the possibility of LOS, but low BS height and dense clutter will play the opposite role. Therefore, the simulation results for InF-SH scenario can be considered as the upper bound in terms of positioning accuracy among all the four InF scenarios. As InF-DH and InF-SH scenarios are suitable for a lot of realistic indoor factory application scenarios, the InF-DH and InF-SH scenarios should be selected as the mandatory scenarios in the positioning evaluation in Rel-17. Other scenarios already defined in 38.901 can also be selected as optional scenarios for evaluation according to the interests of companies.

Proposal 5:  InF-DH and InF-SH scenarios should be selected as the mandatory scenarios for positioning evaluation in Rel-17. Other scenarios already defined in 38.901 can also be selected as optional scenarios for evaluation.

Evaluation parameters for IIoT scenarios

Evaluation parameters common to all scenarios

In Rel-16 the scenario parameters common to all scenarios for positioning evaluation are defined in Table 6.1.1-1 in TR 38.855 [3] (see Table 6.1.1-1 in Appendix 7.2), which include the carrier frequency, the PRS/SRS bandwidth, subcarrier spacing, gNB/UE noise figures, UE maximum TX power, UE antenna configuration, UE radiation pattern and network synchronization, etc. In addition, UE antenna radiation pattern model 1 (FR2) is defined in Table 6.1.1-2 in TR 38.855 [3] (see Table 6.1.1-2 in Appendix 7.2), which includes the antenna element radiation pattern, combining method for 3D antenna element pattern, and maximum directional gain of an antenna element GE, etc. These common parameters may be re-used for IIoT scenarios in the Rel-17 positioning evaluation.

Proposal 6: Reuse the common parameters defined in Table 6.1.1-1 in TR 38.855 (including Table 6.1.1-2 for UE radiation pattern in FR2) for IIoT scenarios.

Scenario parameters common to InF scenario(s)

The scenario parameters common to InF scenario(s) may be developed from the consideration of both the simulation assumptions for large scale calibration for InF scenarios in Table 7.8-7 in TR 38.901 (see Appendix 7.3), Table 7.2-4 in TR 38.901 (see Appendix 7.1) and the evaluation parameters for indoor office scenarios in Table 6.1.1-3 in TR 38.855 (see Appendix 7.2) as the starting point.

We have the following considerations for the Scenario parameters,

Deployment of UE and BS: We think it is reasonable to let UE antenna height to be uniformly distributed within a pre-defined range and BS is deployed at several levels whose height is fixed, in order to support the positioning evaluation of both horizontal accuracy and vertical accuracy. We are open to discuss asymmetrical location for the BSs or a denser BS grid with D is smaller than current 20m/50m.

LOS probability: LOS probability for InF scenarios should be modeled according to Section 7.4.2 in TR 38.901. As LOS probability is very low for DH scenario under default clutter parameters (ProbLOS,InF-DH<20% @ 50% CDF point), the three clutter parameters (Clutter density r; Clutter height hc ; Clutter size  dclutter) should be reviewed to check whether need to be relaxed to increase the LOS probability for DH scenario. Another issue of calculation of LOS probability is the parameter d2D, which represents 2-D distance between UE and TRP in horizontal dimension. If the vertical accuracy is agreed to be evaluated, d2D needs to be updated. E.g., d2D  represents the 3-D distance instead of 2-D distance.

Absolute time of arrival: As absolute time of arrival is important for positioning evaluation in Rel-17, the propagation time delay due to the total path length should considered in the fast fading model. We prefer to model absolute time of arrival for InF scenarios according to Section 7.6.9 in TR 38.901.

Blockage modeling: Blockage modelling is an add-on feature to the channel model. And the modeling of the blockage does not change LOS/NLOS state of each link. Two alternative models (Model A and Model B) are provided for the blockage modelling. Model A adopts a stochastic method while Model B adopts a geometric method for capturing human, vehicular blocking and Industrial clutters. We prefer not to introduce blockage modelling for positioning evaluation in Rel-17 as we need to spend a lot of time to discuss the details of blockage modelling and to calibrate the performance of the modelling.

Proposal 7: It is preferred to model absolute time of arrival for positioning evaluation in Rel-17.

Proposal 8: It is preferred not to introduce blockage modelling for positioning evaluation in Rel-17.

Proposal 9: The scenario parameters common to InF scenario(s) can be developed with the consideration of the parameters for InF scenarios defined in Table 7.8-7 in TR 38.901, Table 7.2-4 in TR 38.901 and the parameters for indoor office scenarios in Table 6.1.1-3 in TR 38.855, e.g., as shown in Table 2.

Table 2 Parameters common to InF scenario(s)

|  |  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- | --- |
| Channel model |  | [InF-SL, InF-DL, InF-SH, InF-DH and/or InF-HH] | [InF-SL, InF-DL, InF-SH, InF-DH and/or InF-HH] |
| Layout | Hall size | InF-SL: 120x60 m InF-DL: 300x150 m InF-SH: 300x150 m InF-DH: 120x60 m InF-HH: 300x150 m |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |
|  | Room height | 10m |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Penetration loss |  | 0dB |  |
| Number of floors |  | 1 |  |
| UE drop procedure |  | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility |  | 3km/h |  |
| UE antenna height |  | 1.5m |  |
| Min gNB-UE distance (2D), m |  | 0m |  |
| gNB antenna height |  | BS height = 1.5 m for InF-SL and InF-DL BS-height = 8 m for for InF-SH and InF-DH |  |
| Clutter density: r |  | Low clutter density: 20% High clutter density: 60% |  |
| Clutter height: hc |  | Low clutter density: 2 m High clutter density: 6 m |  |
| Clutter size: dclutter |  | Low clutter density: 10 m High clutter density: 2 m |  |
| LOS probability |  | LOS probability for InF scenarios is modeled according to Section 7.4.2 in TR 38.901 |  |
| Absolute time of arrival |  | Absolute time of arrival for InF scenarios is modeled according to Section 7.6.9 in TR 38.901 |  |
| Blockage modelling |  | We prefer not to model Blockage for InF scenarios |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |  |

DL PRS and UL SRS Configurations

In order to have a practical understanding on the achievable positioning performance with Rel-16 positioning technologies for the IIoT scenarios, we assume the simulation evaluation may use any, or a combination, of the Rel-16 positioning technologies (e.g., OTDOA, UTDOA, multi-RTT and UL AoA). In addition, given that the positioning performance depends heavily on the DL/UL time and frequency resources and/or the number of RSTD measurements configured for supporting the positioning solutions, there is also a need to have a common understanding on the reasonable DL PRS and UL SRS configurations for simulation evaluation. Section 4.3.1 and 4.3.2 give the recommended DL PRS and UL SRS configuration for simulation evaluation.

Proposal 10: A common understanding is needed on the reasonable DL PRS and UL SRS configurations for Rel-17 positioning simulation evaluation.

DL PRS Configuration for simulation evaluation

In this section, Table 3 presents some suggestions on DL PRS configurations for the purpose of the evaluation of the positioning performance as follows.

Table 3 DL PRS Configuration for Rel-17 positioning evaluation

| Parameters | Values | Additional Comments |
| --- | --- | --- |
| DL-PRS-Periodicity | [10]ms | FFS: [4, 5, 8, 10, 16, 20, 32, 40, 64, 80]ms |
| DL-PRS-ResourceRepetitionFactor | [1] | FFS: [2, 4, 6, 8, 16, 32] |
| DL-PRS-ResourceTimeGap | Not used in evaluation if DL-PRS-ResourceRepetitionFactor = 1 | FFS: [1, 2, 4, 6, 8, 16, 32] |
| DL-PRS-SFN0-Offset | 0 |  |
| DL-PRS-ResourceSetSlotOffset | Configured for avoiding/minimizing overlapping of the DL PRS from different TRPs |  |
| DL-PRS-ResourceId | Uniquely configured for each DL-PRS-Resource |  |
| DL-PRS-SequenceId | Uniquely configured for each DL-PRS-Sequence |  |
| DL-PRS-CombSizeN | N=[6] | FFS: N = [2, 4, 12] |
| DL-PRS-ReOffset | {0,1} for N = 2 {0,1, 2, 3} for N = 4 {0,1, 2, 3, 4, 5} for N = 6 {0,1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11}for N=12; | Configured up to companies to support avoiding/minimizing overlapping of the DL PRS from different TRPs |
| DL-PRS-ResourceSlotOffset | Configured for avoiding/minimizing overlapping of the DL PRS from different TRPs |  |
| DL-PRS-ResourceSymbolOffset | [0 to 13] | Configured up to companies |
| DL-PRS-NumSymbols | [2, 4, 6] | FFS: [12] |
| DL-PRS-QCL-Info | Not used in evaluation |  |
| DL-PRS-SubcarrierSpacing | [15/30] kHz for FR1 [120] kHz for FR2 | FFS: [60] kHz for FR1 FFS: [60] kHz for FR2 |
| DL-PRS-CyclicPrefix | Normal |  |
| DL-PRS-ResourceBandwidth | [50/100 MHz] with [15/30]kHz SCS for FR1 400 MHz with [120]kHz SCS for FR2 | FFS: [24, 28, 32, 36, 40, …, 268, 272] RBs |
| DL-PRS-StartPRB | Not used in evaluation |  |
| DL-PRS-PointA | Not used in evaluation |  |
| DL-PRS-expectedRSTD | +/- 1 us |  |
| DL-PRS-expectedRSTD-uncertainty | FR1/FR2: +/- 1 us FR2: +/- 1 us |  |
| DL-PRS-MutingPattern | can be used in evaluation | In case overlapping of the DL PRS from different TRPs cannot be avoided |
| DL-PRS-MutingBitRepetitionFactor | can be used in evaluation | In case overlapping of the DL PRS from different TRPs cannot be avoided |
| TRP ID | Uniquely configured for each TRP |  |
| ss-PBCH-BlockPower | Not used in evaluation |  |
| DL-PRS-ResourcePower | Determined based on the maximum TRP Tx power divided by the number of DL PRS REs |  |
| Number of DL PRS positioning frequency layer | 1 |  |
| NumDL-PRS-RSTD-MeasurementsPerTRPPair | [1] | FFS: [,2,3,4] Depending on the positioning algorithm |
| NumTrpPerPositioningFrequencyLayer | 18 | All TRPs in simulation layout |
| NumDL-PRS-ResourceSetsPerTRPFrequencyLayer | 1 |  |
| NumDL-PRS-ResourcesPerSet | Configured as nedded for the beamforming in the simulation evaluation |  |

UL SRS Configuration for simulation evaluation

In this section, Table 4 presents some suggestions on UL SRS configurations for the purpose of the evaluation of the positioning performance as follows.

Table 4 Configuration of SRS for Rel-17 positioning evaluation

| Parameters | Values | Additional Comments |
| --- | --- | --- |
| SRS-ResourceId | Uniquely configured for each SRS-sequence |  |
| transmissionComb; combOffset; cyclicShift | n4; combOffset (0,..,3); cyclicShift (0,…,11) | FFS: n2; combOffset (0,1); cyclicShift (0,…,7) FFS: n8; comboffset (0,..,7); cyclicShift (0,… 5) |
| resourceMapping nrofSymbols | [n12] | FFS: nrofSymbols : n1,n2,n4,n8, |
| freqDomainShift | Not used in simulation |  |
| freqHopping c-SRS | [23] for FR1(15kHz/30KHz); [33] for FR2(120kHz) | SRS bandwidths are selected in comparable to PRS bandwidths, e.g., 96 PRBs for FR1 (15kHz/30KHz SCS); 132 PRBs for FR2 (120kHz SCS) FFS: other values |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-Periodicity | [10]ms | SRS-Periodicity will match the DL PRS periodicity for supporting multiple-RTT FFS: other SRS periodicities |
| SRS-Offset | Configured for no overlapping of SRS-resources with other UEs |  |
| sequenceID | Uniquely configured for each SRS-sequence |  |
| spatialRelationInfo | Not considered in evaluation |  |
| srsResourceSetId | Uniquely configured for an SRS for positioning of a UE |  |
| srsResourceIdList | Uniquely configured for the SRS-Resources for positioning in an SRS-ResourceSet. |  |
| resourceType | periodic | FFS: periodic, semipersistent |
| Parameters related to pathloss reference | Not used in evaluation | Assume UE uses maximum power for SRS transmission |
| SFN0 Offset | 0 | Assume TRPs are SFN aligned |
| NumOfUL-PositioningResourcesPerSet | 16 | FFS: Other values |
| NumOfUL-PositioningResources | 16 | FFS: Other values |
| maxNumOfUL-PositioningResourceSets | 1 | FFS: Other values |

Conclusions

In this contribution, we discuss the IIOT use cases and related requirements, IIoT InF scenarios and associated evaluation parameter settings for the Rel-17 SI on NR Positioning Enhancements, and give the following proposals:

Proposal 1:  Select three IIoT use cases (Factories of the Future 13.3, 15.5 and 15.6) from Table 8.1.7 in TR 22.804 as the target IIoT use cases, and the target positioning requirements in Rel-17 should be defined based on the positioning requirements of  the selected IIoT use cases as follows:

| Use case reference | Scenario | Horizontal accuracy | Availability | Heading | Latency for position estimation of UE | UE Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Factories of the Future 13.3 | Process automation – plant asset management | < 1 m | 90% | N/A | < 2 s | < 30 km/h |
| Factories of the Future15.5 | Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of autonomous driving systems) ) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | 99.9% | N/A | 10 ms | < 30 km/h |
| Factories of the Future15.6 | Inbound logistics for manufacturing (for storage of goods) | < 20 cm | 99% | N/A | < 1 s | < 30 km/h |

Proposal 2:  Based on the positioning requirements of the three selected IIoT use cases, one unified target positioning requirements for Rel-17 is defined as follows:

| Requirements | Scenario | Horizontal accuracy | Vertical accuracy | Availability | Latency for position estimation of UE | UE Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Target positioning requirements in Rel-17 | - Process automation – plant asset management - Inbound logistics | < [20] cm | < [1]m | 90% | 100 ms | < 30 km/h |

Proposal 3: For assessing scalability of positioning solutions, latency of positioning procedure should be studied as a function of number of devices to be positioned.

Proposal 4: Average power consumption of devices should be studied as a function of configured time and frequency resources for positioning.

Proposal 5:  InF-DH and InF-SH scenarios should be selected as the mandatory scenarios for positioning evaluation in Rel-17. Other scenarios already defined in 38.901 can also be selected as optional scenarios for evaluation.

Proposal 6: Reuse the common parameters defined in Table 6.1.1-1 in TR 38.855 (including Table 6.1.1-2 for UE radiation pattern in FR2) for IIoT scenarios.

Proposal 7: It is preferred to model absolute time of arrival for positioning evaluation in Rel-17.

Proposal 8: It is preferred not to introduce blockage modelling for positioning evaluation in Rel-17.

Proposal 9: The scenario parameters common to InF scenario(s) can be developed with the consideration of the parameters for InF scenarios defined in Table 7.8-7 in TR 38.901, Table 7.2-4 in TR 38.901 and the parameters for indoor office scenarios in Table 6.1.1-3 in TR 38.855, e.g., as shown in Table 2.

Table 2 Parameters common to InF scenario(s)

|  |  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- | --- |
| Channel model |  | [InF-SL, InF-DL, InF-SH, InF-DH and/or InF-HH] | [InF-SL, InF-DL, InF-SH, InF-DH and/or InF-HH] |
| Layout | Hall size | InF-SL: 120x60 m InF-DL: 300x150 m InF-SH: 300x150 m InF-DH: 120x60 m InF-HH: 300x150 m |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |
|  | Room height | 10m |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Penetration loss |  | 0dB |  |
| Number of floors |  | 1 |  |
| UE drop procedure |  | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility |  | 3km/h |  |
| UE antenna height |  | 1.5m |  |
| Min gNB-UE distance (2D), m |  | 0m |  |
| gNB antenna height |  | BS height = 1.5 m for InF-SL and InF-DL BS-height = 8 m for for InF-SH and InF-DH |  |
| Clutter density: r |  | Low clutter density: 20% High clutter density: 60% |  |
| Clutter height: hc |  | Low clutter density: 2 m High clutter density: 6 m |  |
| Clutter size: dclutter |  | Low clutter density: 10 m High clutter density: 2 m |  |
| LOS probability |  | LOS probability for InF scenarios is modeled according to Section 7.4.2 in TR 38.901 |  |
| Absolute time of arrival |  | Absolute time of arrival for InF scenarios is modeled according to Section 7.6.9 in TR 38.901 |  |
| Blockage modelling |  | We prefer not to model Blockage for InF scenarios |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |  |

Proposal 10: A common understanding is needed on the reasonable DL PRS and UL SRS configurations for Rel-17 positioning simulation evaluation.

References

- RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”,V16.0.0 (2019-10)

- 3GPP TR 38.855, “Study on NR positioning support”,  V16.0.0 (2019-03)

- 3GPP TR 22.804, “Study on Communication for Automation in Vertical Domains”, V16.2.0 (2018-12)

- R1-1909704, “Indoor industrial channel model calibration result”, Ericsson, Reno, USA, May 13th –17th, 2019.

Appendix

Evaluation parameters for InF (TR 38.901)

Table 7.2-4: Evaluation parameters for InF (TR 38.901)

|  |  | InF |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters |  | InF-SL (sparse clutter, low BS) | InF-DL (dense clutter, low BS) | InF-SH (sparse clutter, high BS) | InF-DH (dense clutter, high BS) | InF-HH (high Tx, high Rx) |
| Layout | Room size | Rectangular: 20-160000 m2 |  |  |  |  |
|  | Ceiling height | 5-25 m | 5-15 m | 5-25 m | 5-15 m | 5-25 m |
|  | Effective clutter height hc | < Ceiling height, 0-10 m |  |  |  |  |
|  | External wall and ceiling type | Concrete or metal walls and ceiling with metal-coated windows |  |  |  |  |
| Clutter type |  | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Any |
| Typical clutter size, dclutter |  | 10 m | 2 m | 10 m | 2 m | Any |
| Clutter density r (percentage of surface area occupied by clutter) |  | Low clutter density (<40%) | High clutter density (≥40%) | Low clutter density (<40%) | High clutter density (≥40%) | Any |
| BS antenna height |  | Clutter-embedded, i.e. the BS antenna height is below the average clutter height |  | Above clutter |  | Above clutter |
| UT location | LOS/NLOS | LOS and NLOS |  |  |  | 100% LOS |
|  | Height | Clutter-embedded |  |  |  | Above clutter |

Common scenario parameters applicable for all scenarios (TR 38.855)

Table 6.1.1-1: Common scenario parameters applicable for all scenarios (TR 38.855)

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-2: UE antenna radiation pattern model 1 (FR2) (TR 38.855)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
|  | Note: are in local coordinate system. |

Table 6.1.1-3: Scenario 1. Indoor office scenario parameters (TR 38.855)

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Indoor floor: (12BSs per 120m x 50m), TRP number per floor:12, Inter-gNB distance = 20m - Note 1 |  |
| Total gNB TX power, dBm | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Channel model | Indoor open office – Note 2 |  |
| Peneteration loss | 0dB |  |
| Number of floors | 1 |  |
| UE drop procedure | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility | 3km/h |  |
| UE antenna height | 1.5m |  |
| Min gNB-UE distance (2D), m | 0m |  |
| gNB antenna height | 3m |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Simulation assumptions for calibration of the indoor factory scenario (TR 38.901)

Table 7.8-7: Simulation assumptions for large scale calibration for the indoor factory scenario(TR 38.901)

| Parameter | Values |
| --- | --- |
| Scenario | InF-SL, InF-DL, InF-SH, InF-DH |
| Hall size | InF-SL: 120x60 m InF-DL: 300x150 m InF-SH: 300x150 m InF-DH: 120x60 m |
| Room height | 10 m |
| Sectorization | None |
| BS antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| UT antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| Handover margin (for calibration) | 0dB |
| BS deployment | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m BS height = 1.5 m for InF-SL and InF-DL BS-height = 8 m for for InF-SH and InF-DH |
| UT distribution | uniform dropping for indoor with minimum 2D distance of 1 m UT height = 1.5 m |
| UT attachment | Based on pathloss |
| UT noise figure | 9 dB |
| Carrier frequency | 3.5 GHz, 28 GHz |
| Bandwidth | 100 MHz |
| Clutter density: r | Low clutter density: 20% High clutter density: 60% |
| Clutter height: hc | Low clutter density: 2 m High clutter density: 6 m |
| Clutter size: dclutter | Low clutter density: 10 m High clutter density: 2 m |
| Metrics | 1) Coupling loss – serving cell |
|  | 2) Geometry with and without noise |
|  | 3) CDF of delay and angle spread (ASD, ZSD, ASA, ZSA) according to the definition in Annex A.1 |
|  | 4) CDF of first path excess delay for serving cell |