3GPP TSG RAN WG1 #101	R1- 2003649

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	Discussion on the baseline performance and simulation  assumptions of coverage enhancement for FR1

Agenda Item:	8.4.1.1

Document for:	Discussion and Decision

- Introduction

At RAN#86 meeting, a new Rel-17 SI on NR coverage enhancement was approved [1]. One of the objective of this study item is to identify baseline coverage performance for both DL and UL for the target scenarios and services based on link-level simulation:

- UL channels (including PUSCH and PUCCH) are prioritized for FR1.

- Both DL and UL channels for FR2.

The target scenarios and services include

- Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

- Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

- TDD and FDD for FR1.

- VoIP and eMBB service for FR1.

- eMBB service as first priority and VoIP as second priority for FR2.

- LPWA services and scenarios are not included.

In this contribution, we provide our initial considerations on baseline performance and simulation assumptions of coverage enhancement for FR1.

- Discussions

  - The baseline performance for FR1

The baseline performance in FR1 can be defined as the MCL (Maximum Coupling Loss) from the link budget evaluation based on the given target data rate [3]. The related target data rates for downlink in Urban and Rural scenarios are provided in Table 1 according to the Rel-17 SID on NR coverage enhancement [1]. Accordingly, the target performance is the MPL (Maximum Path Loss) at the cell edge corresponding to the target ISD also shown in table 1 which reuses the assumptions in ITU [2].

Table 1: the target data rate and ISD requirements for different scenarios

| Frequency band | Scenarios | Duplexing scheme | Service | Target data rate | Target ISD |
| --- | --- | --- | --- | --- | --- |
| FR1 | Urban | FDD/TDD | VoIP, eMBB | DL 10Mbps, UL: 1 Mbps for eMBB DL/UL: 12.2 kbps for VoIP | ISD 1: 400m ISD 2: 500m |
|  | Rural | FDD/TDD | VoIP, eMBB | DL 1Mbps, UL: 100 kbps for eMBB DL/UL: 12.2 kbps for VoIP | ISD 1: 1732m ISD 2: 6000m ISD 3: 30Km |

Proposal 1: The baseline performance is the MCL from link budget evaluation based on target data rate.

Proposal 2: The target performance is the MPL at the cell edge corresponding to the target ISD.

  - LLS Simulation assumptions

To identify the bottleneck performance, we provide the detailed link level simulation assumptions to in order to obtain the required SNR corresponding to different scenarios. As a starting point, typical simulation assumptions as in Table 2 - Table 5 can be assumed for PUSCH/PUCCH/PDCCH/PDSCH on FR1.

Table 2: LLS assumptions for PDCCH on FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenarios | Urban | Rural | Rural with long distance |
| System Bandwidth | 20MHz | 20MHz | 20MHz |
| Carrier | 4G | 4GHz TDD/ 2GHz FDD | 700MHz |
| SCS | 30KHz | 30KHz for TDD 15KHz for FDD | 15KHz |
| Antenna configuration | 2T2R | 2T2R | 2T2R |
| Payload size (include 24 bits CRC) | 64bits | 64bits | 64bits |
| AL | 16 | 16 | 16 |
| Length of PDCCH | 2OS | 2OS | 2OS |
| Channel Coding | POLAR | POLAR | POLAR |
| Channel Model | TDL-C, 240ns for NLOS O2I | TDL-C, 37ns for NLOS O2O TDL-C, 34ns for NLOS O2I | TDL-D, 300ns |
| UE Mobility | 3km/h | 3km/h ，120 km/h | 120 km/h |
| Channel estimation | True | True | True |
| Target BLER | 10^-2 | 10^-2 | 10^-2 |

Table 3: LLS assumptions for PDSCH on FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenarios | Urban | Rural | Rural with long distance |
| System Bandwidth | 20MHz | 20MHz | 20MHz |
| Waveform | CP-OFDM | CP-OFDM | CP-OFDM |
| Carrier | 4G | 4GHz TDD/ 2GHz FDD | 700MHz |
| SCS | 30KHz | 30KHz for TDD 15KHz for FDD | 15KHz |
| Antenna configuration | 2T2R | 2T2R | 2T2R |
| Length of TTI | 14 OS | 14 OS | 14 OS |
| length of PDCCH | 3 OS | 3 OS | 3 OS |
| Length of DMRS | 1 OS | 1 OS | 1 OS |
| Occupied channel bandwidth | 51PRBs | 51 PRBs for 30KHz SCS 106 PRBs for 15KHz | 106 PRBs |
| MCS [Note1] | CR=602/1024,QPSK for eMBB CR=30/1024,QPSK for VOIP | For FDD 2GHz: CR=40/1024,QPSK for eMBB CR=30/1024,QPSK for VOIP For TDD 4GHz: CR=64/1024,QPSK for eMBB CR=30/1024,QPSK for VOIP | CR=40/1024,QPSK for eMBB CR=30/1024,QPSK for VOIP |
| Channel Coding | LDPC | LDPC | LDPC |
| Channel Model | TDL-C, 240ns for NLOS O2I | TDL-C, 37ns for NLOS O2O TDL-C, 34ns for NLOS O2I | TDL-D, 300ns |
| UE velocity | 3km/h | 3km/h for indoor, 120km/h for outdoor | 120km/h |
| Channel estimation | True | True | True |
| Target BLER | 10^-1 | 10^-1 | 10^-1 |
| Maximum number of transmission | 1 (No HARQ) | 1 (No HARQ) | 1 (No HARQ) |
| UL DL configuration if TDD is assumed | DDDSUDDSUU | DDDSUDDSUU | FDD for 700Mhz |
| Note 1: MCS index is calculated by the target data rate and the available resource for PDSCH excluding the PDCCH and DMRS resource. MCS table 3 is used. |  |  |  |

Table 4: LLS assumptions for PUCCH on FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and carrier frequency | 4GHz | TDD for 4GHz FDD for 2GHz | 700MHz |
| Format type | Format 1 (long PUCCH with 14 OFDM symbols), 1 bit | Format 1 (long PUCCH with 14 OFDM symbols), 1 bit | Format 1 (long PUCCH with 14 OFDM symbols), 1 bit |
| Subcarrier spacing | 30kHz | 30kHz for TDD 15kHz for FDD | 15kHz |
| Channel model | TDL-C, 240ns for NLOS O2I | TDL-C, 37ns for NLOS O2O TDL-C, 34ns for NLOS O2I | TDL-D, 300ns |
| Occupied channel bandwidth | 1 PRB | 1 PRB | 1 PRB |
| UE velocity | 3 km/h | 120 km/h for outdoor 3 km/h for indoor | 120 km/h |
| Antenna configuration | 1T2R | 1T2R | 1T2R |
| Channel estimation: | True | True | True |
| Frequency hopping | On | On | On |
| PUCCH repetition | Yes | Yes | Yes |

Table 5: LLS assumptions for PUSCH on FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and carrier frequency | TDD DDDSUDDSUU for 4GHz | TDD DDDSUDDSUU for 4GHz FDD for 2GHz | FDD for 700MHz |
| Subcarrier spacing | 30kHz | 30kHz for TDD 15kHz for FDD | 15kHz |
| Channel model | TDL-C, 240ns for NLOS O2I | TDL-C, 37ns for NLOS O2O TDL-C, 34ns for NLOS O2I | TDL-D, 300ns |
| Occupied channel bandwidth | 30 PRBs for eMBB 4 PRBs for VoIP | 4 PRBs for eMBB 4 PRBs for VoIP | 4 PRBs for eMBB 4 PRBs for VoIP |
| MCS [Note2] | CR=193/1024, QPSK for eMBB; CR=251/1024, QPSK for VoIP | For FDD 2GHz: CR=120/1024, QPSK for eMBB; CR=251/1024, QPSK for VoIP For TDD 4GHz: CR=157/1024, QPSK for eMBB; CR=251/1024, QPSK for VoIP | CR=120/1024, QPSK for eMBB; CR=251/1024, QPSK for VoIP |
| PUSCH length | 14 OS | 14 OS | 14 OS |
| DMRS length | 1 OS | 1 OS for 3km/h 2 OS for 120km/h | 2 OS |
| UE velocity | 3 km/h | 120 km/h for outdoor 3 km/h for indoor | 120 km/h |
| Occupied channel bandwidth | 30 PRBs for eMBB 4 PRBs for VoIP | 4 PRBs for eMBB 4 PRBs for VoIP | 4 PRBs for eMBB 4 PRBs for VoIP |
| Antenna configuration | 2T2R | 2T2R | 2T2R |
| Channel coding | LDPC | LDPC | LDPC |
| Channel estimation: | True | True | True |
| Target BLER | 10^-1 | 10^-1 | 10^-1 |
| Maximum number of transmission | 1 (No HARQ) | 1 (No HARQ) | 1 (No HARQ) |
| Note 2: The code rate and modulation order is calculated by the target data rate and the available resource for PUSCH excluding the DMRS resource. MCS table 1 is used. |  |  |  |

Required SINR for PDSCH/ PDCCH/ PUSCH/ PUCCH in different scenarios can be obtained based on the above LLS assumptions. According to the required SINR, the baseline performance for each channel can be calculated via the link budget evaluation used in ITU[3].

Proposal 3: The parameters provided in Table 2 - Table 5 can be used as starting point for LLS.

Furthermore, we think the following three issues are needed to be clarified in order to achieve a common understanding.

- How to understand the target data rate

As an example in Table 3, the target BLER for PDSCH is 10^-1 and the data rate is 10Mbps for Urban scenario. There are two different understanding on the relationship between data rate and the BLER:

- Understanding 1: Target data rate is defined without considering BLER. It means the data rate would be the ideal one assuming there are not error blocks. Taking table 3 as an example where the data rate is 10 Mpbs, the TBS used in LLS will be determined by 10Mbps.

- Understanding 2: Target data rate is defined with considering BLER. It means the data rate is calculated only based on the blocks which are correctly received. Taking table 3 as an example where the data rate is 10 Mpbs, the TBS used in LLS will be determined by (10/0.9) Mbps.

- Whether to define different target coverage for TDD and FDD separately

FDD can provide more resources for data transmission. In the other words, a smaller TBS can be achieved according to the same data rate compared to TDD. Hence the better coverage can be obtained in FDD system in theory if the same target data rate is assumed.

- How to understand the target data rate of VoIP

Currently, the data rate of VoIP is 12.2kps. However, as defined in [4], the 12.2kps is the data rate of voice bits without considering the headers. The header addition and round-up operations will result in a larger TBS which is delivered to physical layer. It is better to clarify how to calculate the TBS for VoIP in link level simulation.

Proposal 4: Clarify the following issues for the common understanding:

- How to understand the target data rate

- Whether to define different target coverage for TDD and FDD separately

- How to understand the target data rate of VoIP

- Conclusion

In this contribution, we provided our view on simulation assumptions for coverage enhancement for FR1 with the following proposals.

Proposal 1: The baseline performance is the MCL from link budget evaluation based on target data rate.

Proposal 2: The target performance is the MPL at the cell edge corresponding to the target ISD.

Proposal 3: The parameters provided in Table 2 - Table 5 can be used as starting point for LLS.

Proposal 4: Clarify the following issues for the common understanding:

- How to understand the target data rate

- Whether to define different target coverage for TDD and FDD separately

- How to understand the target data rate of VoIP

- References

  - RP-193240 New SID on NR coverage enhancement, 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9 – 12, 2019

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - R1- 2003648, Discussion about baseline coverage performance, CATT, RAN1#101

  - 3GPP TS 26.114, Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction, V16.3.0