3GPP TSG RAN WG1 Meeting #101-E						R1-2003755

e-Meeting, May 25th – June 5th, 2020

Source:	Intel Corporation

Title:	On UE features for Rel-16 eURLLC and IIoT

Agenda item:	7.2.11.5

Document for:	Discussion and Decision

# Introduction

In this contribution, we present our views on remaining details of RAN1 UE features for Rel-16 eURLLC and IIoT.

# Rel-16 UE features for eURLLC

Our views on some of the open issues related to Rel-16 eURLLC, based on the latest version of the UE features list [1], are listed below.

FG #11-1:

- Per-UE

- No xDD/FRx differentiation needed

FG #11-1a:

- Per-UE

- No xDD/FRx differentiation needed

FG #11-2:

- FSPC not necessary; FS is sufficient

- No xDD/FRx differentiation needed

FG #11-3:

- Remove component 3).

  - There is no need for this restriction. The impact from “too many PUCCHs” in a slot is mainly relevant to FG 11-4 and FG 11-4a, for which component 6) allows the UE to report a limited number of actual PUCCH transmissions per slot. Enforcing a gap between PUCCH transmissions is neither necessary nor beneficial. For instance, for the case of multiple PUSCHs in a slot, the only limitation is on the number of PUSCHs and there is no restrictions in terms of minimum gaps between two consecutive PUSCHs.

- Reporting is per FS, not per-UE

- No xDD/FRx differentiation needed

FG #11-4:

- Remove brackets for component 4) as this is perfectly aligned with RAN1 agreements

- An additional component should be added as following to cover the case wherein the UE may be configured with both sets of DCI formats 0_1/1_1 and 0_2/1_2:

  - Component 7) Supports a DCI format 1_2 scheduling PDSCH with different HARQ-ACK priorities and DCI format 1_1 scheduling PDSCH with low priority only when DCI formats 0_1/1_1 and DCI formats 0_2/1_2 are configured in a DL BWP

- Keep component 6)

- Reporting is per FS

- No xDD/FRx differentiation needed

- Remove 11-3 from pre-requisite

FG #11-4a:

- Remove brackets for component 4) as this is perfectly aligned with RAN1 agreements

- An additional component should be added as following to cover the case wherein the UE may be configured with both sets of DCI formats 0_1/1_1 and 0_2/1_2:

  - Component 7) Supports a DCI format 1_2 scheduling PDSCH with different HARQ-ACK priorities and DCI format 1_1 scheduling PDSCH with low priority only when DCI formats 0_1/1_1 and DCI formats 0_2/1_2 are configured in a DL BWP

- Keep component 6)

- Reporting is per FS

- No xDD/FRx differentiation needed

- Confirm FGs 11-3 and 11-4 as pre-requisites

FG #11-4b:

- The phrase “mixed DCI formats” should be clarified by replacing the component as: “Dynamic indication of high or low priority for HARQ-ACK feedback in DCI scheduling PDSCH when configured to monitor both sets of DCI formats 0_1/1_1 and 0_2/1_2 in a BWP DL priority indicationwith mixed DCI formats”.

- FGs 11-4 and 11-1 should be the pre-requisites; FG 11-1a should be changed to FG 11-1 (11-1a is about monitoring both sets of DCI formats in the same SS set, while this FG 11-4b is about monitoring both sets of DCI formats in the same DL BWP

FG #11-6:

- Pre-requisite should be FG 5-17

- Per UE

- No xDD/FRx differentiation

- Delete the text in the Notes column – no need for new component for supported number of PUSCH repetitions.

FG #11-7:

- Remove the last component in brackets

- For component 1), change from “[on the same CC as PUSCH or SRS]” to “on the same DL CC scheduling as PUSCH or SRS” as the current text is not correct or clear; with the change, the brackets can be removed

- Remove the text in Notes column

- Reporting should be per FS

- No xDD/FRx differentiation

FG #11-7a:

- Remove the last component in brackets

- Remove the text in Notes column

- Reporting should be per FS

- No xDD/FRx differentiation

FG #11-7b:

- Fine to add the FG, including suggested pre-requisites

FG #11-9:

- Keep components 2) and 3), but should be “configured”

- Pre-requisites should be FG 5-19 or 5-20

- Per UE

- No xDD/FRx differentiation

FG #11-9a:

- Pre-requisite should be FG 11-9

- Per UE

- No xDD/FRx differentiation

- Text in Notes column should be removed

FG #11-10:

- Pre-requisite should be FG 5-20

- Per UE

- No xDD/FRx differentiation

- Text in Notes column should be removed

FG #11-11:

- Pre-requisites should be FG 5-20 and 11-1

- Per UE

- No xDD/FRx differentiation

- Text in Notes column should be removed

# Rel-16 UE features for IIoT

Our views on some of the open issues related to Rel-16 IIoT, based on the latest version of the UE features list [2], are listed below.

FG #12-1:

- It is not necessary for FG 11-4 to be a pre-requisite for FG #12-1.

  - If a UE does not support FG 11-4, but supports FG 12-1, it means that PUCCH with HARQ-ACK is always of low priority, however, PUSCH can still be associated with low and high priorities for handling other overlap scenarios (including dropping of HARQ-ACK in case of overlap with PUSCH instead of multiplexing if PUSCH needs to be protected to satisfy reliability requirements).

- In the Note column, modify as: A UE supporting this feature shall also support the LCP restriction based on DCI priority indication ([lch-ToGrantPriorityRestriction-r16]) and intra-UE prioritization in MAC ([lch-PriorityBasedPrioritization-r16]). Technical reason below:

  - The support of LCP restriction based on DCI priority requires support of 12-1 as a pre-requisite, and this dependency is sufficient. A UE reporting support of FG #12-1 should not be mandated to also support DCI indication based LCP restriction.

- Reporting should be FS

- No xDD/FRx differentiation

FG #12-1a:

- Keep the FG with following updates

- Update the component description to avoid ambiguities and align with RAN1 agreement: “Dynamic indication of high or low priority for PUSCH in DCI scheduling the PUSCH when configured to monitor both sets of DCI formats 0_1/1_1 and 0_2/1_2 in a BWP UL priority indication in DCI with DCI format 0_1 and 0_2”.

- Pre-requisites should be FGs 11-1 and 12-1

- No xDD/FRx differentiation

FG #12-2:

- Per-UE

- No xDD/FRx differentiation

FG #12-2a:

- Per-UE

- No xDD/FRx differentiation

FG #12-3:

- Per-UE

- No xDD/FRx differentiation

- Text in Notes column should be removed

FG #12-3a:

- Per-UE

- No xDD/FRx differentiation

FG #12-5:

- Per-UE

- No xDD/FRx differentiation

FG #12-6:

- Per-UE

- No xDD/FRx differentiation

# Conclusions

In this contribution, we presented our views on defining RAN1 UE features for Rel-16 eURLLC and IIoT as listed in Sections 2 and 3.

# References

- R1-2003199, “Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR_L1enh_URLLC,” Moderator (NTT DOCOMO).

- R1-2003200, “Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR_IIoT,” Moderator (NTT DOCOMO).