3GPP TSG RAN WG1 #101	R1-2003894

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.11.2

Source:	Samsung

Title:	UE features for NR-U

Document for:	Discussion and Decision

# Introduction

This contribution provides our views on UE feature list for NR-U based on the outcome of email discussion [1].

# Discussion

As a conclusion of the email discussion of UE feature for NR-U, the updated UE feature list in [1] is suggested to be considered as a baseline for further discussion.

| 10-1 | UL channel access for dynamic channel access mode | 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol for PUSCH/PUCCH transmission | Per band | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| --- | --- | --- | --- | --- |

For FG 10-1, it is generic FG for dynamic channel access including various types of channel access mode. For Type 1 channel access, contention window size adjustment procedure should be required. However, it is not clearly stated in FG 10-1. During e-mail discussion, it was proposed to include contention window size adjustment procedure as a component in FG 10-1. Although Type 1 channel access in the component includes contention window size adjustment procedure, in our view, explicit description to avoid miss-leading of FG 10-1 is desirable. Therefore, it is proposed to include contention window size adjustment under the first component (i.e., Type 1 channel access) or as a separate component in FG 10-1.

Proposal 1: Include contention window size adjustment under the first component (i.e., Type 1 channel access) or as a separate component in FG 10-1

In the lists, there are many FGs with the statement, ‘This FG may be a part of basic operation for a particular scenario’ in Mandatory/Optional column. Examples are listed as below.

| 10-10 | RSSI and channel occupancy measurement and reporting | 1. RSSI measurement 2. Channel occupancy reporting | FFS: Per band or Per UE | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| --- | --- | --- | --- | --- |
| 10-11 | SRS starting position at any OFDM symbol in a slot | 1. Support transmitting SRS starting in all symbols (0,…,13) of a slot | FFS: Per band or Per UE | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10-20 | Support search space set configuration with freqMonitorLocation-r16 | 1. Support search space set configuration with freqMonitorLocations-r16 | Per band | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| … | … | … | … | … |

However, in our view, all of those FGs are not essential features to operate NR-U in a particular scenario. For example, FG 10-10 and FG 10-11, it is quite sure that those feature groups should not necessary regardless of deployment scenarios. Therefore, it is proposed to remove (or put FFS) the statements in Mandatory/Optional column or put FFS for the further discussion.

Proposal 2: Remove (or put FFS) ‘This FG may be a part of basic operation for a particular scenario’ for FGs other than FG 10-1 and FG 10-1a.

NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each.

Proposal 3: UE features for NR-U should be used only for unlicensed band.

# Conclusion

Proposal 1: Include contention window size adjustment under the first component (i.e., Type 1 channel access) or as a separate component in FG 10-1

Proposal 2: Remove (or put FFS) ‘This FG may be a part of basic operation for a particular scenario’ for FGs other than FG 10-1 and FG 10-1a.

Proposal 3: UE features for NR-U should be used only for unlicensed band.

# References

[1] R1-2003198, Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR-unlicensed, NTT DOCOMO, INC.