3GPP TSG RAN WG1 #101			R1-2004022

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	8.3.2

Source: 	LG Electronics

Title: 	Discussion on PDCCH monitoring for reduced capability NR device

Document for:	Discussion and decision

- Introduction

According to SID on support of reduced capability NR device, followings will be identified and studied in REDCAP SI;

Identify and study potential UE complexity reduction features, including [RAN1, RAN2]: - Reduced number of UE RX/TX antennas - UE bandwidth reduction Note: Rel-15 SSB bandwidth should be reused and L1 changes minimized - Half-Duplex-FDD - Relaxed UE processing time - Relaxed UE processing capability Study UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g., delay tolerant) [RAN2, RAN1]: - Reduced PDCCH monitoring by smaller numbers of blind decodes and CCE limits [RAN1] - Extended DRX for RRC Inactive and/or Idle [RAN2] - RRM relaxation for stationary devices [RAN2] Study functionality that will enable the performance degradation of such complexity reduction to be mitigated or limited, including [RAN1]: - Coverage recovery to compensate for potential coverage reduction due to the device complexity reduction Study standardization framework and principles for how to define and constrain such reduced capabilities – considering definition of a limited set of one or more device types and considering how to ensure those device types are only used for the intended use cases [RAN2, RAN1] Study functionality that will allow devices with reduced capabilities to be explicitly identifiable to networks and network operators, and allow operators to restrict their access, if desired [RAN2, RAN1]

In this contribution, we discuss considerations for studying the objectives from the perspective of PDCCH.

- Discussion

  - Potential UE complexity reduction features

Regarding the reduced number of UE RX/TX antenna, it can decrease power consumption and processing complexity, while coverage of PDCCH could be also decreased due to lack of signal power. So, if the reduced number of UE RX/TX is introduced for REDCAP UE, coverage recovery scheme for PDCCH should be also studied. For example, higher aggregation level larger than 16 or PDCCH repetition could be considered as a PDCCH coverage recovery scheme.

The bandwidth reduction is useful for power saving of receiving procedure, but PDCCH resources may be insufficient. According to the SID, minimum bandwidth for REDCAP UE may be as low as 5MHz (which is from Rel-15 SSB bandwidth), while AL 16 candidate needs 32 RBs when CORESET duration is configured to 3 symbols. It means higher aggregation level cannot be used on the reduced bandwidth, even though the higher ALs are needed for coverage recovery. Therefore, further studies for increasing CORESET resources on the top of reduced bandwidth seem to be needed. For example, extended CORESET in time domain (e.g., 4, 5, and/or 6-symbol CORESET) can be considered, and CORESET configuration (e.g., REG bundle size, CCE-to-REG mapping, interleaving parameter, etc.) for the extended CORESET could be studied further.

Observation 1: Impacts on PDCCH configuration caused by potential UE complexity reduction features should be studied, and compensation methods for the impacts should be also discussed.

  - Blind decoding/channel estimation complexity of PDCCH

In Rel-15, BD/CCE limit and corresponding PDCCH mapping rule were defined. The BD limit is the maximum number of monitored PDCCH candidates, and the CCE limit is the maximum number of non-overlapped CCEs (which are used for channel estimation of each PDCCH candidate). If the number of monitored candidates or non-overlapped CCEs is larger than the limit in a certain slot, a UE selects search space set(s) to be monitored based on the PDCCH mapping rule. In our understanding, this procedure can be also applied to the case of smaller number of BD/CCE limit, and there may be no critical problem if a network indicates search space set configuration (such as the number of candidates for each AL, monitoring periodicity …) properly.

Observation 2: If smaller number of BD/CCE limit is defined, corresponding PDCCH monitoring procedure could be supported by a current specification.

Regarding the channel estimation complexity, the hierarchical search space structure could be considered for reducing complexity. With the hierarchical structure, the channel estimation results of some candidates can be reused to decode other control channel candidates of other aggregation levels. In addition to hierarchical structure, DMRS mapping type can be also considered for decreasing channel estimation complexity. In NR, full-loaded DMRS (i.e., DMRS is located on all REGs) is used for control channel. However, if channel condition is stable enough (e.g., such as industrial wireless sensor, video surveillance, etc.), full-loaded DMRS requires more complex operation with similar performance compared to front-loaded DMRS (i.e., DMRS is located on first symbol of a CORESET). Thus, for REDCAP scenarios, front-loaded DMRS could be considered to decrease channel estimation complexity, and network can configure DMRS type considering channel condition/service type.

Proposal 1: In REDCAP scenarios, schemes which decrease channel estimation complexity of control channel (such as hierarchical search space structure, first-loaded DMRS, etc.) should be studied.

In NR, the channel estimation complexity of PDCCH is measured as the number of non-overlapped CCEs regardless of REG bundle size/precoder granularity. However, a matrix size for MMSE channel estimation is determined based on REG bundle size, for example, different number of multiplications are required for candidates with REG bundle size 2 and 6, respectively. In additiona to REG bundle size, regarding the precoder granularity, the wideband RS (i.e., precoderGranularity = allContiguousRBs) may be estimated by DFT-based channel estimation, while MMSE channel estimation could be used for narrowband RS (i.e., precoderGranularity = sameAsREG-bundle). From our perspective, it is not desirable to apply same criterion (i.e., number of CCEs) to the different channel estimation types.

In REDCAP, since smaller number of BD/CCE limit could be used, more accurate estimation for channel estimation complexity is needed for effective resource usage on a given limit. Therefore, further studies for calculation of channel estimation complexity seems to be needed.

Proposal 2: In REDCAP, the channel estimation complexity could be measured considering REG bundle size and precoder granularity.

- Conclusion

In this contribution,

Observation 1: Impacts on PDCCH configuration caused by potential UE complexity reduction features should be studied, and compensation methods for the impacts should be also discussed.

Observation 2: If smaller number of BD/CCE limit is defined, corresponding PDCCH monitoring procedure could be supported by a current specification.

Proposal 1: In REDCAP scenarios, schemes which decrease channel estimation complexity of control channel (such as hierarchical search space structure, first-loaded DMRS, etc.) should be studied.

Proposal 2: In REDCAP, the channel estimation complexity could be measured considering REG bundle size and precoder granularity.