3GPP TSG RAN WG1 Meeting #101-e 		 		R1-2004137

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.1.11.1

Source: 	LG Electronics

Title: 	Discussion on UE features for NR 2step RACH

Document for:	Discussion and Decision

- Introduction

In this document, we will discuss about remaining issues about UE features for 2-step RACH such as operation in CA / SUL.

- Discussion

  - UE features list

After previous email discussion [1], there are some controversial issue as shown in below:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 9. NR_2step_RACH | [9-3] | [Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA] | [Parallel MsgA and SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell] |
| 9. NR_2step_RACH | [9-4] | [MsgA operation in a band combination including SUL] | [MsgA operations in a band combination including SUL] |

For 4-step RACH, parallel transmission of PRACH and other UL signals (e.g. PUSCH/PUCCH/SRS) is already supported. Likewise, even if TA = 0 is applied for msgA PUSCH which is different from PUSCH, there is no problem because PRACH is transmitted before certain timing gap. Furthermore, it is clear that the UE who doesn’t support 2-step RACH can try to initial access with 4-step RACH in case of CA / SUL. So, parallel transmission of msgA with other signals and msgA operation in SUL are needed.

Proposal 1: Parallel transmission of msgA with other signals and msgA operation in SUL are needed.

Conclusion

In this contribution, we discussed the remaining detail about UE feature list. As a conclusion of the discussion, we summarize our views as follows:

Proposal 1: Parallel transmission of msgA with other signals and msgA operation in SUL are needed.

Reference

- [1] R1-2003197, “Summary on 100b-e-NR-UEFeatures-Remaining”, RAN1, NTT DOCOMO