3GPP TSG RAN WG1 Meeting #101-e	R1-2004165

E-meeting, May 25 – June 5, 2020

Agenda Item:	6.2.1.4

Source:	Huawei, HiSilicon

Title:	Corrections on eMTC co-existence with NR

Document for:	Discussion and Decision

# Introduction

This paper discusses the corrections for the coexistence of eMTC and NR based on the latest Rel-16 spec.

# Remaining issues

In 36.331, the framework of resource reservation has been captured as below:

-- ASN1START NR-ResourceReservationConfig-r16 ::= SEQUENCE { periodicity-r16 ENUMERATED {ms10, ms20, ms40, ms80, ms160} OPTIONAL, startPosition-r16 INTEGER (0..15) OPTIONAL, resourceReservationFreq-r16 CHOICE { rbg-bw1dot4MHz BIT STRING (SIZE (6)), rbg-bw3MHz BIT STRING (SIZE (8)), rbg-bw5MHz BIT STRING (SIZE (13)), rbg-bw10MHz BIT STRING (SIZE (17)), rbg-bw15MHz BIT STRING (SIZE (19)), rbg-bw20MHz BIT STRING (SIZE (25)) } OPTIONAL, -- Cond DL slotConfig-r16 SEQUENCE { slotBitmap-r16 CHOICE { slotPattern10ms BIT STRING (SIZE (20)), slotPattern40ms BIT STRING (SIZE (80)) } OPTIONAL, -- Cond FDD-OR-TDD-DL symbolBitmap1-r16 BIT STRING (SIZE (7)) OPTIONAL, symbolBitmap2-r16 BIT STRING (SIZE (7)) OPTIONAL } OPTIONAL, ... } -- ASN1STOP Conditional presence; Explanation. DL; The field is mandatory present if NR-ResourceReservationConfig configures downlink parameters; otherwise the field is not present.. FDD-OR-TDD-DL; The field is mandatory present for FDD and mandatory present for TDD downlink; otherwise the field is not present..

However, the terms used in 36.211/212/213 are not aligned with it, therefore, the following text proposal is proposed to remove any potential ambiguity:

Proposal 1: Endorse the following text proposals.

==============================Start of text proposal to 36.211=======================

### 6.4.1	Physical downlink shared channel for BL/CE UEs

<unchanged parts are omitted>

For BL/CE UEs, if the PDSCH is not carrying SIB1-BR the PRB resources for PDSCH transmission in the first subframe are obtained from the DCI as described in clauses 5.3.3.1.12, 5.3.3.1.13, and 5.5.1.3.14 in [3], or provided by higher layers. Each of the NTB≥1 PDSCH codewords is transmitted with  repetitions, where NTB is the number of transport blocks defined in clause 7.1.11 of 3GPP TS 36.213 [4]. The PDSCH transmission spans NabsPDSCH≥NTBNrepPDSCH consecutive subframes, including subframes that are not BL/CE DL subframes where the the PDSCH transmission is postponed.

- If higher layer parameter resourceReservationFreq is configured, and the Resource reservation field in the DCI is set to 1, then in case of PDSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space,

-	In a subframe that is fully reserved as defined in clause 7.1 in [4], the PDSCH transmission is postponed until the next BL/CE downlink subframe that is not fully reserved.

-	In a subframe that is partially reserved, the reserved resource elements shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

<unchanged parts are omitted>

### 6.8B.5	Mapping to resource elements

<unchanged parts are omitted>

- -	The narrowband  for MPDCCH transmission in the first subframe of MPDCCH search space is provided by higher layers. Starting subframe configuration of a search space where UE monitors an MPDCCH is also provided by higher layers. The MPDCCH search space uses  subframes, spanning  consecutive subframes, including subframes that are not BL/CE DL subframes where the MPDCCH transmission is postponed.

-	If higher layer parameter resourceReservationFreq is configured, then in case of MPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space,

-	In a subframe that is fully reserved as defined in clause 7.1 in [4], the MPDCCH transmission is postponed until the next BL/CE downlink subframe that is not fully reserved.

-	In a subframe that is partially reserved, the reserved resource elements shall be counted in the MPDCCH mapping but not used for transmission of the MPDCCH.

<unchanged parts are omitted>

#### 6.10.3A.2	Mapping to resource elements

<unchanged parts are omitted>

For BL/CE UEs, if higher layer parameter resourceReservationFreq is configured, then in case of MPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space,

- -	If all OFDM symbols in a PRB are reserved, the demodulation reference signal transmission in that PRB is dropped.

==============================End of text proposal to 36.211=======================

==============================Start of text proposal to 36.212=======================

##### 5.3.3.1.12	Format 6-1A

<unchanged parts are omitted>

-	Resource reservation – 1 bit as defined in clause 7.1 of [3]. This field is only present if higher layer parameter resourceReservationFreq is configured and the DCI is mapped onto the UE-specific search space given by C-RNTI as defined in [3].

<unchanged parts are omitted>

##### 5.3.3.1.13	Format 6-1B

<unchanged parts are omitted>

-	Resource reservation – 1 bit as defined in clause 7.1 of [3]. This field is only present if higher layer parameter resourceReservationFreq is configured and the DCI is mapped onto the UE-specific search space given by C-RNTI as defined in [3].

==============================End of text proposal to 36.212=======================

==============================End of text proposal to 36.213=======================

## 7.1	UE procedure for receiving the physical downlink shared channel

<unchanged parts are omitted>

For BL/CE UEs, the set of BL/CE DL subframes is indicated as follows

-	If higher layer parameter resourceReservationFreq is configured,

-	for PDSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space,

-	if the Resource reservation field in the DCI is set to 0, then the set of BL/CE DL subframes corresponds to all downlink subframes during the PDSCH transmission;

-	if the Resource reservation field in the DCI is set to 1, then the set of BL/CE DL subframes corresponds to all downlink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols of all PRBs of the PDSCH transmission are reserved in the subframe);

-	for MPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space,

-	the set of BL/CE DL subframes corresponds to all downlink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols of all PRBs of the MPDCCH transmission are reserved in the subframe).

-	In all other cases, the set of BL/CE DL subframes is indicated by the higher layers according to fdd-DownlinkOrTddSubframeBitmapBR [11].

==============================End of text proposal to 36.213=======================

# Conclusion

In this paper, we discuss the corrections for the coexistence of eMTC and NR with following proposals.

Proposal 1: Endorse the text proposal in section 2.

# References

- 3GPP TS 36.331, Radio Resource Control (RRC); V16.0,0, March, 2020.