3GPP TSG-RAN/WG1 Meeting #101	R1-2004321

# e-Meeting,   May 25th – June 5th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | <CR#> | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on rate matching output sequence length |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In subclause 6.3.1.2.1 of 38.212, the rate matching output sequence length (E) is referred to the one defined in subclause 6.3.1.4.1 of 38.212. However, there are two definitions for the ratematching output sequence length in the subclause 6.3.1.4.1, EUCI and Er, as shown in highlighted below. ##### 6.3.1.4.1 UCI encoded by Polar code The input bit sequence to rate matching is where is the code block number, and is the number of coded bits in code block number . Table 6.3.1.4.1-1: Rate matching output sequence length UCI(s) for transmission on a PUCCH; UCI for encoding; Value of. HARQ-ACK; HARQ-ACK. HARQ-ACK, SR; HARQ-ACK, SR. CSI (CSI not of two parts); CSI. HARQ-ACK, CSI (CSI not of two parts); HARQ-ACK, CSI. HARQ-ACK, SR, CSI (CSI not of two parts); HARQ-ACK, SR, CSI. CSI (CSI of two parts); CSI part 1. CSI part 2. HARQ-ACK, CSI (CSI of two parts); HARQ-ACK, CSI part 1. CSI part 2. HARQ-ACK, SR, CSI (CSI of two parts); HARQ-ACK, SR, CSI part 1. CSI part 2. Rate matching is performed according to Subclause 5.4.1 by setting and the rate matching output sequence length to , where is the number of code blocks for UCI determined according to Subclause 6.3.1.2.1 and the value of is given by Table 6.3.1.4.1-1: - is the number of bits for HARQ-ACK for transmission on the current PUCCH; - is the number of bits for SR for transmission on the current PUCCH; - is the number of bits for CSI part 1 for transmission on the current PUCCH; - is the number of bits for CSI part 2 for transmission on the current PUCCH; - if , ; otherwise, is the number of CRC bits determined according to subclause 6.3.1.2.1, where equals for "CSI (CSI of two parts)", equals for "HARQ-ACK, CSI (CSI of two parts)", and equals for "HARQ-ACK, SR, CSI (CSI of two parts)" respectively in Table 6.3.1.4.1-1;; - is the configured maximum PUCCH coding rate; - is given by Table 6.3.1.4-1. The output bit sequence after rate matching is denoted as where is the length of rate matching output sequence in code block number .. The E in 6.3.1.2.1 is used to determine whether the code block segmentation is performed or not,and this is exactly same meaning of EUCI, while the Er is the rate matching output sequence length per a codeblock after code block segmentation. Therefore the E should be replaced with EUCI to avoid ambiguity. |
| Summary of change: |  | Change the symbol E to EUCI in sublause 6.3.1.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There remain two definitions for the rate matching output sequence length, and thus a UE who refers E to Er other than EUCI may mis-calculate the size of the CRC bits, and furthermore, wrongly perform code block segmentation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR resolves a right reference for the rate matching output sequence length. A reasonable UE implementation without this change request already implemented based on the RAN1’s common understanding. Therefore, this CR is backward-compatible. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

6.3.1.2.1	UCI encoded by Polar code

If the payload size , code block segmentation and CRC attachment is performed according to Subclause 5.2.1. If ( and EUCI≥1088) or if , ; otherwise , where EUCI is the rate matching output sequence length as given in Subclause 6.3.1.4.1.

[…]

< Unchanged parts are omitted >