3GPP TSG RAN WG1 #101 e-Meeting	R1-2004332

May 25th – June 5th, 2020

Source:	Sharp

Title:	Remaining issues on PUSCH enhancements for NR URLLC

Agenda Item:	7.2.5.3

Document for:	Discussion and Decision

Introduction

In RAN1 previous meetings, some agreements and conclusions were made [1]. There are still remaining issues on enhancement to URLLC PUSCH and we show our views in this contribution.

Discussions

A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH

In last e-meeting, following was proposed though it has not been concluded [2].

- Proposal 1a: For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition, and other actual repetitions are discarded. Down-select among the following: - Option A: there is no scheduling restriction. - Option B: UE is not expected to be indicated with numberofrepetitions >1. - Option C: The first nominal repetition is not expected to be segmented into multiple actual repetitions.

Firstly, this proposal proposes that “A-CSI/SP-CSI is transmitted on the first actual repetition without any repetitions”. The definition of “actual repetition” in current specification is “the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot.” Therefore, even for option C, there is no issue to say “actual repetition” since “first nominal repetition” is same with “first actual repetition” in option C.

Proposal 1:

- For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition, and other actual repetitions are discarded

For down-selection among 3 options, either option is acceptable for us since the restriction of option B and option C can be achieved by gNB scheduling. The differences among 3 options are the operation in case of TDRA with numberofrepetition > 1 and the operation of resources with segmentation.

- Option A: there is no scheduling restriction.

  - TDRA with numberofrepetition > 1: ignoring the value of numberofrepetition

  - Resource with segmentation: using first actual repetition

    - pros: less scheduling restriction

    - cons: spec impact to ignore the number of repetitions indicated by DCI

- Option B: UE is not expected to be indicated with numberofrepetitions >1.

  - TDRA with numberofrepetition > 1: error case

  - Resource with segmentation: using first actual repetition

    - pros: less specification effort

    - cons: limited available TDRA entries

- Option C: The first nominal repetition is not expected to be segmented into multiple actual repetitions.

  - TDRA with numberofrepetition > 1: ignoring the value of numberofrepetition

  - Resource with segmentation: error case

    - pros: enough REs to transmit A-CSI/SP-CSI

    - cons: limited available TDRA entries

From specification impact perspective, our preference is option B since the requirement of scheduling flexibility is not high from the nature of A-CSI/SP-CSI perspective. However, option A is also OK if current specification description is also acceptable for Rel-16 to avoid the repetition for CSI.

Proposal 2:

- For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, UE is not expected to be indicated with numberofrepetitions >1

UCI multiplexing on PUSCH for PUSCH repetition Type B

In last e-meeting, following was proposed though it has not been concluded [3].

- Proposal 1-4a: An actual repetition of a single symbol in case of L>1 is not considered for UCI multiplexing.

In current specification describes that “An actual repetition with a single symbol is omitted except for the case of L=1” and there is no issue to exclude the single symbol case at least for L>1.

Proposal 3:

- An actual repetition of a single symbol in case of L>1 is not considered for UCI multiplexing

Conclusion

In this contribution, we have following observations and conclusions

Proposal 1:

- For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition, and other actual repetitions are discarded

Proposal 2:

- For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, UE is not expected to be indicated with numberofrepetitions >1

Proposal 3:

- An actual repetition of a single symbol in case of L>1 is not considered for UCI multiplexing

References

RAN1 Chairman’s note, RAN1#100b-e

Apple, R1-2003006, “Summary of Email discussion [100b-e-NR-L1enh-URLLC-PUSCH-04] on PUSCH enhancements for NR eURLLC (AI 7.2.5.3)”, RAN1#100b-e

Apple, R1-2003004, “Summary of Email discussion [100b-e-NR-L1enh-URLLC-PUSCH-02] on PUSCH enhancements for NR eURLLC (AI 7.2.5.3)”, RAN1#100b-e