3GPP TSG-RAN WG1 Meeting # 101-e	R1-20xxxxx

# E-meeting,  May 25th – June 5th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | XX | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on conditions of rate matching pattern overlapping with PDSCH DMRS symbols |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS 38.214, a UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any REs not available for PDSCH. Such unavailable REs at least include REs corresponding to PDSCH rate matching patterns and SSB. In other words, the network shall not schedule or configure a PDSCH whose DM-RS would be overlapped with either rate matching patterns or SSB, which will cause significant restrictions on scheduling. As a consequence, in the slot containing rate matching patterns or SSB, full bandwidth part scheduling for PDSCH is impossible for a UE, resulting in substantial performance loss in downlink throughput. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct to allow UE to handle the overlapping case between PDSCH DM-RS and REs corresponding to rate matching patterns or SSB rather than treating it as an error case. Specifically, a UE shall assume that the PRBs are not available for PDSCH and DM-RS in all symbols scheduled or configured for the PDSCH within a single slot if PDSCH DM-RS REs in the PRBs would overlap, even partially, with REs not available for PDSCH and the number of resulting fractional PRGs is less than or equal to 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the slot containing rate matching patterns or SSB, PDSCH scheduling for a UE with full-bandwidth DCI indication is impossible, thus resulting in substantial performance loss in PDSCH throughput and strict restriction to RM pattern applications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact on rate-matching patterns and SSB for PDSCH. Since it is only for TEI16, there is no interoperability issue. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

### 5.1.4	PDSCH resource mapping

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1.

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE is not expected to handle the case where PDSCH DM-RS REs are partially overlapping with any RE(s) not available for PDSCH. A UE shall assume that a PRB indicated by the frequency domain resource assignment field in a DCI for PDSCH is not available for both PDSCH and DM-RS in all symbols scheduled by the DCI if all PDSCH and DM-RS REs in the PRB in all scheduled symbol overlap with any RE(s) not available for PDSCH. If PRG size  is determined as "wideband", the UE is not expected to be scheduled with any PRB not available for PDSCH DM-RS in any scheduled symbol. If  is determined as one of the values among {2, 4}, the UE is not expected to be scheduled with more than 4 PRGs which each contains at least one PRB not available for PDSCH DM-RS in any scheduled symbol.

<Unchanged parts are omitted>