3GPP TSG-RAN WG1 Meeting #101-e	R1-2004643

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.8.1

Source:	Ericsson

Title:	maintenance of DL reference signals for NR positioning

Document for:	Discussion, Decision

# Introduction

This contribution addresses maintenance issues pertaining to DL reference signals for NR positioning.

# Parameter alignments for DL PRS

## PRS comb offset

In 38.211, the PRS comb offset is given by the higher-layer parameter combOffset. In the latest available version of 38.357, the same parameter is refered to as dl-PRS-ReOffset-r16.  The proposed TP#1 aligns both specifications.

TP 1: changes to 38.211 for PRS resource element offset

***** unchanged text omitted *********** ##### 7.4.1.7.3 Mapping to physical resources in a downlink PRS resource ***** unchanged text omitted *********** - the resource-element offset koffsetPRS∈0,1,…,KcombPRS-1 is given by the higher-layer parameter dl-PRS-ReOffset-r16; ***** unchanged text omitted ***********

Endorse TP#1 for 38.211

## PRS muting

The text in 38.211 section 7.4.1.7.4 on muting requires correction to be grammatically correct. The proposed TP#2 corrects this.

TP 2:  description of PRS muting in 38.211

##### 7.4.1.7.4 Mapping to slots in a downlink PRS resource set ***** unchanged text omitted *********** - the higher-layer parameter mutingOption1-r16 is provided with bitmap b1 but mutingOption2-r16 with bitmap b2 is not provided, and bit bi1 is set; - the higher-layer parameter mutingOption2-r16 is provided with bitmap b2 but not mutingOption1-r16 with bitmap b1 is provided, and bit bi2 is set; ***** unchanged text omitted ***********

Endorse TP#2 for 38.211

In 38.214, the description of PRS muting does not include the name of the different parameters for the bitmap and the muting options. We propose to complete the text with the parameter names:

TP 3:  description of PRS muting in 38.214

#### 5.1.6.5 PRS reception procedure ***** unchanged text omitted *********** dl-PRS-MutingPatternList-r16defines a bitmap of the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. The bitmap is configured by the parameter mutingPattern-r16 with a size {2, 4, 6, 8, 16, 32} bits long. The bitmap has two options for applicability. In the first option set by the parameter mutingOption1-r16 each bit in the bitmap corresponds to a configurable number provided by higher layer parameter dl-PRS-MutingBitRepetitionFactor-r16of consecutive instances of a DL PRS resource set where all the DL PRS resources within the set are muted for the instance that is indicated to be muted. In the second option set by the parameter mutingOption2-r16 each bit in the bitmap corresponds to a single repetition index for each of the DL PRS resources within each instance of a nr-DL-PRS-ResourceSet-r16and the length of the bitmap is equal to the values of dl-PRS-ResourceRepetitionFactor-r16. Both options may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in Clause 7.4.1.7.4 of [4, TS 38.211]. ***** unchanged text omitted ***********

Endorse TP#3 for 38.214

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Endorse TP#1 for 38.211

Proposal 2	Endorse TP#2 for 38.211

Proposal 3	Endorse TP#3 for 38.214