3GPP TSG-RAN WG1 Meeting #101-e	R1-2004902

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	6.2.2.1

Source:	Ericsson

Title:	TP for 36.211 on common WUS for LTE-MTC and NB-IoT

Document for:	Discussion, Decision

# Introduction

RAN1 #101-e discussed the common WUS for both LTE-MTC and NB-IoT as presented in the feature lead summary REF. This contribution presents a text proposal based on the discussion.

| Reason for change: | To better align the terminology in specs 36.211 and 36.331. |
| --- | --- |
| Summary of change: | More accurate referencing to parameter configured by higher layers. |
| Consequences if not approved: | Risk of misinterpreting the spec. |
| Clauses affected: | 6.11B.1, 10.2.6B.1 |

# Text proposal for TS 36.211

## 6.11B.1	Sequence generation

<Unchanged parts are omitted>

For a UE not configured with group MWUS, g=0. For a UE configured with group MWUS, g=14NgroupWUS+1 for 0≤NgroupWUS≤7, where NgroupWUS is determined by the UE group to which the UE is associated as determined by higher layers [10]. In a resource that is not shared with non-group MWUS, the common MWUS sequence shall be determined by  g=126. In the resource that is shared with non-group MWUS, the common MWUS sequence is determined by higher layers [9]. g=0

<Unchanged parts are omitted>

## 10.2.6B.1	Sequence generation

<Unchanged parts are omitted>

For a UE not configured with group NWUS, g=0. For a UE configured with group NWUS, g=14NgroupWUS+1 for 0≤NgroupWUS≤7, where NgroupWUS is determined by the UE group to which the UE is associated as determined by higher layers [10]. In a resource that is not shared with non-group NWUS, the common NWUS sequence shall be determined by  g=126. In the resource that is shared with non-group NWUS, the common NWUS sequence is determined by higher layers [9]. g=0

<Unchanged parts are omitted>

# References

R1-2004832, Feature lead summary #2 of Group WUS for NB-IoT, Moderator (Ericsson), RAN1 #101-e, May 2020.