3GPP TSG RAN WG1 #101-e						   	                R1-2005000

May 25th – June 5th, 2020

Agenda item:	7.2.4.4

Source: 	Moderator (Qualcomm)

Title: 	Text Proposals for In-device Coexistence Aspects in NR-V2X

Document for:	Discussion and Decision

# Introduction

This document provides text proposals for the list of issues identified during the preparation pertaining to the coexistence aspects (AI 7.2.4.4) of NR V2X.

[101-e-NR-5G_V2X_NRSL-InDevice-Coex-01] Email discussion/approval  to TPs for the following issues with already existing agreements starting from the TPs in R1-2003064:

- PSFCH priority is set to the priority of the corresponding PSSCH.

- Prioritization is performed across RATs, not within a RAT, for in-device coexistence.

- When multiple transmissions using NR sidelink are overlapped with transmission/reception using LTE sidelink, and the priorities of multiple transmissions are different, the highest priority of multiple transmissions using NR sidelink is used for compared with transmission/reception using LTE sidelink

By 5/29 – Gabi (Qualcomm)

# Text Proposal to Capture PSFCH Priority

| Reason for change: | Capturing agreement from RAN1 #98bis |
| --- | --- |
| Summary of change: | Capture that PSFCH is considered when performing prioritization for in-device coexistence and that its priority is determined by the SCI scheduling the associated PSSCH |
| Consequences if not approved: | Potential ambiguity about whether PSFCH is considered for in-device coexistence between NR sidelink and E-UTRA sidelink |
| Clauses affected: | 16.2.4.1 |

----------------------------------------------------begin text proposal for 38.213----------------------------------------------------

16.2.4.1 Simultaneous NR and E-UTRA transmission/reception

If a UE

-	would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and

-	a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and

-	the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions

the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

If a UE

-	would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and

-	a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and

-	the priorities of the two channels/signals are known to the UE T msec prior to the start of the later transmission or reception

the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers or, in case of PSFCH, equal to the priority of the corresponding PSSCH.

-----------------------------------------------------end text proposal for 38.213-----------------------------------------------------

# Text Proposal to Correct Synchronization Signal Prioritization

| Reason for change: | Correct specification regarding agreement from RAN1 #98bis |
| --- | --- |
| Summary of change: | Capture that NR S-SS/PSBCH block priority is compared with transmissions on E-UTRA sidelink and that the E-UTRA sidelink synchronization signal priority is compared with transmissions on NR sidelink. |
| Consequences if not approved: | Contradiction in specifications and potentially incorrect prioritization of NR S-SS/PSBCH with other NR sidelink signals/channels. Contradiction in specifications and potentially incorrect prioritization of E-UTRA sidelink synchronizations signals with other E-UTRA sidelink signals/channels. |
| Clauses affected: | 16.1 |

----------------------------------------------------begin text proposal for 38.213----------------------------------------------------

16.1	Synchronization procedures

<<<unchanged text omitted>>>

If a UE would transmit or receive an S-SS/PSBCH block , and the transmission or reception would overlap in time with transmissions and/or receptions on the sidelink using E-UTRA radio access, the UE transmits or receives the signal/channel with the higher priority.

If a UE would transmit or receive sidelink synchronization signals for E-UTRA radio access, and the transmission or reception would overlap in time with transmissions and/or receptions on the sidelink using NR radio access, the UE transmits or receives the signal/channel with the higher priority.

-----------------------------------------------------end text proposal for 38.213-----------------------------------------------------

# Text Proposal to Capture Prioritization of Multiple Overlapping Transmissions

| Reason for change: | Incorrect reference for prioritization timeline |
| --- | --- |
| Summary of change: | Correctly capture that Rx/Tx prioritization requires knowledge of the priorities before the earlier reception/transmission, not the later. |
| Consequences if not approved: | Incorrect prioritization timeline would increase UE processing burden. |
| Clauses affected: | 16.2.4.1 |

----------------------------------------------------begin text proposal for 38.213----------------------------------------------------

16.2.4.1 Simultaneous NR and E-UTRA transmission/reception

If a UE

-	would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and

-	a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and

-	the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions

the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers.

If a UE

-	would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and

-	a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and

-	the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier transmission or reception

the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers.

-----------------------------------------------------end text proposal for 38.213-----------------------------------------------------