3GPP TSG RAN WG1 #101			                                   					       												                  R1-2005037

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.7

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-NR- UE_pow_sav-UEFeatures-01]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-NR- UE_pow_sav-UEFeatures-01] during RAN1 #101-e. According to the Chairman’s Notes:

[101-e-NR- UE_pow_sav-UEFeatures-01] Email discussion/approval till 5/29 – Ralf (AT&T) - Whether or how to handle licensed/unlicensed differentiation - Discuss whether or how to handle licensed/unlicensed differentiation for FG 19-1 - Note: Whether the need for FRx differentiation needs to be updated for FG 19-1 is also discussed as part of this email discussion - Discuss whether or how to handle licensed/unlicensed differentiation for FG 19-2 - Discuss whether or how to handle licensed/unlicensed differentiation for FG 19-3 - Note: Type and FR1/FR2 differentiation for FG 19-3 is also discussed as part of this email discussion - Discuss whether or how to handle licensed/unlicensed differentiation for FG 19-4a

The following was discussed and agreed during RAN1 #101-e within the scope of [101-e-NR- UE_pow_sav-UEFeatures-01].

# Summary of email discussion/approval [101-e-NR- UE_pow_sav-UEFeatures-01]

The following table represents the latest version of the NR UE feature list for UE power savings [1].

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI / L1-SINR report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON | N/A |  | Yes |  | N/A |  | Per UE | No | Yes | N/A | The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | (1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset (2) minimumSchedulingOffset K2 configuration for PUSCH |  | N/A |  | Yes | N/A | Dynamic adaptation of the minimum value of K0min/K2min for cross-slot scheduling is not supported | Per UE | No | No | N/A | FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP |  | See Note |  | Yes | N/A |  | [Per UE ] | No | [Yes] | N/A | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |  | 19-2 |  | Yes | N/A |  | Per UE | No | No | N/A | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |

Companies are invited to provide their views on whether or how to handle licensed/unlicensed differentiation for FG 19-1 in the following table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | The UE power saving features do not impact the channel access mechanisms in any way, and they do not depend on dynamic relationship with the channel occupancy time, or other specific features to operate in unlicensed spectrum. The amount of power savings obtained in licensed and unlicensed operation may change as the UE might need to perform other tasks in unlicensed band, but those aspects are unrelated to the ability to support the FGs themselves. Hence there is no need to explicitly handle licensed/unlicensed differentiation for this feature neither for the remain UE power saving features. |
| OPPO | The DRX adaptation and DCI 2-6 is not depending on channel occupancy scheme. Putting it as spectrum dependency is not good method. The motivation for differentiating for licensed/unlicensed was also to help IoDT. But the benefit is too little to be justified. UE should support 19-1 uniformly among bands. |
| Qualcomm | The licensed/unlicensed differentiation is about IODT. It is not related to whether the implementation is the same for licensed and unlicensed or not, or whether the amount of power saving is different in licensed and unlicensed bands or not. If a feature is based on per-UE signaling, in order to introduce the feature for either licensed or unlicensed band, it requires IODT for both licensed and unlicensed bands. Therefore, the feature can be introduced only when both licensed and unlicensed bands are fully deployed. However, because it is unlikely that deployment schedules of NR in licensed and unlicensed bands are the same, the introduction of the feature could be delayed. If the feature is differentiated between licensed and unlicensed, the feature can be introduced for licensed band after IODT in the licensed band first and without IODT in the unlicensed band, and vice versa. Among the types of signaling, “Per Band” should be used to support the licensed-unlicensed differentiation. |
| CATT | WUS only indicates the UE behavior of next DRX ON. It does not have any indication of channel occupancy at any time when access in unlicensed band |
| Ericsson | For UE power savings features, we do not see a need to differentiate licensed vs unlicensed operation. Even if licensed and unlicensed bands are not deployed at the same time, implementations will be reused between the two and new testing should not be needed for every single feature for every new band. We also agree with Nokia comments that there is no impact to channel access mechanisms of the power savings feature. |
| Samsung | We do not see a need of licensed/unlicensed differentiation for UE power saving features. We agree with Ericsson’s view. |
| ZTE | There is no need for UE power saving features to differentiate between licensed and unlicensed band. We agree with many other companies that the implementations of power saving features are the same for licensed and unlicensed band, the difference may be the power saving gain, which is not relevant to UE capability. |
| Huawei, HiSilicon | The feature group does not impact on the channel occupancy mechanism. There is no need to differentiate licensed/unlicensed operation for FG 19-1. |
| Panasonic | We agree Qualcomm except the last part of "per band". We are ok with just licensed/unlicensed differentiation. |
| MediaTek | Our preference is to have licensed/unlicensed differentiation. But we can also compromise with majority view if most companies think the differentiation is not important for IoDT. |
| Intel | Our preference is to have licensed/unlicensed differentiation. Per Band is not needed. |
| NTT DOCOMO | We think there is no need of licensed/unlicensed differentiation. We agree with Nokia and Ericsson. |
| Apple | We do not see a need for licensed/unlicensed differentiation. |
| vivo | Vivo observes that the PDCCH-based power saving signal/channel has no big difference to PDCCH reception, and thus there is no strong need to differentiate licensed/unlicensed for FG 19-1. |

Companies are invited to provide their views on whether the need for FRx differentiation needs to be updated for FG 19-1 in the following table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | Not needed |
| OPPO | No |
| CATT | Yes. Current MAC would start the DRX for all serving cells including FR1/FR2. If UE indicates not supporting WUS in FR2, MAC behavior is unclear when WUS indicates UE not to wakeup |
| Ericsson | No need. |
| Samsung | No need |
| ZTE | Not needed |
| Huawei, HiSilicon | No need to update “per UE with FR1/FR2 differentiation” for FG 19-1. A clarification can be added in the note column: When FR1 is ‘yes’, FR2 is ‘no’, the UE is not expected to be configured WUS on FR1 when FR1-FR2 CA is configured. Configuration of WUS on FR1 only is still supported when UE is NOT configured as FR1 and FR2 CA. |
| Panasonic | No need but it need to clarify cross carrier between FR1 and FR2 situation when FR1 only or FR2 only is supported. Our view is the carrier to have DCI 2_6 is the corresponding carrier. |
| MediaTek | Not needed. We agree with Huawei and Panasonic that the meaning of FR1/FR2 differentiation should be clarified because there is different understanding among companies. |
| Intel | Not needed |
| NTT DOCOMO | Not needed, but the clarification is needed as suggested by Huawei. |
| Apple | Yes |
| vivo | Prefer to have FR1 and FR2 differential. Therefore no need to update the current table. Further clarification may be needed on the ‘description’ part or somewhere.For example, when FR1 is ‘yes’, FR2 is ‘no’, it should be clarified that the UE is not expected to be configured WUS on FR1 when FR1-FR2 CA is configured. However, configuration of WUS on FR1 only is still supported when UE is NOT configured as FR1 and FR2 CA. |

Companies are invited to provide their views on whether or how to handle licensed/unlicensed differentiation for FG 19-2 in the following table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | See comment for FG 19-1 above. |
| OPPO | X-slot scheduling is should not have relation with licensed/unlicensed band. This is even clearer than 19-1. Not differentiate. |
| Qualcomm | Please see Qualcomm’s comment on FG 19-1 above. |
| CATT | No needed. The K0/K2 values are no related to channel access scheme |
| Ericsson | Not needed. See our comment for 19-1 above. |
| Samsung | No need. |
| ZTE | Not needed. Please see our comments on FG 19-1 |
| Huawei, HiSilicon | No need to differentiate licensed/unlicensed operation. |
| Panasonic | From IODT perspective, it is necessary. It needs to discuss cross-slot and cross-carrier case handling. Especially cross-slot and cross carrier between licensed and unlicensed are very unclear. |
| MediaTek | Please see comment for FG 19-1 above. |
| Intel | Our preference is to have licensed/unlicensed differentiation. |
| NTT DOCOMO | Not needed. |
| Apple | No need to differentiate licensed/unlicensed operation. |
| Vivo | Not needed |

Companies are invited to provide their views on whether or how to handle licensed/unlicensed differentiation for FG 19-3 in the following table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | See comment for FG 19-1 above. |
| OPPO | Not differentiate. Configuration per BWP MIMO layer is basically configuration issues. There is no even requirements for the switching of per BWP MIMO layer. |
| Qualcomm | Please see Qualcomm’s comment on FG 19-1 above. |
| CATT | No difference in UE behavior in licensed and unlicensed bands. |
| Ericsson | Not needed. See our comment for 19-1 above. |
| Samsung | No need. |
| ZTE | Not needed. Please see our comments on FG 19-1 |
| Huawei, HiSilicon | No need to differentiate licensed/unlicensed operation. |
| Panasonic | As the report from UE is expressed by the following RRC parameter, when a UE does not support FG 19-3, UE can simply indicate the supported capability in either FR1/2. Therefore, we are okay with no separation between licensed and unlicensed band. MaxMIMO-LayerPreference-r16 ::= SEQUENCE { reducedMaxMIMO-LayersFR1-r16 SEQUENCE { reducedMIMO-LayersFR1-DL-r16 INTEGER (1..8), reducedMIMO-LayersFR1-UL-r16 INTEGER (1..4) } OPTIONAL, reducedMaxMIMO-LayersFR2-r16 SEQUENCE { reducedMIMO-LayersFR2-DL-r16 INTEGER (1..8), reducedMIMO-LayersFR2-UL-r16 INTEGER (1..4) } OPTIONAL |
| MediaTek | Please see comment for FG 19-1 above. |
| Intel | Our preference is to have licensed/unlicensed differentiation. However, we think per band type makes more sense for this FG, so licensed/unlicensed differentiation separately is not needed if per band is agreed. |
| NTT DOCOMO | Not needed. |
| Apple | We think this should be per band combination and as such licensed/unlicensed differentiation is not needed. |
| Vivo | Not needed |

Companies are invited to provide their views on how to finalize type and FR1/FR2 differentiation for FG 19-3 in the following table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | It is OK to have it per UE without FRx differentiation |
| OPPO | No, the SCS dependency already take care of it. |
| CATT | Either per BC/per band for the type without FR1/FR2 differentiation or per UE for the type and with FR1/FR2 differentiation |
| Ericsson | Per-UE with FR1/FR2 differentiation. |
| Samsung | Per-UE with FR1/FR2 differentiation. |
| ZTE | Okay with per-UE and FR1/FR2 differentiation. |
| Huawei, HiSilicon | Per-UE with FR1/FR2 differentiation. |
| Panasonic | As the indicated reporting is per FR1/2, no need to have FR1/FR2 differentiation as a capability. |
| MediaTek | Per-UE with FR1/FR2 differentiation. |
| Intel | Per band. FR1/FR2 differentiation is not needed. |
| NTT DOCOMO | We are fine with per-UE with FR1/FR2 differentiation. |
| Apple | Per BC for the type without FR1/FR2 differentiation |
| Vivo | We prefer to have per band. As compromise, we are also fine to have FR1/FR2 differential. |

Companies are invited to provide their views on whether or how to handle licensed/unlicensed differentiation for FG 19-4 in the following table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | See comment for FG 19-1 above. |
| Qualcomm | Please see Qualcomm’s comment on FG 19-1 above. |
| CATT | No difference in UE assistance information in licensed and unlicensed bands |
| Ericsson | Not needed. See our comment for 19-1 above. |
| Samsung | No need. |
| ZTE | Not needed. Please see our comments on FG 19-1 |
| Huawei, HiSilicon | No need to differentiate licensed/unlicensed operation. |
| Panasonic | The report from the UE is expressed by the following RRC parameter. When a UE does not support the feature, UE can simply indicate the supported k0/k2 are possible. Therefore, we are okay with no separation between licensed and unlicensed band. MinSchedulingOffsetPreference-r16 ::= SEQUENCE { preferredK0-r16 SEQUENCE { preferredK0-SCS-15kHz-r16 ENUMERATED {sl1, sl2, sl4, sl6} OPTIONAL, preferredK0-SCS-30kHz-r16 ENUMERATED {sl1, sl2, sl4, sl6} OPTIONAL, preferredK0-SCS-60kHz-r16 ENUMERATED {sl2, sl4, sl8, sl12} OPTIONAL, preferredK0-SCS-120kHz-r16 ENUMERATED {sl2, sl4, sl8, sl12} OPTIONAL } OPTIONAL, preferredK2-r16 SEQUENCE { preferredK2-SCS-15kHz-r16 ENUMERATED {sl1, sl2, sl4, sl6} OPTIONAL, preferredK2-SCS-30kHz-r16 ENUMERATED {sl1, sl2, sl4, sl6} OPTIONAL, preferredK2-SCS-60kHz-r16 ENUMERATED {sl2, sl4, sl8, sl12} OPTIONAL, preferredK2-SCS-120kHz-r16 ENUMERATED {sl2, sl4, sl8, sl12} OPTIONAL } OPTIONAL } |
| MediaTek | Not needed. |
| Intel | We concur with Panasonic. |
| NTT DOCOMO | Not needed. |
| Apple | No need for licensed/unlicensed differentiation. |
| Vivo | Not needed. |

After further discussion by email in the first week of RAN1 #101e, companies were invited to provide further views on whether or how to handle licensed/unlicensed differentiation at the beginning of week 2. Their views, preferences, objections, and comments are captured in the following tables.

Proposal: Update FG 19-1 as follows:

Alt. 1:

| 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI / L1-SINR report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON | N/A | Yes | N/A |  | Per UE | No | Yes | N/A | The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 2:

| 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI / L1-SINR report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON | N/A | Yes | N/A |  | Per UE | No | Yes | N/A | The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap Note: this FG only applies to bands where the UE is operating without shared spectrum channel access FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19-1a | DRX Adaptation with shared spectrum channel access | Support of DRX Adaptation with shared spectrum channel access | 19-1 | Yes | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |

In the table below, companies are invited to provide their preference, possible objection, and other comments.

| Company Name | Indication which alternative is preferred to be agreed Please respond with - Alt. 1 - Alt. 2 - None | Indication whether an alternative is not agreeable at all (i.e., company will object) Please respond with - Alt. 1 - Alt. 2 - None | Comments/Questions/Suggestions |
| --- | --- | --- | --- |
| Panasonic | Alt. 2 | Alt. 1 | - We are also ok with that 19-1 containing candidate value set {non-shared spectrum channel access, shared spectrum access} - We are also ok to describe "if 19-1 is supported and corresponding IODT is available, it shall support 19-1a". |
| Nokia, NSB | Alt. 1 | None |  |
| MediaTek | Alt. 2 | Alt. 1 |  |
| Huawei, HiSilicon | Alt. 1 | Alt. 2 | We are also fine to compromise to only have the FG19-1 in Alt.1, with a conclusion that the feature is not supported for NR-U. |
| Qualcomm | Alt. 2 | Alt. 1 | To be aligned with the agreement in the CLI/RIM features, we are also fine with Alt. 1 with a note that the feature is not supported in unlicensed bands in Rel-16. |
| Ericsson | Alt 1 | None |  |
| CATT | Alt 1 | Alt 2 |  |
| Apple | Alt 1 | None | As mentioned by Ericsson/MTK in the discussion phase, we would like to capture the following in the Notes to explain what FR1-FR2 differentiation is i.e. Interpretation #2: FR1/FR2 bit allows a UE to indicate UE support of DCI 2-6 monitoring on primary cell in FR1 or on primary cell in FR2, respectively |
| ZTE | Alt 1 | Alt 2 |  |
| Intel | Alt 2 | None |  |

Proposal: Update FG 19-2 as follows:

Alt. 1:

| 19-2 | Cross Slot Scheduling | (1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset (2) minimumSchedulingOffset K2 configuration for PUSCH | N/A | Yes | N/A | Dynamic adaptation of the minimum value of K0min/K2min for cross-slot scheduling is not supported | Per UE | No | No | N/A | FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 2:

| 19-2 | Cross Slot Scheduling | (1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset (2) minimumSchedulingOffset K2 configuration for PUSCH | N/A | Yes | N/A | Dynamic adaptation of the minimum value of K0min/K2min for cross-slot scheduling is not supported | Per UE | No | No | N/A | FFS: whether or how to handle licensed/unlicensed differentiation Note: this FG only applies to bands where the UE is operating without shared spectrum channel access | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19-2a | Cross Slot Scheduling with shared spectrum channel access | Support of Cross Slot Scheduling with shared spectrum channel access | 19-2 | Yes | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |

In the table below, companies are invited to provide their preference, possible objection, and other comments.

| Company Name | Indication which alternative is preferred to be agreed Please respond with - Alt. 1 - Alt. 2 - None | Indication whether an alternative is not agreeable at all (i.e., company will object) Please respond with - Alt. 1 - Alt. 2 - None | Comments/Questions/Suggestions |
| --- | --- | --- | --- |
| Panasonic | Alt. 2 | Alt. 1 | - We are also ok with that 19-2 containing candidate value set {non-shared spectrum channel access, shared spectrum access} - We are also ok to describe "if 19-2 is supported and corresponding IODT is available, it shall support 19-2a". |
| Nokia, NSB | Alt. 1 | None |  |
| MediaTek | Alt. 2 | Alt. 1 |  |
| Huawei, HiSilicon | Alt. 1 | Alt. 2 | We are also fine to compromise to only have the FG19-2 in Alt.1, with a conclusion that the feature is not supported for NR-U. |
| Qualcomm | Alt. 2 | Alt. 1 | To be aligned with the agreement in the CLI/RIM features, we are also fine with Alt. 1 with a note that the feature is not supported in unlicensed bands in Rel-16. |
| Ericsson | Alt 1 | None |  |
| CATT | Alt 1 | Alt 2 |  |
| Apple | Alt 1 | None |  |
| ZTE | Alt 1 | Alt 2 |  |
| Intel | Alt 2 | None |  |

Proposal: Update FG 19-3 as follows:

Alt. 1:

| 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP | See Note | Yes | N/A |  | [Per UE] | No | [Yes] | N/A | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 2:

| 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP | See Note | Yes | N/A |  | [Per UE] Per Band | No | [Yes] N/A | N/A | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 3:

| 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP | See Note | Yes | N/A |  | [Per UE] | No | [Yes] | N/A | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication FFS: whether or how to handle licensed/unlicensed differentiation Note: this FG only applies to bands where the UE is operating without shared spectrum channel access | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19-3a | Maximum MIMO Layer Adaptation with shared spectrum channel access | Support of Maximum MIMO Layer Adaptation with shared spectrum channel access | 19-3 | Yes | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |

In the table below, companies are invited to provide their preference, possible objection, and other comments.

| Company Name | Indication which alternative is preferred to be agreed Please respond with - Alt. 1 - Alt. 2 - Alt .3 - None | Indication whether an alternative is not agreeable at all (i.e., company will object) Please respond with - Alt. 1 - Alt. 2 - None | Comments/Questions/Suggestions |
| --- | --- | --- | --- |
| Panasonic | Alt1, 2, 3 are ok | None |  |
| Nokia, NSB | Alt. 1 | Alt. 2 |  |
| MediaTek | Alt. 1, 3 are ok | None |  |
| Huawei, HiSilicon | Alt. 1 | Alt.2 and Alt.3 | We are also fine to compromise to only have the FG19-3 in Alt.1, with a conclusion that the feature is not supported for NR-U. |
| Qualcomm | Alt. 2, 3 |  | Alt. 1 is also acceptable as a compromise, preferably with a note that the feature is not supported in unlicensed bands in Rel-16. |
| Ericsson | Alt 1 | Alt 2, Alt 3 | FG 19-3 is just an indication that UE supports a NW configuration of an RRC parameter with different max MIMO layers for different BWPs – there is no need for separate indication for shared spectrum as there seems to be no technical reason for differentiation. |
| CATT | Alt 2, Alt 1 is OK | Alt 3 |  |
| Apple | Alt 2 |  | We can compromise to Alt 1 |
| ZTE | Alt 1 | Alt 2, Alt 3 | FG19-3 is a RRC configured parameter. There is no need to differentiate between licensed or unlicensed band. |
| Intel | Alt 2 | None |  |

Proposal: Update FG 19-4a as follows:

Alt. 1:

| 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots | 19-2 | Yes | N/A |  | Per UE | No | No | N/A | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 2:

| 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots | 19-2 | Yes | N/A |  | Per UE | No | No | N/A | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted FFS: whether or how to handle licensed/unlicensed differentiation Note: this FG only applies to bands where the UE is operating without shared spectrum channel access | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19-4b | UE assistance information with shared spectrum channel access | Support of UE assistance information with shared spectrum channel access | 19-4 | Yes | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |

In the table below, companies are invited to provide their preference, possible objection, and other comments.

| Company Name | Indication which alternative is preferred to be agreed Please respond with - Alt. 1 - Alt. 2 - None | Indication whether an alternative is not agreeable at all (i.e., company will object) Please respond with - Alt. 1 - Alt. 2 - None | Comments/Questions/Suggestions |
| --- | --- | --- | --- |
| Panasonic | Alt 1, 2 are ok | None |  |
| Nokia, NSB | Alt. 1 | None |  |
| MediaTek | Alt. 1, 2 are ok | None |  |
| Huawei, HiSilicon | Alt. 1 | Alt.2 | We are also fine to compromise to only have the FG19-4a in Alt.1, with a conclusion that the feature is not supported for NR-U. |
| Qualcomm | Alt. 1, 2 |  | Although we slightly prefer Alt 2, we also agree with other companies’ view that this FG is not as problematic as other features for IODT. Therefore, Alt 1 is also acceptable. |
| Ericsson | Alt 1 | Alt 2 | FG 19-4 is just an indication that UE can signal UE assistance info about min K0/K2 based on RRC ignalling – there is no need for separate indication for shared spectrum as there seems to be no technical reason for differentiation. |
| CATT | Alt 1 | Alt 2 | There is no need to differentiate UE assistance information for operation in licensed and unlicensed spectrum |
| Apple | Alt 1 | None |  |
| ZTE | Alt 1 | Alt 2 | FG19-4a is a UE assistance information. There is no need to differentiate between licensed or unlicensed band. |
| Intel | Alt 1 | None |  |

# Conclusion

After further discussion by email on the RAN1 email reflector, the email discussion/approval was closed with the following agreements:

Agreement:

| 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP | See Note | Yes | N/A |  | [Per UE] | No | [Yes] | N/A | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots | 19-2 | Yes | N/A |  | Per UE | No | No | N/A | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |

Agreement:

| 19-1 | DRX Adaptation | (1) Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration (2) Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 (3) Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time (4) Configured P-CSI / L1-SINR report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON (5) Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON | N/A | Yes | N/A |  | Per UE | No | Yes | N/A | The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing.  The reporting is per SCS in units of slots of the respective SCS  The reported value for a SCS is taken from two possible values per SCS  The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap Note: RAN1 agreed it should be possible to separately indicate support of this FG based on whether the UE is operated with or without shared spectrum access. It is left to RAN2 how to implement this while leaving the type as “per UE” FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19-2 | Cross Slot Scheduling | (1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset (2) minimumSchedulingOffset K2 configuration for PUSCH | N/A | Yes | N/A | Dynamic adaptation of the minimum value of K0min/K2min for cross-slot scheduling is not supported | Per UE | No | No | N/A | Note: RAN1 agreed it should be possible to separately indicate support of this FG based on whether the UE is operated with or without shared spectrum access. It is left to RAN2 how to implement this while leaving the type as “per UE” FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |

# References

- R1-2003195, Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — UE Power Saving Aspects, Moderator (AT&T)