3GPP TSG RAN WG1 Meeting #101			R1-200xxxx

e-meeting, May 25th – June 5th, 2020

Agenda Item:	6.2.1.7

Source: 	Moderator (LG Electronics)

Title: 	FL summary#1 of CE mode A and B improvements for non-BL UEs

Document for:	Discussion and decision

# Introduction

One contribution was submitted in this agenda item for clarification on the DCI format 6-1B for direction indication. This document is to gather inputs from companies on whether the proposed correction is needed and can be recommended for an e-mail discussion during RAN1#101-e meeting.

# Discussion

## Issue #1 	Clarification of DCI Format 6-1B for direct indication

Based on the following agreement in RAN1#99 meeting, both DCI format 6-1A and DCI format 6-1B can be used for direct indication.

Agreement The content of the DCI format 6-1A/1B for ETWS/CMAS indication to a non-BL UE in CE in connected mode using Type 0 CSS is as follows: - Direct indication information (8 bits) with the content left for RAN2 to define in TS 36.331. - All the remaining bits in format 6-1A/1B for ETWS/CMAS indication are set to zeros.

ZTE’s contribution [1] brought up an issue that the DCI format 6-1B can be used for ETWS/CMAS indication in connected mode when the DCI CRC is scrambled by SI-RNTI and ce-etws-cmas-config is configured by higher layers, but in the latest TS 36.212 [2], the description on this function of DCI format 6-1B is missing. Regarding this issue, ZTE also prepared a proposal below adding a text to clarify that the DCI format 6-1B is used for direct indication.

Proposal 1: The Text Proposal #1 is endorsed. TP to be included in 36.212 editor’s CR.

# Text Proposal #1 for TS 36.212 [1] <Unchanged parts are omitted> ##### 5.3.3.1.13 Format 6-1B DCI format 6-1B is used for the scheduling of one PDSCH codeword in one cell, notifying SC-MCCH change, operation on preconfigured UL resources, and direct indication. The following information is transmitted by means of the DCI format 6-1B: <Unchanged parts are omitted>

# Inputs from companies

Please provide inputs on the issue#1 and the proposal. Inputs on whether the proposed correction is needed and can be recommended for an e-mail discussion will be helpful.

| Companies | Comments |
| --- | --- |
| Ericsson | Ok. To us this as an Editorial issue on the opening paragraph of DCI Format 6-1B, because this DCI Format indeed contains direct indication statements for ETWS/CMAS. Thus, in our view we can kindly ask the Editor of TS 36.212 to perform the amendment. |
| ZTE | We think the correction is needed. If we kindly ask the editor to perform the amendment, it seems we still need a conclusion/agreement. |
| Ericsson v004 | It is just that “, and direct indication” was editorially missed to be appended for first paragraph of DCI Format 6-1B, and not that the direct indication functionality was missed in DCI Format 6-1B. Basically, what we want to say is that it seems that having an e-mail discussion for this so small issue doesn’t seem to be necessary. Perhaps a conclusion where we kindly request an action to the Editor of TS 36.212 would be a more efficient way to solve the issue. |

# References

R1-2003794		Clarification of DCI Format 6-1B for direct indication,	ZTE

3GPP TS36.212 V16.1.0