3GPP TSG-RAN WG1 Meeting #101-e	                 R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.3.3

Source:	Moderator (AT&T)

Title:	Summary of [101-e-NR-IAB-04]: Email discussion on potential 38.213 and 38.331 editorial issues for IAB

Document for:	Discussion/Approval

# Introduction

This contribution provides a summary of [101-e-NR-IAB-04]: Email discussion on potential 38.213 and 38.331 editorial issues for IAB.

# Alignment of 38.331 and 38.213 parameters related to DCI Format 2_5

Source: R1-2003732, R1-2004133, R1-2004582

Background: During RAN1#100bis-e the following agreements were reached:

Agreements: Confirm that from a RAN1 perspective all Rel-15 UE common search space types are also applicable to Rel-16 IAB nodes. Signaling details are left to RAN2.

Agreements: Confirm DCI Format 2_0 and DCI Format 2_5 can be monitored by an IAB-MT in at least a common search space. The same number of aggregation levels and candidates can be separately configured for both DCI Format 2_0 and DCI Format 2_5.

Agreements: DCI Format 2_0 is not monitored by an IAB-MT in a UE(MT)-specific search space. DCI Format 2_5 can be additionally monitored by an IAB-MT in a UE(MT)-specific search space. Signaling details (e.g. whether the configuration is in the existing UE-specific search space configuration or a new MT-specific search space configuration is left up to RAN2).

Based on these agreements there is a need to align RAN1 specifications with the RRC configurations for an IAB-MT related to DCI Format 2_0 and 2_5. Specifically for RAN1#101-e two issues were identified as potential editorial corrections to be handled by the 38.213 and 38.331 editors.

## Issue 1: Add reference SCSs for soft resource availability indication configuration in the RRC IE AvailabilityCombinationPerCell (R1-2003732)

Current, the reference SCS for a DCI Format 2_5 availability indication can be derived from the IAB-DU resource configuration. However, when DCI Format 2_5 is sent in the common search space instead of the UE(MT)-specific search space, if multiple IAB nodes are the intended recipients of the availability indication and some of the IAB-DUs have different reference SCS configurations, the time duration applicability of the availability indication will vary across the IAB nodes, which may not be the intended behavior.

One solution proposed in R1-2003732 is to align the DCI Format 2_5 configuration with DCI Format 2_0 and add reference SCSs for soft resource availability indication configuration in the RRC IE AvailabilityCombinationPerCell.

- For unpaired spectrum operation, a reference SCS configuration µ_AI is provided by subcarrierSpacing-AI and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration µ_(AI,SUL) is provided by subcarrierSpacing2-AI for the supplementary UL carrier.

- For paired spectrum operation, a reference SCS configuration µ_(AI,DL) for a DL BWP is provided by subcarrierSpacing-AI and a reference SCS configuration µ_(AI,UL) is provided for an UL BWP by subcarrierSpacing2-AI.

Discussion:

| Company | Do you agree that the monitoring DCI Format 2_5 in a common search space results in the issue described above? If so, is the solution proposed in R1-2003732 acceptable? | Comments |
| --- | --- | --- |

## Issue 2: positionInDCI-AI/dci-PayloadSize-AI used for USS (R1-2004133)

Based on RAN1#100bis-e agreements, DCI Format 2_5 can be monitored in a CSS and additionally in a USS by an IAB-MT. It is not clear whether DCI Format 2_5 can be monitored simultaneously by an IAB-MT in a CSS and USS. In case of simultaneous monitoring, R1-2004133 proposes that positionInDCI-AI and dci-PayloadSize-AI which may be based on the number of availability combinations configured should be separately configured for the CSS and USS:

In addition to dci-PayloadSize-AI and positionInDCI-AI used for CSS, introduce new RRC parameters dci-PayloadSize-AI-MSS and positionInDCI-AI-MSS used for the UE(MT)-specific search space.

Figure 1. Location of availability indicator field for an IAB-node DU-cell in DCI format 2_5 (R1-2004133)

Discussion:

| Company | Do you agree that monitoring DCI Format 2_5 in a CSS and USS is(should be) supported? If so, is the solution proposed in R1-2004133 necessary? | Comments |
| --- | --- | --- |

# Summary

TBD