3GPP TSG-RAN WG1 Meeting #101-e	                 R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.3.3

Source:	Moderator (AT&T)

Title:	Summary of [101-e-NR-IAB-04]: Email discussion on potential 38.213 and 38.331 editorial issues for IAB

Document for:	Discussion/Approval

Introduction

This contribution provides a summary of [101-e-NR-IAB-04]: Email discussion on potential 38.213 and 38.331 editorial issues for IAB.

Alignment of 38.331 and 38.213 parameters related to DCI Format 2_5

Source: R1-2003732, R1-2004133, R1-2004582

Background: During RAN1#100bis-e the following agreements were reached:

Agreements: Confirm that from a RAN1 perspective all Rel-15 UE common search space types are also applicable to Rel-16 IAB nodes. Signaling details are left to RAN2.

Agreements: Confirm DCI Format 2_0 and DCI Format 2_5 can be monitored by an IAB-MT in at least a common search space. The same number of aggregation levels and candidates can be separately configured for both DCI Format 2_0 and DCI Format 2_5.

Agreements: DCI Format 2_0 is not monitored by an IAB-MT in a UE(MT)-specific search space. DCI Format 2_5 can be additionally monitored by an IAB-MT in a UE(MT)-specific search space. Signaling details (e.g. whether the configuration is in the existing UE-specific search space configuration or a new MT-specific search space configuration is left up to RAN2).

Based on these agreements there is a need to align RAN1 specifications with the RRC configurations for an IAB-MT related to DCI Format 2_0 and 2_5. Specifically for RAN1#101-e two issues were identified as potential editorial corrections to be handled by the 38.213 and 38.331 editors.

Issue 1: Add reference SCSs for soft resource availability indication configuration in the RRC IE AvailabilityCombinationPerCell (R1-2003732)

Current, the reference SCS for a DCI Format 2_5 availability indication can be derived from the IAB-DU resource configuration. However, when DCI Format 2_5 is sent in the common search space instead of the UE(MT)-specific search space, if multiple IAB nodes are the intended recipients of the availability indication and some of the IAB-DUs have different reference SCS configurations, the time duration applicability of the availability indication will vary across the IAB nodes, which may not be the intended behavior.

One solution proposed in R1-2003732 is to align the DCI Format 2_5 configuration with DCI Format 2_0 and add reference SCSs for soft resource availability indication configuration in the RRC IE AvailabilityCombinationPerCell.

- For unpaired spectrum operation, a reference SCS configuration µ_AI is provided by subcarrierSpacing-AI and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration µ_(AI,SUL) is provided by subcarrierSpacing2-AI for the supplementary UL carrier.

- For paired spectrum operation, a reference SCS configuration µ_(AI,DL) for a DL BWP is provided by subcarrierSpacing-AI and a reference SCS configuration µ_(AI,UL) is provided for an UL BWP by subcarrierSpacing2-AI.

Discussion:

| Company | Do you agree that the monitoring DCI Format 2_5 in a common search space results in the issue described above? If so, is the solution proposed in R1-2003732 acceptable? | Comments |
| --- | --- | --- |
| ZTE, Sanechips | No. The concerned issue can be avoided. | As mentioned in our response in preparation phase, the table entry (set of slots holding availability indications) pointed by the index contained in DCI 2_5 can be configured and interpreted on a per IAB-node basis. The combination of individual ref-SCS and individual table entry interpretation could reach the same time duration applicability of availability indication. This is even the mechanism supported by DCI 2_0 as well. The solution in the current spec does not need to change anything in 38.213 and 38.331. But the proposed solution needs to modify both specs. |
| Intel | Yes. | Let’s first look at DCI 2_0 configuration, which has reference SCS configuration. The propoed reference SCS configuration for DCI 2_5 is inherited from DCI 2_0 configuration. Copied below is DCI 2_0 reference SCS configuration in TS38.331 SlotFormatCombinationsPerCell ::= SEQUENCE { servingCellId ServCellIndex, subcarrierSpacing SubcarrierSpacing, subcarrierSpacing2 SubcarrierSpacing OPTIONAL, slotFormatCombinations SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination OPTIONAL, positionInDCI INTEGER(0..maxSFI-DCI-PayloadSize-1) OPTIONAL, ..., [[ enableConfiguredUL-r16 ENUMERATED {enabled} OPTIONAL ]] } Copied below is DCI 2_0 reference SCS description in TS38.213 11.1.1 (unpaired spectrum reference SCS is listed below; paired spectrum reference SCS operation can also be found in the spec) - for unpaired spectrum operation, a reference SCS configuration by subcarrierSpacing and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration by subcarrierSpacing2 for the supplementary UL carrier - For unpaired spectrum operation for a UE on a serving cell, the UE is provided by subcarrierSpacing a reference SCS configuration for each slot format in a combination of slot formats indicated by a SFI-index field value in DCI format 2_0. The UE expects that for a reference SCS configuration and for an active DL BWP or an active UL BWP with SCS configuration , it is . Each slot format in the combination of slot formats indicated by the SFI-index field value in DCI format 2_0 is applicable to consecutive slots in the active DL BWP or the active UL BWP where the first slot starts at a same time as a first slot for the reference SCS configuration and each downlink or flexible or uplink symbol for the reference SCS configuration corresponds to consecutive downlink or flexible or uplink symbols for the SCS configuration . The propoed reference SCS configuration for DCI 2_5 is similar to DCI 2_0 reference SCS configuration. We don’t know without reference SCS configuration for DCI 2_5, how individual SCS interpretation can achieve same time duration applicability of availability indication. Among multiple IAB nodes receiving the same DCI 2_5, one IAB node cannot know other IAB nodes’ SCS configuration. |
| Nokia | Yes | Tend to agree with Intel. DCI 2-5 was introduced mainly following the same framework as DCI 2-0, and AvailabilityCombinationsPerCell-r16 contained similar parameter set as SlotFormatCombinationsPerCell. We think that introducing subcarrierSpacing and subcarrierSpacing2 also within AvailabilityCombinationsPerCell-r16 is cleaner solution to configure the reference sub-carrier spacing applied with the DCI 2-5 indication. ZTE suggestion may work when the indication of DCI 2-5 mainly targeting single IAB node. But as FL summarized, it may not solve the issue when the DCI 2-5 is sent for multiple IAB nodes. |
| Huawei | No | Agree with ZTE that each IAB node can follow each the SCS of DU to interpret DCI format 2_5. As another alternative, DCI format 2_5 is a group-common PDCCH, one can always group the IAB nodes with same SCS together at cost of the control signaling overhead at the gNB. |
| Qualcomm | Yes to the first question. No to the second question in the context of Rel-16 | We acknowledge there is a potential issue. However it is questionable, particularly when sending an AI with sub-slot resolution, whether grouping IAB-nodes with different IAD-DU SCS has any value. So the issue can be avoided by grouping IAB-nodes with the same IAB-DU SCS, as Huawei suggested. We think this is acceptable in Rel-16. At this point we do not want to introduce any signaling change except to address a critical issue. This particular one does not seem to raise to that level. |
| LG | No | Agree with ZTE that DCI format 2_5 can be interpreted per-IAB-node basis as described in TS 38.213: “The IAB-node DU can assume a same SCS configuration for availabilityCombinations for a serving cell as an SCS configuration provided by IAB-DU-Resource-Configuration-TDD-Config for the serving cell.” So, we don’t see critical problem on this issue. |

Issue 2: positionInDCI-AI/dci-PayloadSize-AI used for USS (R1-2004133)

Based on RAN1#100bis-e agreements, DCI Format 2_5 can be monitored in a CSS and additionally in a USS by an IAB-MT. It is not clear whether DCI Format 2_5 can be monitored simultaneously by an IAB-MT in a CSS and USS. In case of simultaneous monitoring, R1-2004133 proposes that positionInDCI-AI and dci-PayloadSize-AI which may be based on the number of availability combinations configured should be separately configured for the CSS and USS:

In addition to dci-PayloadSize-AI and positionInDCI-AI used for CSS, introduce new RRC parameters dci-PayloadSize-AI-MSS and positionInDCI-AI-MSS used for the UE(MT)-specific search space.

Figure 1. Location of availability indicator field for an IAB-node DU-cell in DCI format 2_5 (R1-2004133)

Discussion:

| Company | Do you agree that monitoring DCI Format 2_5 in a CSS and USS is(should be) supported? If so, is the solution proposed in R1-2004133 necessary? | Comments |
| --- | --- | --- |
| ZTE, Sanechips | No | We do not see strong motivation to support simultaneous monitoring of DCI 2_5 in both CSS and USS. It seems RAN1 needs to Inform RAN2 of such, if agreed, by either an LS or a note in higher layer signaling spreadsheet. |
| Intel | No | As our comments in preparation stage: - RRC parameter iabDuCellId-AI is configured in case of multiple cells in one IAB DU. Accordingly, RRC parameter positionInDCI-AI is pointing at DCI format 2_5 payload for different cells in one IAB DU. Those two parameters already exist for DCI format 2_5 sending to one IAB MT. They are not parameters to differentiate different DUs in case of one DCI format 2_5 sending to multiple MTs. - When CSS is used for DCI format 2_5, the configuration is supposed to be common for multiple MTs; when USS is used for DCI format 2_5, the configuration is supposed to be MT-specific. So there will be no case that part of DCI format 2_5 payload is used for one MT, and part of DCI format 2_5 payload is used for another MT. Hence, there is no intention to define different dci-PayloadSize-AI and positionInDCI-AI for UE(MT)-specific search space and CSS. |
| Nokia | No | Agree with ZTE and Intel. |
| Huawei | No | Agree with ZTE. |
| Qualcomm | No | Agree with ZTE. |
| LG | More clarification is needed. | As we expressed in the preparation phase, according to current specification, positionInDCI-AI can be configured IAB-node DU cell specifically even in case of CSS is used for DCI format 2_5. In other word, depending on the network configuration for positionInDCI-AI, the AI-field in DCI format 2_5 monitored in CSS can be applied to a certain IAB-node DU cell or IAB-node DU cell group. Obviously, DCI format 2_5 monitored in USS can be consists of AI fields corresponding to IAB-node DU cells associated with IAB-node MT monitoring that DCI format 2_5. So, depending on the monitoring search space (i.e. CSS or USS), AI-fields construction in DCI format 2_5 can be different. In that sense, if simultaneous monitoring of DCI 2_5 in both CSS and USS is allowed, positionInDCI-AI and dci-PayloadSize-AI should be distinguishable according to the monitoring search space. If the majority companies (given this situation) think simultaneous monitoring of DCI 2_5 in both CSS and USS is not needed, we can at least make a conclusion as “simultaneous monitoring of DCI 2_5 in both CSS and USS is not supported”. |

Summary

TBD