3GPP TSG RAN WG1 Meeting #101-e	  R1-20xxxxx

E-meeting, 25 May – 5 June, 2020

Agenda Item:	7.2.5.1

Source:	Moderator (Huawei)

Title:	Email discussion/approval [101-e-NR-L1enh-URLLC-PDCCH enhancements-02] on remaining issues on scaling PDCCH monitoring capability

Document for:	Discussion and Decision

Introduction

The email discussion is to discuss the remaining issues on DCI format design.

[101-e-NR-L1enh-URLLC-PDCCH enhancements-02] Remaining issues on scaling PDCCH monitoring capability by 5/29 and corresponding TP (if any) by 6/5 – Chengyan (Huawei) including

- Issue C-1: Corrections on span duration

- Issue C-2: Corrections on “aligned spans” case

- Issue C-3: Corrections on “unaligned spans” case

- Issue C-4: Enhanced PDCCH monitoring capability for cross-carrier scheduling

This document summarizes the above issue and provide some initial proposals for discussion. Companies are encouraged to provide the first round views by 5/26, then we can adjust the proposals and prepare the TPs for the next step discussions.

Enhanced PDCCH monitoring capability

This section summarize the issues on enhanced PDCCH monitoring capability to be discussed under this email discussion.

Remaining issues on scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability

- Issue C-1: Corrections on span duration

The following text has been captured in section 10 of TS38.213 to define a combination (X, Y).

A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. A last span in a slot can have a shorter duration than other spans in the slot.

During the email discussion in RAN1#100b-e, it was observed that some further clarification are needed for the span duration in TS 38.213.

Question C-1-1: With the formula dspan=maxdCORESET,max,Ymin defined in TS 38.213, whether to allow the span duration larger than Y for a combination (X, Y).

Based on the discussion in RAN1#100b-e, it is common understanding that the span duration is not allowed to be larger than Y for a combination (X, Y), and it was observed that some further clarification is needed to reflect this in the spec. Samsung (R1-2003865), ZTE (R1-2003317), Huawei (R1-2003525) and Quectel (R1-2003942) provide some text proposal accordingly.

Text proposal 1 (R1-2003865):

If a UE monitors PDCCH on a cell according to combination X,Y, the duration of a span is Y dspan=maxdCORESET,max,YmindCORESET,maxYminYX,Y. A last span in a slot can have a shorter duration than other spans in the slot.

Feature lead view: As shown in R1-2003865, the assumption of the above TP is that UE will determine the combination (X, Y) for PDCCH monitoring first. Firstly, if we go this way, we may need some further clarification in the spec to define the order of determination of span duration and determination of combination (X, Y). Note that in the current TS 38.213, determination of combination (X, Y) is in the later section 10.1 than the definition of span duration in section 10. Secondly, according to the agreements we made, the logic is to determine the valid combinations (X, Y), and then pick one from all the valid combinations (X, Y).  Thirdly, in order to determine the separation of two consecutive spans, UE needs to know the span duration first.

Text proposal 2 (R1-2003317):

10 UE procedure for receiving control information <---------------------------Other parts are omitted -------------------------------> A UE reports one or more combinations of number of symbols, where , for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span is and is of length up to consecutive OFDM symbols, where is a maximum duration among durations of CORESETs that are configured to the UE and is a minimum value of in the combinations of that are reported by the UE. A last span in a slot can have a shorter duration than other spans in the slot. <---------------------------Other parts are omitted ------------------------------->

Feature lead view: When UE reports more than one combination (X, Y), it is not clear which Y to be used here.

Text proposal 3 (R1-2003525):

******************************** Start of TP **************************** 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot on a serving cell in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span other than the last span in a slot is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. The duration of a last span in a slot is mindspan,dboudary, where dboudary is the time separation between the start of the span and the end of the slot in number of symbols. When a UE reports in pdcch-MonitoringAnyOccasionsWithSpanGap combinations (X, Y) corresponding to value set 3 and is configured a CORESET with duration of 3 OFDM symbols, the UE is not expected to monitor PDCCH according to combination (2, 2). < Unchanged parts are omitted > ************************************* End of TP *********************************

Feature lead view: Value set 3 is not defined in the spec. In addition, it cannot preclude the case of the configuration as shown in CC3 in the figure below.

Figure C-1-1 Whether three cells are aligned case for (2,2) or not

Text proposal 4 (R1-2003525):

*** Unchanged text is omitted *** A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. A last span in a slot can have a shorter duration than other spans in the slot. When dspan is equal to 3, a UE is not expected to be configured with PDCCH monitoring occasions resulting into a separation of the first symbol of two consecutive spans that is smaller than 4. *** Unchanged text is omitted ***

Feature lead view: The only case that may have misunderstanding is when UE reports the support of combination (2, 2) together with combination (4, 3) and/or combination (7, 3). In this case, if gNB decides to configure CORESET(s) with 3-symbol duration, then gNB needs to ensure that the separation X should not be smaller than 4. It can preclude the configuration similar as what for CC3 in the figure C-1-1. It seems text proposal 4 is more complete.

Proposal 1: Adopt the following text proposal for section 10 in TS 38.213:

*** Unchanged text is omitted *** A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. A last span in a slot can have a shorter duration than other spans in the slot. When dspan is equal to 3, a UE is not expected to be configured with PDCCH monitoring occasions resulting into a separation of the first symbol of two consecutive spans that is smaller than 4. *** Unchanged text is omitted ***

Please provide your views and your reasons on the above proposal 1.

| Company | View |
| --- | --- |

Question C-1-2: Whether to keep “A last span in a slot can have a shorter duration than other spans in the slot. ”, and if yes whether any further clarification needed.

Samsung R1-2003865 Last span in a slot can have shorter duration Another issue is the following statement that was inherited from FG 3-5b in Rel-15. A last span in a slot can have a shorter duration than other spans in the slot.. The above may be beneficial for example as it allows for X,Y=(4, 3) PDCCH monitoring to start at the beginning of a slot and have Y=1 at the 13th symbol of the slot. However, in that case, the following statement is not applicable for the “including across slots”. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots.. Proposal 4: RAN1 to clarify when and how both following statements are applicable. If no such case, remove the second statement from the specifications as otherwise the UE behavior is ambiguous. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots.. and A last span in a slot can have a shorter duration than other spans in the slot..

Quectel R1-2003942 Calculations of the duration of a last span in a slot should be specifically captured in the specification as well. Accordingly, the proposed text changes are as follows: ******************************** Start of TP **************************** 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot on a serving cell in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span other than the last span in a slot is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. The duration of a last span in a slot is mindspan,dboudary, where dboudary is the time separation between the start of the span and the end of the slot in number of symbols. When a UE reports in pdcch-MonitoringAnyOccasionsWithSpanGap combinations (X, Y) corresponding to value set 3 and is configured a CORESET with duration of 3 OFDM symbols, the UE is not expected to monitor PDCCH according to combination (2, 2). < Unchanged parts are omitted > ************************************* End of TP *********************************

Feature lead view: The text “A last span in a slot can have a shorter duration than other spans in the slot. ” can be kept, even consider the separation for cross slots, it can be valid when the first span in the slot doesn’t start at the beginning of the slot. The text proposal from R1-2003942 can make the specification clearer.

Proposal 2: Adopt the following text proposal for section 10 in TS 38.213:

******************************** Start of TP **************************** 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. The duration of a last span in a slot is mindspan,dboudary, where dboudary is the time separation between the start of the span and the end of the slot in number of symbols. < Unchanged parts are omitted > ************************************* End of TP *********************************

Please provide your views and your reasons on the above proposal 2.

| Company | View |
| --- | --- |

- Issue C-2: Corrections on “aligned spans” case

The following text has been captured in section 10.1 of TS38.213 for scaling PDCCH monitoring capability.

If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y, - TBD, otherwise where Ncells,r16DL,j is a number of configured cells using Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with downlink cells using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

Regarding the text for “aligned spans” case, the following updates were proposed by companies:

Update #1 for discussion:

Ericsson R1-2003439 However, we found that the term “per span” used in the above TP is not clear. A span is defined for a cell only, not for multiple cells. For the CA case, there are multiple cells, and the total limits MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ should be applied to PDCCH monitoring occasions in multiple spans across the DL cells. Moreover, if a UE reports different sets of (X,Y) value(s) for each cell from the Ncells,r16DL,X,Y,μ Ncells,r16DL,X,Y,μ downlink cells, the span duration in different cells may not even be the same. This is illustrated in Figure 3 where for CC2, when max CORESET duration is not more than 2, the span duration according to the span definition in the specification is equal to 2. However, according to the specification, CC2 is considered as monitoring with the limit corresponding to (7,3) and therefore grouped together with CC1 for the purpose of CA limit scaling. Span with (7,3) Span with (7,3) Span with (7,3) Span with (7,3) Figure 3 Example of different DL cells from the Ncells,r16DL,X,Y,μ Ncells,r16DL,X,Y,μ downlink cells having different span durations. A UE repots (7,3) for CC1 and (2,2),(4,3),(7,3) for CC2. For each CC, spans pattern are determined according to the span definition in the specification where span duration = max(max CORESET duration, Ymin). Hence, the term “per span” should be changed or at least clarified. It is more appropriate, e.g., to refer instead to a “resulting span” derived from the union of PDCCH monitoring occasions which then includes all PDCCH monitoring occasions in the “aligned” spans across the DL cells. It is reasonable to base the definition of “aligned span” on PDCCH monitoring occasions on all the cells from the Ncells,r16DL,X,Y,μ Ncells,r16DL,X,Y,μ downlink cells. Span is defined only for a cell. It is not clear to use the term “per span” for the total limits MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ since they are applied to PDCCH monitoring occasions in multiple spans across the DL cells. Revise the TP above by clarifying that the span in “per span” refers to the resulting span derived from the union of PDCCH monitoring occasions on all scheduling cells.

From feature view: the above change is not needed. Following the definition of span pattern, combination (7, 3) should be used for CC2 shown in the above example, i.e. span duration is 3 not 2. In addition, according to the current specification, only the cells using the same combination (X, Y) will be grouped together, that is the span duration is aligned among all the serving cells. Therefore, similar as Rel-15, it is ok to use the terminology “per span” in this case.

Update #2 for discussion:

Intel R1-2003737 The above definition may cause ambiguity in identification of certain span combinations across cells as aligned or non-aligned. For instance, when UE is configured with 1-symbol CORESET(s), then the above text still satisfies the condition defined for “union of PDCCH monitoring occasions” for the example in Figure 1, while this case should be considered as unaligned. Figure 1: An example case of unaligned span combinations across CC1 and CC2 that may be identified as aligned spans per current definition of “aligned spans”. To address this, the following alternative characterization that was also discussed during RAN1 #100b-E meeting is proposed: < unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0> per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if any pair of spans on the active DL BWP(s) of any two scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells X,Y are within a same set of up to Y consecutive symbols, or have their first symbols separated by at least X symbols, < unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0>.

- Support: Intel, Apple

From feature view: It seems there is some issue at least when search space sets are configured with 1-symbol CORESET(s). The TP from Intel can work. Alternatively, if people want to keep the current description as much as possible, the following potential update can be considered also.

< unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0> - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y and the starting symbol of any pair of overlapping spans are the same, - TBD, otherwise < unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0>

Question C-2-1: Which option do you prefer for the text proposal to correct the definition of “aligned spans” case?

- Option 1: Adopt the following text proposal for section 10.1 in TS 38.213:

< unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0> - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if any pair of spans on the active DL BWP(s) of any two scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells X,Y are within a same set of up to Y consecutive symbols, or have their first symbols separated by at least X symbols, - TBD, otherwise < unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0>

- Option 2: Adopt the following text proposal for section 10.1 in TS 38.213:

< unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0> - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y and the starting symbol of any pair of overlapping spans is the same, - TBD, otherwise < unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0>

Please provide your views and your reasons on the above text proposals for “aligned spans” case.

| Company | View |
| --- | --- |
| Feature lead | As discussed in RAN1#100b-e meeting, UE will first determine the combination (X, Y) on each serving cell, and then check whether the spans on the serving cell with the same combination (X, Y) is aligned or not. Therefore, if the first symbol of any pair of spans is different, then the ending symbol of the pair of spans is different either, since the span duration for all serving cells using combination (X, Y) is the same. Of course, it is possible that only some symbols in a span has PDCCH monitoring occasions, but these empty symbols should located at the end of a span, not at the start of a span. Therefore, it seems ok to only define the start symbol of a span as shown in option 2. The following detailed procedure should be applied according to the current spec: - Step 1: Determine the applicable combination (X, Y) to acquire the maximum number of non-overlapping CCEs and PDCCH candidates for each cell. This is done according to: “If the UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for two or more of the multiple combinations X,Y, the UE is expected to monitor PDCCH on the cell according to the combination X,Y associated with the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ.” - Step 2: For all cells that result into the same (X, Y) according to step 1, construct the union of all PDCCH monitoring occasions. Determine the applicable (X, Y) for the union of PDCCH monitoring occasions. If it is the same (X, Y) as for the single cells, the span configuration is considered as aligned. Otherwise, the spans are not aligned. |

- Issue C-3: Corrections on “unaligned spans” case

The following text has been captured in section 10.1 of TS38.213 for scaling PDCCH monitoring capability.

If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y, - TBD, otherwise Where Ncells,r16DL,j is a number of configured cells using Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with downlink cells using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by Ncells,r16cap-r16

As shown in the above text, the text for “unaligned spans” case is still TBD. Some companies provide views on this.

Option 1: Rel-15 slot-based PDCCH monitoring is applied for “unaligned spans”, i.e. if a configuration of search space sets in one or more cells would result to “unaligned” spans, the UE expects to be configured for Rel-15 PDCCH monitoring on those cells

- Support: Samsung

- Reasons

  - “Unaligned spans” is not a typical configuration

  - Maintain optional simplicity

- Cons

  - Revert the agreements

If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE - expects that the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,X,Y,μ downlink cells results to PDCCH monitoring according to the combination X,Y, and - is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cellsNcells,r16DL,(X,Y),μX,Y where Ncells,r16DL,j is a number of configured cells using Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with downlink cells using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

Option 2: Adopt the following text proposal for section 10.1 in TS 38.213:

< Unchanged parts omitted, TS 38.213, Subclause 10.1, Editor’s CR with changes accepted> If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y, - for the sum of the number PDCCH candidates and corresponding number of non-overlapped CCEs across any set of spans on the active DL BWP(s) of different scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, with at most one span per scheduling cell for each set of spans, otherwise < Unchanged parts omitted, TS 38.213, Subclause 10.1, Editor’s CR with changes accepted>

- Support: Intel

Option 3 (original proposal in RAN1#100b-e): Adopt the following text proposal for section 10.1 in TS 38.213:

< Unchanged parts omitted, TS 38.213, Subclause 10.1, Editor’s CR with changes accepted> If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y, - per set of spans across the active DL BWP(s) of all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells, with at most one span per scheduling cell for each set of spans, otherwise < Unchanged parts omitted, TS 38.213, Subclause 10.1, Editor’s CR with changes accepted>

- Support:

From feature view: It seems we need more discussion on this issue. Firstly, it would be good for us not to revert the agreement if possible. Relying on gNB to ensure aligned span cases or if impossible then following Rel-15 as proposed by option 1 may be too restrictive, especially considering potential limitation on the periodicity and slot offset of CSS. Of course, if gNB can be able to configure it as span aligned case, I think gNB do that as much as possible. More views are needed before we can make any proposal here.

Question C-3-1: Which option do you prefer for defining “unaligned spans” case?

Please provide your views and your reasons on the above question C-3-1 for “unaligned spans” case.

| Company | View |
| --- | --- |

- Issue C-4: Enhanced PDCCH monitoring capability for cross-carrier scheduling

Huawei (R1-2003525), Intel (R1-2001998) and Quectel (R1-2003942) discusses Rel-16 PDCCH monitoring capability for cross-carrier scheduling case. Two alternatives were discussed in the contributions:

- Option 1: Both the scheduling cell and scheduled cell for cross-carrier scheduling are restricted to be configured the same PDCCH monitoring capability (i.e., Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability) , and provided the same combination (X, Y) when Rel-16 PDCCH monitoring capability is configured,

  - Support: Intel, Huawei/HiSilicon, Spreadtrum,

  - Reasons:

    - To simplify the PDCCH BD/CCE dimensioning and overall operation considering typical use cases

- Option 2: There is no restriction on the PDCCH monitoring capabilities for scheduling cell and scheduled cell, but the calculations of MPDCCHtotal,(X,Y),μand CPDCCHtotal,(X,Y),μare based on the number of scheduled cells whose scheduling cells are configured with the same PDCCH monitoring capability and provided the same combination (X, Y) when Rel-16 PDCCH monitoring capability is configured.

  - Support: Quectel

  - Reasons:

    - Does not prohibit the use of cross-carrier scheduling with different PDCCH monitoring capabilities or with different combinations and maintains the scheduling flexibility with limited specification efforts

From feature view: More views are needed before making any proposal here.

Please provide your preference between option 1 and option 2 for enhanced PDCCH monitoring capability for cross-carrier scheduling.

| Company | View |
| --- | --- |