3GPP TSG RAN WG1 Meeting #100bis-e	  R1-20xxxxx

E-meeting, 25 May – 5 June, 2020

Agenda Item:	7.2.5.1

Source:	Moderator (Huawei)

Title:	Feature lead summary#1 on PDCCH enhancements

Document for:	Discussion and Decision

Introduction

In the RAN1 #99 meeting, the WID [1] on Physical Layer Enhancements for NR URLLC was concluded. In the RAN1#100bis-e meeting, remaining open issues and corrections will be discussed.

This document summarizes the key issues discussed under agenda item 7.2.5.1 based on the views in [2][3][4][5][6][7][8][9][10][11][12][13][14][15][16][17][18][19][20][21][22], and aims to identify a set of critical issues for RAN1#101-e email discussion.

Summary of issues raised for PDCCH enhancements

This section summarize the issues raised by companies on PDCCH enhancements, among which a set of issues can be identified for RAN1#101-e email discussions per the guidance from Chairman.

Draft recommendation on the email threads and scope are given in section 2.1.1 and the summary of detailed issues are given in section 2.2.

Recommendation for the scope of email threads

Draft recommendation for the scope of email threads (i.e. draft for first round email discussion)

Based on the summary of issues in section 2.2, the following recommendation are made for the scope of email threads.

---------------------------------------------------------------------------------------------------------------------------

Email discussion #1

Email discussion/approval on remaining issues on DCI format design:

- Issue A-1: Further extension of DCI size alignment due to the introduction of DCI format 0_2/1_2

- Issue A-3: Determination of DCI field sizes for the case of two HARQ-ACK codebooks

- Issue A-5: Completion of DMRS and PTRS reception procedure for PDSCH scheduled by DCI formats 1_2

- Issue A-4: Whether to change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2

- Issue A-10: Whether to use RBG as the granularity of the starting position and hopping offset for PUSCH scheduled by DCI format 0_2 with resource allocation type 1

Note 1: Issue A-2 will be included under email discussion #1 if it will be discussed under PDCCH enhancements based on the coordination

Note 2: Issue A-8, issue A-9 and Issue A-12 are postponed to next meeting right now, can be reconsider based on the discussion on the issues in red and whether to include A-2 in PDCCH enhancements.

Email discussion #2

Email discussion/approval on remaining issues on scaling PDCCH monitoring capability:

- Issue C-1: Corrections on span duration

- Issue C-2: Corrections on “aligned spans” case

- Issue C-3: Corrections on “unaligned spans” case

- Issue C-4: Enhanced PDCCH monitoring capability for cross-carrier scheduling

Email discussion #3

Email discussion/approval on remaining issues and miscellaneous corrections on enhanced PDCCH monitoring capability:

- Issue B-1: Minimum value of (pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16) for case 3

- Issue B-2: Whether to adopt nested reporting structure for reporting the supported combination (X, Y) for Rel-16 PDCCH monitoring capability

- Issue D-1: Correction on determination of a combination (X, Y) for PDCCH monitoring in section 10.1 in TS 38.213

- Issue D-2: PDCCH candidates allocation for primary cell in a slot or a span

- Issue C-5: Whether/how to extend Rel-16 PDCCH monitoring capability to multi-TRP case

Companies are encouraged to provide views on the scope of the email threads, especially on the issues highlight in red.

| Company | View |
| --- | --- |
| Nokia, NSB | - Issue A-4: we don’t think this to be required, but better to have a conclusion once and for all (to not need to see this repeatedly over different meeting cycles). - Issue A-6 will require a TP independently on the outcome of A-3, because what is there now is just simply wrong (CBG for PUSCH has nothing to do with HARQ-Ack codebook)! So I do not see any relation here – and if time permits, would be nice to include. But if FL thinks there is a connection, maybe we could discuss this as part of A-3 (after having an agreement there, during the 2nd week). - Issue A-10: we think this could be handled by gNB implementation, but if this will anyhow come up again in the next meeting – then let’s find at least some official conclusion now. Our comments from last meeting still apply… - Issue C-5: At least some conclusions could be captured, if to extend in Rel-16 or leave this to Rel-17 would be good to have (so that the Rel-16 focus is clear after this meeting). |
| Apple | Issue B-2 is against a standing agreement, and should be removed. |
| Qualcomm | 1st email thread: - If any of the red topics are to be removed, it would be good to include A-12 since it seems to be an easy fix. 2nd email thread: To balance the load, probably C4 can be deferred to the next meeting? 3rd email thread: - B2 was discussed before and concluded. There is no reason to discuss again. Besides that, the (X,Y) combination for the new PDCCH capability have different impacts on the implementation; in that regard, they are different from how (X,Y) for FG 3-5b was structured. - D1 is not clear; if there is only one valid (X,Y), is there an issue to solve? - Besides the extension of the new PDCCH capability to mTRP, there is also a case for it in NR-DC. We think both can be discussed; however, given the load in this meeting (overall, and not in this thread particularly), maybe we can have a conclusion to support/not support the new PDCCH for mTRP and NR-DC this time, and leave the discussion to the next meeting? |
| Samsung | A-4: Should discuss – a decision is needed – should align with the same setup from NR-U A-10: No need to discuss now C-5: We have been proponents of this but have realized that URLLC aspects for multi-TRP are supported by single-DCI. Fine to skip C-5. We had previously also suggested to consider NR-DC aspects but this was not picked up as there were more imminent topics – may be good to have some discussion now primarily for whether a new RRC parameter is needed (for –R16) – the rest should be a trivial extension of what currently exists in 38.213 and may be addressed next time. |
| Intel | 1st thread: Similar views as Nokia on A-4 and A-10. In particular, A-10 seems achievable by gNB implementation, and thus we could skip it in thread #1 in favor of a simpler one like A-12 as suggested by Qualcomm. 2nd thread: Support with suggestions from FL. 3rd thread: While we are also wary of discussing the issue of nesting reporting again, we are open to discussing B-2 in light of the agreed limits from last meeting wherein the requirements satisfy a nested processing load for the UE across the combinations. Having said that, there are still some benefits in allowing separate reporting from the UEs, and in this regard, we agree with Qualcomm that directly comparing to Rel-15 FG 3-5b may not be accurate. In this regard, we also hold the view that, in UE features, FG 3-5b should not be a pre-requisite for R16 enhanced PDCCH monitoring. Thus, overall, it would be good to discuss such details next week. On C-5, we think only the second part of C-5 is relevant – that is, for support of m-DCI in the CCs with R15 monitoring capability configuration. However, this does not seem high priority given the number of items in this thread already, and in fact, may even be addressed by MIMO folks along with Aris. |
| Quectel | 3rd thread: B-2: how to determine the span duration in case of 3-symbol CORESET may be dependent on what reporting structure is adopted (nested vs. independent). Not sure whether it can be fully handled by UE feature discussion. C-5: Similar view as Nokia. At least some high-level conclusions could be beneficial so that companies could be clear whether to continue paying efforts on these combined issues in Rel-16. |
| Vivo | On email #1, we think A-2 can be discussed in this PDCCH thread because it is much relevant to DL control design and due to the fact that there are lots of issues to be solved in UCI sessions. If A-2 or any other issue offloaded from UCI session are to be discussed here, it would be good to not include A-4 or A-10 in this thread considering the workload for the discussion. On email #3, we are open to discuss whether Rel-16 PDCCH monitoring capability can be extended to multi-TRP case. But we think C-5 is more relevant to MIMO and better to discuss in eMIMO session. |
| ZTE | On A-4: We are fine to discuss this issue. But we don’t see much difference between DL and UL, and we should either not adopt this change or adopt such change for both DCI format 0_2 and DCI format 1_2. Thus, we suggest adding the case for ‘DCI format 1_2’ also if we agreed to discuss this. On A-10: Agree that it is a gNB implementation issue. No need to discuss in this or future meetings. On B-2: Prefer to discuss in this meeting. As noted by Intel, the agreed limits C/M satisfy a nested processing load for the UE across the combinations. While allowing a UE reports combination (X,Y) separately may cause Rel-16 PDCCH monitoring capability could be even lower than Rel-15. On C-5: Similar with Nokia/Quectel, we can at least have a conclusion on whether to extend to multi-TRP case or not. |
| DOCOMO | 1st thread: A-12 can be included in the 1st email thread instead of A-10. A-12 is straightforward. As A-10 could be handled by gNB implementation, no need to discuss now. 2nd thread: support the FL suggestion. 3rd thread: on C-5, similar view with Nokia/Quectel/ZTE. Better to have high-level conclusion whether to discuss it or not. |
| Spreadtrum | We are fine with the scope of email discussion #1 and #2. For Issue C-5 in email discussion #3, we agree with Samsung that there is only single DCI enhancements for URLLC PDSCH, and MIMO session does not consider multi-DCI case for URLLC traffic. Thus it is not essential to extent Rel-16 PDCCH monitoring to multi-DCI case. So we prefer skip Issue C-5. |
| HW/HiSi | Thread #1: A-10 should be included. We understand that some companies argue that it can be left to gNB implementation. We have a different view on this. This should be the outcome of a technical discussion. We agree with Nokia in the sense that we should try to reach an official conclusion. Besides the need to specify an offset granularity in RBG level, we also see a potential update of 38.214, where the starting position is calculated in RB granularity, but the indication in the DCI is in RBG granularity. Thread#3: Not necessary to discuss C-5. We agree with the observation from Samsung, that URLLC aspects for multi-TRP are only supported by single-DCI. |
| OPPO | Generally, we are fine with FL' suggestion in addition with DAI issues proposed by OPPO in R1-2004114. We suggest to discuss it due to there is configuration restriction between HARQ-ACK codebook/CBG transmission under small DAI bit width. However, according to previous agreement, we just want to reduce DAI bits, but it does not restrict dynamic HARQ-ACK codebook or CBG transmission configuration. So we suggest to correct this issue. |

In addition, issue A-11 and issue D-3 are proposed to be handled by editor during editor CR phase.

Status for the scope of email threads after first round email discussion

Based on the views shared in the first round email discussion, the status is summarized as below:

- A-4:

  - 2 companies (Nokia, Intel): Not necessary but ok to discuss.

  - 1 company (ZTE): prefers either not to discuss, or discuss for both DCI format 0_2 and DCI format 1_2

  - FL suggestion: Recommend to include issue A-4. Better to have a conclusion since this issue was submitted several meetings. As to whether to also apply to DCI format 1_2, we can further check when we start the formal email discussion without changing the title of the issue, since only have 1 company providing the view that it can be applied for DL. From feature lead point of view, I think DL and UL is different, since for DL gNB cannot tell whether the UE received the first transmission and stored the corresponding soft values or not, in this case it would be better to use RV 0 and RV 3, which are self-decodable.

- A-10:

  - 6 companies (Samsung, Nokia, Intel, ZTE, NTT DOCOMO, MTK): Prefer to remove. Could be handled by gNB implementation.

  - FL suggestion: Remove issue A-10 with some actions below:

    - Have an official conclusion: Leave it to gNB implementation to configure the starting position and hopping offset for PUSCH scheduled by DCI format 0_2 with resource allocation type 1 in the granularity of RBG

    - Double check if any update needed in TS 38.214 to transform the indicated RBG to RB. If needed, can discuss it next meeting

- C-5:

  - 5 companies (Spreadtrum, Samsung, Intel, Huawei/HiSilicon, MTK): prefers to remove since multi-DCI is aimed for throughput, i.e., for eMBB

  - 4 companies (Nokia, Quectel, ZTE, DOCOMO): Prefer to discuss to have some conclusion on whether to support it or not

  - 2 companies (Vivo, Intel): More appropriate to discuss in MIMO

  - 1 company (Qualcomm): prefers not to conclude that “M-DCI based Multi-TRP is not extended to Rel-16 span-based PDCCH monitoring capability in Rel-16”, prefer to discuss in this meeting but if impossible can delay to next meeting

  - FL suggestion: Remove issue C-5 with some actions below:

    - Have an official conclusion: M-DCI Multi-TRP based on CA framework is not extended to Rel-16 span-based PDCCH monitoring capability in Rel-16.

    - Check question C-5-2 for whether to apply multi-DCI based multiple TRP on serving cell using Rel-15 PDCCH monitoring capability for case 3, and discuss in next meeting.

  - FL suggestion 2 (update): Remove issue C-5 for this meeting, can see the situation for next meeting

- B-2:

  - 3 companies (Apple, MTK and Qualcomm): prefers to remove since it is to revert the agreement we made in RAN1#99

  - Samsung prefers to remove assuming FG 3-5b is a prerequisite or can be discussed under UE feature, however at least two companies (Qualcomm and Intel) already expressed that they don’t agree FG 3-5b should be the prerequisite

  - FL suggestion: Recommend to include B-2. Firstly, although it may revert the agreement, it is also true that without nested structure then it may not be aligned with the note “The total number of monitored PDCCH candidates is not smaller than the limit per slot in Rel-15” in the agreement. Secondly, as Intel and ZTE commented that the new agreed number should be able to support nested structure. Thirdly, we cannot leave this kind of complicated issue to UE feature since we don’t have much time there. Finally, anyway good to have conclusion whether to have it or not, otherwise I don’t think we can move on UE feature discussion.

- A-12:

  - 2 companies (Qualcomm and DOCOMO): prefer to include A-12 if some can be removed from the current list.

  - FL suggestion: Recommend to include A-12. Since it is a very straightforward correction

- C-6:

  - 2 companies (Qualcomm and Samsung): prefer to have some discussion on NR-DC

  - FL suggestion: Recommend to include C-6 since we will remove C-5. Essential to make the spec complete though there is no any input from any paper.

Based on the above status, the updated recommendation is as below:

Email discussion #1

Email discussion/approval on remaining issues on DCI format design:

- Issue A-1: Further extension of DCI size alignment due to the introduction of DCI format 0_2/1_2

- Issue A-3: Determination of DCI field sizes for the case of two HARQ-ACK codebooks

- Issue A-5: Completion of DMRS and PTRS reception procedure for PDSCH scheduled by DCI formats 1_2

- Issue A-13 (from UCI): Type2 HARQ-ACK codebook construction related to DAI bit width

- Issue A-4: Whether to change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2

- Issue A-12: Correction on bandwidth part operation

Email discussion #2

Email discussion/approval on remaining issues on scaling PDCCH monitoring capability:

- Issue C-1: Corrections on span duration

- Issue C-2: Corrections on “aligned spans” case

- Issue C-3: Corrections on “unaligned spans” case

- Issue C-4: Enhanced PDCCH monitoring capability for cross-carrier scheduling

Email discussion #3

Email discussion/approval on remaining issues and miscellaneous corrections on enhanced PDCCH monitoring capability:

- Issue B-1: Minimum value of (pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16) for case 3

- Issue B-2: Whether to adopt nested reporting structure for reporting the supported combination (X, Y) for Rel-16 PDCCH monitoring capability

- Issue D-1: Correction on determination of a combination (X, Y) for PDCCH monitoring in section 10.1 in TS 38.213

- Issue D-2: PDCCH candidates allocation for primary cell in a slot or a span

- Issue C-6: Text proposal for Rel-16 PDCCH monitoring capability for NR-DC in section 10 in TS 38.213

Please comment if you have strong concern on the above scope.

| Company | View |
| --- | --- |

Potential conclusion #1:

Leave it to gNB implementation to configure the starting position and hopping offset for PUSCH scheduled by DCI format 0_2 with resource allocation type 1 in the granularity of RBG.

Please comment if you have strong concern on the above conclusion.

| Company | View |
| --- | --- |

Potential conclusion #2:

M-DCI based Multi-TRP based on CA framework is not extended to Rel-16 span-based PDCCH monitoring capability in Rel-16.

Please comment if you have strong concern on the above conclusion.

| Company | View |
| --- | --- |

Note: Issue A-11 and issue D-3 are to be handled by editor during editor CR phase.

Please comment if you have strong concern on the above note.

| Company | View |
| --- | --- |

Summary of detailed issues

Issues raised on DCI format design

| Issue # | Description | Source | Recommended handling |
| --- | --- | --- | --- |
| A-1 | DCI size alignment in TS 38.212 - If both formats 0_1/1_1 and 0_2/1_2 are configured to monitor in USS, their size should be made distinguishable - Other extension due to the introduction of DCI format 0_2/1_2 Note: Details seen in section 3 | Ericsson [4] Huawei/HiSi [5] Vivo [3] Nokia [6] MTK [8] LG [16] CATT [7] Samsung [12] Intel [11] Spreadtrum [15] Qualcomm [21] NTT DCM [20] ZTE [2] Panasonic [14] FUTUREWEI [10] | Included in email discussion #1 Reason: - Open issues need to be solved; - Proposed by Top 1 number of companies; |
| A-2 | Priority indication via DCI format 0_1/1_1 and 0_2/1_2 - How to determine the priority of transmissions scheduled by the DCI formats 0_1/1_1/0_2/1_2 for a UE that doesn’t support a DCI format scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to monitor? Note: Details seen in section 3 | Intel [12] MTK [8] ZTE [2] | Further coordination based on the workload for PDCCH enhancement and UCI enhancements Reason: - Open issues need to be solved; - Some offline suggestion to move to PDCCH since workload for UCI is too heavy |
| A-3 | Determination of DCI field sizes for the case of two HARQ-ACK codebooks - Whether to confirm the working assumption from RAN1#99 with or without updates | Ericsson [4] Vivo [3] Nokia [6] MTK [8] NTT DCM [20] Panasonic [14] Spreadtrum [15] LG [16] | Included in email discussion #1 Reason: - Needs to be fixed in the specification |
| A-4 | Changing the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2 | Ericsson [4] Samsung [12] MTK [8] | Included in email discussion #1 unless strong concern Reason: - Belongs to optimization - Need some time to discuss |
| A-5 | Completion of DMRS and PTRS reception procedure for PDSCH scheduled by DCI formats 1_2 (Sec. 7.3.1.2.3 in 38.212, Sec. 5.1.6.2 & 5.1.6.3 in 38.214) | Nokia [6] | Included in email discussion #1 Reason: - Missing PTRS reception procedure for PDSCH scheduled by DCI format 1_2 in the spec |
| A-6 | Remove CBG PUSCH additions to DCI format 0_1 (38.212, Sec. 7.3.1.1.2) | Nokia [6] MTK [8] CATT [7] Sharp [19] | Depending on the outcome of issue A-3 Reason: - Whether the TP is needed or not depend on the outcome of the proposal under issue A-3. |
| A-7 | PUCCH resource determination for reduced size of PRI field | Spreadtrum [15] MTK [8] | To be discussed under UCI enhancement as in RAN1#100b-e Reason: - Was discussed under UCI enhancements in RAN1#100b-e, and thus can be continued there considering more contributions submitted there |
| A-8 | Correction on missing case of PUSCH release for search space sharing | Sharp [19] | Postpone to next meeting. Can be discussed if workload available Reason: - Issue is valid |
| A-9 | Correction on Transmission configuration indication in DCI format 1_2 | ASUSTeK [20] | Postpone to next meeting. Can be discussed if workload available Reason: - The issue is valid, but some companies doubt the necessity |
| A-10 | Correction on frequency hopping for PUSCH scheduled by DCI format 0_2 - Whether to use RBG to replace RB for the granularity of the starting position and hopping offset for PUSCH scheduled by DCI format 0_2 with resource allocation type 1 | Huawei/HiSi [5] | Included in email discussion #1 unless strong concern Reason: - Belongs to optimization - Need some time to discuss |
| A-11 | Align the RRC parameter names between TS 38.331 and TS 38.212 | CATT [7] | Can be handled by editor in the editor CR phase Reason: - It is straightforward editorial changes |
| A-12 | Correction on bandwidth part operation to capture BWP switching with DCI format 0_2 and DCI format 1_2 | Huawei/HiSi [5] | Postpone to next meeting. Can be discussed if workload available Reason: - To address some missing case |
| A-13 | Type2 HARQ-ACK codebook construction related to DAI bit width |  | Included in email discussion #1 Reason: - Open issue to be addressed - Moved from UCI to PDCCH enhancements |

Issues raised on enhanced PDCCH monitoring capability

| Issue # | Description | Source | Recommended handling |
| --- | --- | --- | --- |
| B-1 | Minimum value of (pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16) for case 3 | Ericsson [4] Huawei/HiSi [5] Intel [11] NTT DCM [20] | Included in email discussion #3 Reason: - Open issues need to be solved; |
| B-2 | Whether to adopt nested reporting structure for reporting the supported combination (X, Y) for Rel-16 PDCCH monitoring capability | Ericsson [4] | Included in email discussion #3 Reason: - Better to discuss since it will have impact on UE feature signaling |
| B-3 | UE behavior if the obtained span monitoring configuration is invalid | MTK [8] | No discussion in RAN1#101-e Reason: - Should be ok if no any specification on this issue |
| C-1 | Corrections on span duration | Huawei/HiSi [5] Samsung [12] ZTE [2] Quectel [143] | Included in email discussion #2 Reason: - Remaining issues from RAN1#100b-e |
| C-2 | Corrections on “aligned spans” case | Ericsson [4] Intel [11] Apple [18] | Included in email discussion #2 Reason: - Remaining issues from RAN1#100b-e |
| C-3 | Corrections on “unaligned spans” case | Samsung [12] ZTE [2] Intel [11] | Included in email discussion #2 Reason: - Remaining issues from RAN1#100b-e |
| C-4 | Enhanced PDCCH monitoring capability for cross-carrier scheduling | Intel [11] Huawei/HiSi [5] Quectel [13] Spreadtrum [15] | Included in email discussion #2 Reason: - Open issue to be solved |
| C-5 | Whether/how to extend Rel-16 PDCCH monitoring capability to multi-TRP case | Samsung [12] Ericsson [4] ZTE [2] Apple [18] Quectel [13] MTK [8] | Included in email discussion #3 is no strong concern Reason: - Related to both eURLLC and eMIMO |
| C-6 | Text proposal for Rel-16 PDCCH monitoring capability for NR-DC in section 10 in TS 38.213 |  | Included in email discussion #3 Reason: - Open issue to be solved |
| D-1 | Correction on determination of a combination (X, Y) for PDCCH monitoring in section 10.1 in TS 38.213 | Samsung [12] | Included in email discussion #3 Reason: - Workload tradeoff |
| D-2 | PDCCH candidates allocation for primary cell in a slot or a span | Sharp [19] | Included in email discussion #3 Reason: - Workload tradeoff |
| D-3 | Correction on higher layer parameter | Huawei/HiSi [5] | Handled by editor |

DCI format scheduling Rel-16 URLLC

Based on the contributions from companies, the following issues related to DCI format design are discussed.

- Issue A-1: Further extension of DCI size alignment due to the introduction of DCI format 0_2/1_2

In the current TS 38.212, DCI size alignment is extended due to the introduction of DCI format 0_2/1_2 with a few places in bracket due to lack of explicit agreements. WILUS (R1-2002634) provides a very nice table to summarize the DCI format sizes after each step as below:

Table 1. DCI format size after each step (TS38.212 v16.1.0)

|  | After Step 0 | After Step 1 | After Step 2 | After Step 2A | After Step 3 Check | After Step 4A | After Step 4B | After Step 4C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCI format 0_0 in CSS | Size A (padding) |  |  |  | 3 different DCI sizes with C-RNTI ? If no, go to step 4A~4C Otherwise, finish | Size A | Size A | Size A |
| DCI format 1_0 in CSS | Size A (truncation of FDRA field) |  |  |  |  | Size A | Size A | Size A |
| DCI format 0_0 in USS |  | Size B (padding) |  |  |  | Size B Size A | Size A | Size A |
| DCI format 1_0 in USS |  | Size B (padding) |  |  |  | Size B Size A | Size A | Size A |
| DCI format 0_1 |  |  | Size C (one bit insertion to differentiate Size B is needed) |  |  | Size C | Size C | Size C Size H |
| DCI format 1_1 |  |  | Size D (one bit insertion to differentiate Size B is needed) |  |  | Size D | Size D | Size D Size H |
| DCI format 0_2 |  |  |  | Size E (one bit insertion to differentiate Size B is needed) |  | Size E | Size E Size G | Size G |
| DCI format 1_2 |  |  |  | Size F (one bit insertion to differentiate Size B is needed) |  | Size F | Size F Size G | Size G |

The remaining issue is how to distinguish the following cases.

Case 1: Fallback DCI format (0_0, 1_0) and new DCI format (0_2, 1_2)

Case 2: Non-fallback DCI format (0_1, 1_1) and new DCI format (0_2, 1_2)

The issue of DCI size alignment (i.e. appending zero padding bit(s) when necessary, or if gNB should guarantee the size to be different) has been discussed in the RAN1#100-e email discussion thread [100e-NR-L1enh_URLLC_PDCCH-01]. Unfortunately, consensus was not achieved.

Based on the inputs from the contributions, company positions are summarized as below:

- Option 1:

  - One zero-padding bit is added to DCI format 0_2/1_2 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_0/1_0 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_1/1_1 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 and DCI format 0_1/1_1 monitored in the same USS.

  - Support: FUTUREWEI, WILUS, Ericsson, ZTE, Huawei/HiSilicon, Vivo, NTT DOCOMO, CATT, Spreadtrum, Panasonic, OPPO

  - Reason

    - Same principle in Rel-15 when both DCI formats 0_0/1_0 and DCI formats 0_1/1_1 are configured to be monitored in USS

    - Cons from option 2 and option 3

      - Restrictions on configured functionality: it is not guaranteed that there will be many fields which can be easily adjusted without impacting desired functionality

        - e.g., if a large number of processes are configured to differentiate the DCI size, then the UE will not know that only a few processes that could possibly be used, thus potentially degrading performance. The performance difference will increase if the gNB had desired to configure FBRM along with the reduced processes, assuming that the extra memory could be used to improve performance of that ultra-reliable application.

- Option 2:

  - A UE is not expected to monitor a first decoding candidate with DCI format 0_0/1_0  and a second candidate with DCI format 0_2/1_2, where the two decoding candidates are mapped to the same resource and the DCI formats 0_0/1_0 and 0_2/1_2 have the same size.

  - A UE is not expected to monitor a first decoding candidate with DCI format 0_1/1_1  and a second candidate with DCI format 0_2/1_2, where the two decoding candidates are mapped to the same resource and the DCI formats 0_1/1_1 and 0_2/1_2 have the same size.

  - Support: Intel, Qualcomm, Huawei/HiSilicon, MTK, LG, Apple

  - Reasons

    - It would be feasible to have different DCI sizes by gNB configuration

- Option 3: Supporting zero padding in case the DCI size budget is not exceeded (i.e. in steps 2A, 2B) and not supporting zero padding in case the DCI size budget is exceeded (i.e. in steps 4X), i.e.

  - For step 2A, one zero-padding bit is added to DCI format 0_2/1_2 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_0/1_0 monitored in another USS, i.e. remove the brackets in step 2A of the DCI size alignment of Sec. 7.3.1.0 of TS 38.213.

  - For step 2B, one zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 and DCI format 0_1/1_1 monitored in the same or a different USS when having the same size after step 2A, i.e. support the intention of step 2B as outlined in the RAN1#100-e FL summary in R1-2001404 with a slight change to include the same or different search space.

  - For steps 4X, do not support zero padding to distinguish the DCI sizes as in case of Rel-15, i.e. no changes to steps 4A to 4C are seen as needed.

  - Support: Nokia

- Option 4 There is no need to specify any mechanism to distinguish DCI formats 0_2/1_2 and DCI formats 0_0/1_0.

  - Support: Samsung

  - Reasons

    - It would be feasible to have different DCI sizes by gNB configuration

Feature lead view: We had intensive discussion in RAN1#100-e meeting, and unfortunately we was not able to achieve agreement since both option 1 and option 2 got objections. From feature lead perspective, I do agree with some companies that this should not be considered as an objectionable issue. In addition, we did have tried different ways to progress the discussion but in the end stuck still. At this stage, I don’t have a good recommendation but go with the majority view, i.e. option 1.

Proposal 3-1:

  - One zero-padding bit is added to DCI format 0_2/1_2 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_0/1_0 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_1/1_1 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 and DCI format 0_1/1_1 monitored in the same USS

Please provide your views here. Please also share if you think option 3 from Nokia can be a potential compromise solution if you don’t accept the above proposal 3-1.

| Company | View |
| --- | --- |

Question A-1-1: Whether to use FBRM on the downlink with configurable size for HARQ process number field to improve the reliability of URLLC

As discussed in R1-2003722, as a UE may implement its soft buffer management according to the maximum number of HARQ processes, when a smaller number of HARQ processes are configured, it can be expected that the average memory per process can increase. However, if a large number of processes are configured to differentiate the DCI size, then the UE will not know that only a few processes that could possibly be used, thus potentially degrading performance.

As discussed in R1-2003722, with the configurable size number of HARQ channels, the performance will increase if the gNB uses FBRM instead of LBRM along with the reduced processes, with the UE using the extra memory for FBRM to improve performance of that ultra-reliable application. In Rel-15 the number of HARQ processes is not configurable, and LBRM is always used on the downlink. In Rel-16 LBRM or FBRM may not have been discussed. When the number of bits is smaller than the maximum, FBRM should be possible. Therefore, R1-2003722 proposes that FBRM is used on the downlink to improve the reliability of URLLC.

Feature lead view: More inputs are needed on this aspect before being able to make any proposal or conclusion.

Companies are encouraged to provide your views on whether to use FBRM on the downlink with configurable size for HARQ process number field.

| Company | View |
| --- | --- |

Question A-1-2: Whether DCI size alignment is performed slot-by-slot basis or in a BWP across slots

## Apple (R1-2004221) In offline discussions with companies, it also appears that different companies have different understandings concerning the DCI size alignment procedure in Rel-15: - Understanding 1: DCI size alignment is performed over all search spaces configured at a BWP and across all slots, then for a given DCI format at a given search space, the DCI size is the same across slots; - Understanding 2: DCI size alignment is performed on a slot by slot basis over search spaces present in each slot, then for a given DCI format at a given search space, the DCI size may vary across slots. We have checked the discussion history on DCI size alignment in Rel-15: At RAN1 NR Ad Hoc meeting in Jan. 2018, the following working assumptions were reached: Agreements: For one carrier: - (working assumption) Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP - (working assumption) At most 4 different DCI sizes are monitored by the UE per slot - At most 3 different DCI sizes are monitored per C-RNTI per slot - Payload size for formats 0-1 and 1-1 may differ Working assumption: - The number of bits in the resource allocation field for format 0-0 and 1-0 depends on search space: - In CSS(s) in CORESET 0, use initial DL BWP for DCI size determination and RB numbering - FFS If a UE monitors 0-0/1-0 in CSS in CORESET 0 in a slot, it does not monitor formats 0-0 or 1-0 (or 2-x family in case they have a size aligned with 0-0/1-0) in any other search space - Otherwise, use active BWP for DCI size determination and RB numbering At RAN1 #92, the following were reached: Agreement: Above working assumption is replaced by the following: - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWP - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled - FFS: Otherwise, for format 0-0/1-0, the size is given by the initial DL BWP - FFS: how to meet the C-RNTI size and DCI size budget per slot - align 0-1 and 1-1 - configure active BWP such that the DCI size is the same as of the initial BWP - do not configure 0-1 and 1-1 - do not configure 0-0/1-0 in USS - other are not precluded - FFS: for format 0-0/1-0, how to interpret the frequency-domain field in a DCI with a size defined from a BWP with a different size than the BWP it is applied to At RAN1 #92bis, the following were reached: Agreement: - Confirm the following working assumption: - (working assumption) At most 4 different DCI sizes are monitored by the UE per slot - At most 3 different DCI sizes are monitored per C-RNTI per slot Conclusion: - It is understood that DCI sizes to monitor do not vary dynamically from slot-to-slot (other than impact due to BWP switching) but are based on RRC configuration, CSS vs. USS, and/or active BWP. Agreements: - To confirm the following working assumption with update Working assumption: - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWP - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled - FFS: Otherwise, for format 0-0/1-0, the size is given by the initial DL BWP From the discussion history, it can be seen while the early working assumptions might leave a room for different readings on how to ensure at most 4 DCI sizes are monitored by a UE per slot: - DCI size alignment is per slot, the limit is checked and met on a slot by slot basis; - DCI size alignment is per BWP, the limit is checked and met by examining all search spaces and across slots, and consequently there are at 4 DCI sizes per slot; The conclusion from RAN1 #92bis indicates unambiguously that DCI size alignment is not performed on a slot-by-slot basis (otherwise DCI sizes to monitor can vary dynamically from slot to slot). We conclude previous RAN1 decisions support the position that DCI size alignment is conducted for a BWP across slots rather than on a slot-by-slot basis. To facilitate discussion on DCI size alignment, we have Proposal 4: Note it in the chairman’s notes that DCI size alignment is conducted for search spaces in a BWP across slots rather than on a slot-by-slot basis.

Feature lead view: It would be good to clarify since it was observed that different companies may have different understanding. However, since it will have impact on Rel-15 also, it would be better to clarify it under Rel-15 maintenance first.

- Issue A-2: Priority indication via DCI format 0_1/1_1 and 0_2/1_2

The following agreements was made in RAN1#99:

Agreement:

When both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities.

- This feature is UE optional

Intel, MTK and ZTE also discuss how to determine the priority of transmissions scheduled by the DCI formats 0_1/1_1/0_2/1_2 for a UE that is not capable of supporting this feature under PDCCH enhancements. The following two options were proposed:

- Option 1: DCI formats 0_1/1_1 may only schedule PUSCH or HARQ-ACK transmission with priority index 0, while DCI formats 0_2/1_2 may still schedule PUSCH or HARQ-ACK transmission associated with either priority index 0 or 1.

  - Support: Intel, MTK, ZTE

- Option 2: DCI formats 0_1/1_1 only schedule PUSCH or HARQ-ACK transmission with priority index 0, while DCI formats 0_2/1_2 only schedule PUSCH or HARQ-ACK transmission associated with priority 1.

  - Support: Samsung

Feature lead view: The issue was discussed under UCI enhancements agenda in RAN1#100bis-e but not concluded yet. The assumption is that it will be discussed again under UCI enhancements. However, we got some offline suggestion that it can be discussed again under PDCCH enhancements since the load for UCI enhancements is too heavy. Further coordination will be done among the feature leads.

- Issue A-3: Determination of DCI field sizes for the case of two HARQ-ACK codebooks (38.212, Section 7.3.1.2.2 and 7.3.1.2.3)

In the RAN1#99 meeting, the following agreement was made:

Working assumption:

When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

Ericsson, Vivo and CATT propose to confirm the above working assumption. Some companies propose to confirm the above working assumptions with some updates as summarized as below:

Proposed update #1:

If the UE is configured with dynamic priority indication for DCI formats 0_1, 0_2, 1_1 or 1_2 (using PriorityIndicator-ForDCIFormat0_1/1_1/0_2/1_2)When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following DCI fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for the low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

  - Support: Nokia, Ericsson, MTK, NTT DOCOMO

  - Reasons:

    - Configuration of two HARQ-ACK codebooks does not necessarily mean that a particular DCI format can be used to schedule both HARQ-ACK priorities.

    - A more precise condition is to check whether the priority indicator field is present in the format, i.e. if more than one priority is scheduled by the respective DCI format as otherwise, the DCI format size may be unnecessarily large.

Feature lead view: When the agreements and TS 38.212 was made, it was not clear whether configuring priority indicator in DCI is the only way to enable one DCI format scheduling different priorities, e.g. some RRC configured way can be considered. Based on the discussion for the recent meetings, it seems the proposal is reasonable and the change can be update accordingly.

Proposed update #2:

- When the UE is configured with two HARQ-ACK codebooks and for the case when any of DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for the low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

- PRI

  - Support: Vivo，Panasonic

  - Reasons:

    - PUCCH resources are also separately configured for HARQ-ACK codebooks.

  - Not support: Spreadtrum

  - Reasons:

    - The bit width is explicitly configured by gNB, not relying on any other configuration.

Feature lead view: PRI was actually discussed also when we made the agreement, the reason we didn’t include it is similar as what Spreadtrum gave here, since for PRI RRC will configure the number of bits for PRI directly, it seems not really necessary to have the alignment for different HARQ-ACK codebook, since gNB should be able to configure the same number of bits directly.

Proposed update #3:

When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for the low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

- PUCCH resource indicator

  - Support: LG

  - Reasons:

    - The CBG operation for PUSCH is nothing to do with two HARQ-ACK codebook constructions.

Feature lead view: For PRI, similar views as that for proposed update #2. For deleting uplink DCI format 0_1, it seems reasonable. Similar issue raised under issue A-6 also.

Proposal 3-2: Confirm the working assumption below with updates:

Working assumption:

If the UE is configured with dynamic priority indication for DCI formats 0_1, 0_2, 1_1 or 1_2 (using PriorityIndicator-ForDCIFormat0_1/1_1/0_2/1_2)When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following DCI fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

Please provide your views and the corresponding reasons on the above proposal 3-2.

| Company | View |
| --- | --- |

- Issue A-4: Whether to change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2?

Ericsson R1-2003439 For DCI format 1_2 scheduling PDSCH, if only one bit is signalled, the redundancy version to be applied is either 0 or 3. This is a reasonable choice for PDSCH since both RV 0 and 3 are self-decodable for high code rate, and error cases exist where the gNB cannot tell whether the UE received the first transmission and stored the corresponding soft values or not. This is not the case for PUSCH. If the UE does not transmit the PUSCH correctly due to a missed grant, it is possible for the gNB to detect this, e.g. by looking at the noise level estimate based on DMRS. In this case the gNB can schedule the retransmission using RV 0 (basically treating it as the first transmission), which gives better performance than using RV 3 for a first transmission. On the other hand, if the first PUSCH transmission is transmitted correctly, but not decoded at the gNB due to a noisy transmission, the gNB would like to schedule the retransmission using RV 2, and soft combine with the first transmission. This gives better performance than using RV 3, as can be seen in [3] where Figure 1 appears. For this case, LDPC base graph (BG) #1 is used for information block size of K=1056 bits, and two consecutive transmissions are soft combined before decoding. As can be observed from Figure 1, for medium to high code rates above 2/3 (=0.67), the difference between using RV 3 and RV 2 for the second transmission is more than 1.5 dB over an AWGN channel. Figure 3 Required SNR for decoding after two transmissions for different RV orders for BG1. K is the TBS including CRC bits. Dynamically scheduled PUSCH is a case where there is no ambiguity about whether transmission occurred, or which instance of a transmission occurred. Self-decodability is not important for an individual retransmission. Hence the gNB should be able to schedule for best performance, i.e. it should be able to signal RV 2. For dynamically scheduled PUSCH, there is no ambiguity at the gNB whether the first transmission occurred or not, and RV should be chosen to maximize performance. The note above did not capture these aspects and thus cannot be used to conclude on the RV field for DCI format 0_2. For performance reasons and for alignment with NR-U agreement, we have the following proposal. The text proposal for TS 38.212 is also provided below. When only one bit is used to signal RV in DCI format 0_2, it indicates either RV 0 or RV 2. ------------------ Text Proposal for 38.212 Section 7.3.1.1.3 ------------------ - Redundancy version – 0, 1 or 2 bits determined by higher layer parameter NumberofbitsforRV-ForDCIFormat0_2 - If 0 bit is configured, rvid to be applied is 0; - 1 bit according to Table 7.3.1.2.3-1 7.3.1.1.2-34; - 2 bits according to Table 7.3.1.1.1-2. ----------------------------------------------End of proposed TP --------------------------------------------------.

Samsung (R1-2003865), MTK (R1-2003684) has the same view as Ericsson. More views are needed before making any decision on this issue. Companies are encouraged to provide their views on this.

Feature lead view: It looks beneficial to make the change. However, more inputs are needed before we can make any proposal here.

Question A-4-1: Is it necessary to change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2?

Please provide your views and the corresponding reasons on the above question A-4-1.

| Company | View |
| --- | --- |

- Issue A-5: Completion of DMRS and PTRS reception procedure for PDSCH scheduled by DCI formats 1_2 (Sec. 7.3.1.2.3 in 38.212, Sec. 5.1.6.2 & 5.1.6.3 in 38.214)

R1-2003577 proposed to simply follow the same logic for PDSCH with DCI format 1_2 as agreed for PUSCH with DCI format 0_2 by agreeing several related TPs (which are the DL / PDSCH / DCI format 1_2 equivalent of the related TPs agreed as part of Email discussion [100b-e-NR-L1enh-URLLC-eCG-01] with the outcome reported in R1-2002804.

TP2-2a:

- 5.1.6.2 DM-RS reception procedure ## <Unchanged text is omitted> If a UE receiving PDSCH scheduled by DCI format 1_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 or a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB DMRS-DownlinkConfig, the UE may assume that the following configurations are not occurring simultaneously for the received PDSCH: - any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and the other UE(s) sharing the DM-RS REs on the same CDM group(s), and - PT-RS is transmitted to the UE. ## <Unchanged text is omitted> - 5.1.6.3 PT-RS reception procedure The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 1_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 and to a UE receiving PDSCH scheduled by DCI format 1_0 or format 1_1 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB. A UE shall report the preferred MCS and bandwidth thresholds based on the UE capability at a given carrier frequency, for each subcarrier spacing applicable to data channel at this carrier frequency, assuming the MCS table with the maximum Modulation Order as it reported to support. ## <Unchanged text is omitted>

TP 2-2b:

- 7.3.1.2.3 Format 1_2 ## <Unchanged text is omitted> - Antenna port(s) – 0, 4, 5, or 6 bits - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2 is not configured; - Otherwise 4, 5 or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4. If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 and is configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2 but configured with one or more of dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.2.2-1/2/3/4. ## <Unchanged text is omitted>

Feature lead view: The issue is valid and needs to be corrected. However, for the TP 2-2b above for DCI format 1_2, it seems not necessary. This is different from the case for PUSCH, where the sentence is deleted to cover configured grant case. However, for downlink, there is no similar issue, and if none of the parameter dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 is configured, it should follow the behavior for DCI format 1_0.

Proposal 3-3: Adopt the following text proposal with changes marked in red for PTRS reception procedures with DCI format 1_2:

#### 5.1.6.2 DM-RS reception procedure ## <Unchanged text is omitted> If a UE receiving PDSCH scheduled by DCI format 1_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 or a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB DMRS-DownlinkConfig, the UE may assume that the following configurations are not occurring simultaneously for the received PDSCH: - any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and the other UE(s) sharing the DM-RS REs on the same CDM group(s), and - PT-RS is transmitted to the UE. ## <Unchanged text is omitted> #### 5.1.6.3 PT-RS reception procedure The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 1_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 and to a UE receiving PDSCH scheduled by DCI format 1_0 or format 1_1 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB. A UE shall report the preferred MCS and bandwidth thresholds based on the UE capability at a given carrier frequency, for each subcarrier spacing applicable to data channel at this carrier frequency, assuming the MCS table with the maximum Modulation Order as it reported to support. ## <Unchanged text is omitted>

Please provide your views on the above proposal 3-3.

| Company | View |
| --- | --- |

- Issue A-6: Remove CBG PUSCH additions to DCI format 0_1 (38.212, Sec. 7.3.1.1.2)

R1- 2003577 (Nokia), MTK (R1-2003684), CATT (R1-2003620) and R1-2004331 (Sharp) proposes to remove the addition of CBG based PUSCH additions to DCI format 0_1 in Sec. 7.3.1.1.2 of TS 38.212 by agreeing to the following TP with changes in red:

TP to TS 38.212, 7.3.1.1.2: Removal of CBGTI size alignment clause for DCI format 0_1 ##### 7.3.1.1.2 Format 0_1 ## <Unchanged text is omitted> - CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize. - CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PUSCH is not configured or if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1; otherwise, 2, 4, 6, or 8 bits determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock for PUSCH. [When two HARQ-ACK codebooks are configured for the same serving cell, if the bit width of the CBG transmission information in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the CBG transmission information in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG transmission information until the bit width of the CBG transmission information in DCI format 0_1 for the two HARQ-ACK codebooks are the same.] - PTRS-DMRS association – number of bits determined as follows ## <Unchanged text is omitted>

The reasons given in R1-2001694 are as below:

- The CBG PUSCH operation has nothing to do with the operation of two PDSCH HARQ-ACK codebooks.

- RAN1 did not agree to support separate CBG operation for PUSCH with respect to different PUSCH priorities (which somehow would be corresponding to different HARQ-ACK codebooks for PDSCH).

- It does no really make sense to configure separately, as the DCI overhead would be given by the larger size of the CBGTI field for DCI format 0_1 – therefore, no advantage of allowing such separate configuration is identified which is very much in contrast to CBG of PDSCH for the two HARQ-Ack codebooks resulting in different codebook sizes.

Feature lead view: Depending on the outcome of issue A-3, it will be clear whether the TP is needed or not. If proposal 3-2 is agreed under issue A-3, then the TP here will be adopted accordingly.

- Issue A-7: PUCCH resource determination for reduced size of PRI field

Spreadtrum R1-2002255 PRI field in DCI format 1_2 can be configured as 0/1/2/3 bits. - PUCCH resource indicator – 0 or 1 or 2 or 3 bits determined by higher layer parameter Numberofbits-forPUCCHresourceindicator-ForDCIFormat1_2 One open issue is how to determine the PUCCH resource when it is without PRI. This issue raised by many contributions and two options can be for further study. Option 1: defined as the first PUCCH resource configured in the set Option 2: using a first CCE index to determine the PUCCH resource Clearly, Option 1 is the easier solution and also used for bit width of PRI field is 1 or 2 bits, as well as other fields in DCI format 0_2/1_2 which bit widths are less than corresponding fields in DCI 0_1/1_1. - PUCCH resources corresponding to a PUCCH resource allocation field with 0 bits are the first configured PUCCH resources.

MTK (R1-2003684) has similar view.

Feature lead view: The issue is valid, however it was discussed under UCI enhancements in RAN1#100b-e, thus it can be continued to discuss there.

- Issue A-8: Missing case of PUSCH release for search space sharing

Sharp (R1-2004331) proposes to adopt the following TP2 for section 10.1 in TS 38.213 to compensate for a missing case of PUSCH release for search space sharing.

TP2 TS 38.213 V16.1.0 (2020-03) - 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > A UE that - is configured for operation with carrier aggregation, and - indicates support of search space sharing through searchSpaceSharingCA-UL or through searchSpaceSharingCA-DL, and - has a PDCCH candidate with CCE aggregation level in CORESET for a first DCI format scheduling PUSCH transmission or releasing PUSCH transmission, other than DCI format 0_0, or for a second DCI format scheduling PDSCH reception or SPS PDSCH release, other than DCI format 1_0, having a first size and associated with serving cell , can receive a corresponding PDCCH through a PDCCH candidate with CCE aggregation level in CORESET for a first DCI format or for a second DCI format, respectively, having a second size and associated with serving cell if the first size and the second size are same. < Unchanged parts are omitted >

Feature lead view: The issue is valid. As to the TP, some companies commented that it might have impact on the Rel-15 behavior, e.g. it may give the impression that in Rel-15 DCI format 0_1 can be used for release of type 2 PUSCH. However, if it is an issue then it seems the issue exist for SPS PDSCH release also. Probably ok to have simple correction here since anyway there is other sections in the spec which define the corresponding DCI format (s) for PUSCH release and SPS release.

Proposal 3-4: Adopt the following text proposal in section 10.1 in TS 38.213, to capture the missing case of PUSCH release for search space sharing:

< Unchanged parts are omitted > - 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > A UE that - is configured for operation with carrier aggregation, and - indicates support of search space sharing through searchSpaceSharingCA-UL or through searchSpaceSharingCA-DL, and - has a PDCCH candidate with CCE aggregation level in CORESET for a first DCI format scheduling PUSCH transmission or releasing PUSCH transmission, other than DCI format 0_0, or for a second DCI format scheduling PDSCH reception or SPS PDSCH release, other than DCI format 1_0, having a first size and associated with serving cell , can receive a corresponding PDCCH through a PDCCH candidate with CCE aggregation level in CORESET for a first DCI format or for a second DCI format, respectively, having a second size and associated with serving cell if the first size and the second size are same. < Unchanged parts are omitted >

Please provide your views on the above proposal 3-4.

| Company | View |
| --- | --- |

- Issue A-9: Correction on Transmission configuration indication in DCI format 1_2

ASUSTeK (R1-2003702) proposes to adopt the following TP to enable same size of TCI bitfield as current CORESET for all CORESETs in target BWP for a BWP switching DCI similar as in Rel-15.

Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI-ForDCIFormat1_2 is not configured; otherwise 1 or 2 or 3 bits determined by higher layer parameter tci-PresentInDCI-ForDCIFormat1_2 as defined in Subclause 5.1.5 of [6, TS38.214]. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part, - if the higher layer parameter tci-PresentInDCI-ForDCIFormat1_2 is not configured for the CORESET used for the PDCCH carrying the DCI format 1_2, - the UE assumes tci-PresentInDCI-ForDCIFormat1_2 is not configured for all CORESETs in the indicated bandwidth part; - otherwise, - the UE assumes tci-PresentInDCI-ForDCIFormat1_2 for all CORESETs in the indicated bandwidth part is configured and with same value as the higher layer parameter tci-PresentInDCI-ForDCIFormat1_2 for the CORESET used for the PDCCH carrying the DCI format 1_2.

Feature lead view: The issue looks valid. However, during the preparation phase in RAN1#100b-e, some companies commented that the change is not needed. More views are needed. Can be discussed if time permits.

Question A-9-1: Is it necessary to discuss this issue? If yes any comment on the above text proposal?

Please provide your views and the corresponding reasons on the above question A-9-1.

| Company | View |
| --- | --- |

- Issue A-10: Whether to use RBG as the granularity of the starting position and hopping offset for PUSCH scheduled by DCI format 0_2 with resource allocation type 1

In TS 38.214, the formula for frequency hopping for PUSCH is defined as below:

… In case of intra-slot frequency hopping, the starting RB in each hop is given by: , … In case of inter-slot frequency hopping, the starting RB during slot is given by: … In case of inter-repetition frequency hopping, the starting RB for an actual repetition within the n-th nominal repetition (as defined in Clause 6.1.2.1) is given by: , …

Huawei (R1-2003225) proposes to correct the granularity of the starting position and hopping offset to RBG for PUSCH scheduled by DCI format 0_2 with resource allocation type 1, to maintain the benefit of aligning with the common RB grid.

Huawei R1-2003525 In RAN1#100e, it was agreed that inter-repetition FH and inter-slot FH are supported for PUSCH repetition type B. In Rel-15, as shown in section 6.3 of TS 38.214, the granularity of the starting position in each hop and the hopping offset is 1 RB. The same formula is reused for PUSCH scheduled by DCI format 0_2 with resource allocation type 1 as shown in section 6.3.2 and 6.3.1 in TS 38.214, however the indicated starting RBG and number of RBGs should be transferred to starting RB and the number of RBs, which seems not to be reflected in the current specification. Also, there could be a case that the second hop would not occupy RBGs fully, which would eliminate the benefit to align the RBG grid for resource allocation type 1 as defined in section 6.1.2.2.2. For example, assuming for a 12 RBs BWP, the configured RBG size is 4 RBs, then there would be 3 RBGs in this BWP, also assuming indicated RBGstart=0 and LRBG=1, and the indicated hopping offset is 6 RBs, then frequency resource before and after hopping could be as shown in figure 1 (a) and (b) separately. We can see that the second hop is not aligned with the RBG grid. From figure (b), we can see that the alignment with the common RB grid is not maintained. Figure 1 - Frequency resource before and after FH To avoid this issue, one simple way is to use RBG as the granularity for the starting position and the hopping offset configuration. For the same example above, assuming the indicated hopping offset is 2 RBG, then frequency resource after hopping could be shown in Figure 1 (c). It is seen that the second hop is aligned with the RBG grid. If we use RBG as the granularity for the starting position and the hopping offset configuration, the hopping formulas in different cases could be seen in the following, where the NRBG is number of the resource block groups defined as in 5.1.2.2.1, and the number of RBGs of each hop could be equal to LRBG as given in section 6.1.2.2.2. - In case of intra-slot frequency hopping, the starting RBG in each hop is given by: RBGstart =RBGstart i=0RBGstart +RBGoffsetmod NRBG i=1 - In case of inter-slot frequency hopping, the starting RBG during slot i is given by: RBGstart =RBGstart i mod 2=0RBGstart +RBGoffsetmod NRBG i mod 2=1 - In case of inter-repetition frequency hopping, the starting RBG in i-th repetition is given by: RBGstart =RBGstart i mod 2=0RBGstart +RBGoffsetmod NRBG i mod 2=1 Proposal 5: Use RBG as the granularity of the starting position and hopping offset for PUSCH scheduled by DCI format 0_2 with resource allocation type 1.

Feature lead view: The issue looks valid and some discussion/clarification is needed. If time permits, we can discuss.

Question A-10-1: What’s your views on using RBG as the granularity of the starting position and hopping offset for PUSCH scheduled by DCI format 0_2 with resource allocation type 1?

Please provide your views and the corresponding reasons on the above question A-10-1.

| Company | View |
| --- | --- |

- Issue A-11: Text proposals to align the RRC parameter names between TS 38.331 and TS 38.212

CATT (R1-2003620) provides a set of TPs to align the RRC parameter names between TS 38.331 and TS 38.212, which a valid issue. However, it is some editorial changes. We can leave it to editors to make the change.

- Issue A-12: Correction on bandwidth part operation

In the RAN1#98bis meeting [1], the “bandwidth part indicator field” has been agreed to support BWP switching with DCI formats 0_2 and 1_2. The same RRC parameters are used for both DCI format 0_1/1_1 and DCI format 0_2/1_2. However, the corresponding specification for DCI format 0_2/1_2 are missing in TS 38.213. Huawei (R1-2003525) proposed to adopt the following TP for it.

- 12 Bandwidth part operation *** Unchanged text is omitted *** If a bandwidth part indicator field is configured in DCI format 1_1 or DCI format 1_2, the bandwidth part indicator field value indicates the active DL BWP, from the configured DL BWP set, for DL receptions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in DCI format 0_1 or DCI format 0_2, the bandwidth part indicator field value indicates the active UL BWP, from the configured UL BWP set, for UL transmissions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in DCI format 0_1/0_2 or DCI format 1_1/1_2 and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP, respectively, the UE shall - for each information field in the received DCI format 0_1 or DCI format 1_1 or DCI format 0_2 or DCI format 1_2 - if the size of the information field is smaller than the one required for the DCI format 0_1/0_2 or DCI format 1_1/1_2 interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, respectively, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format 0_1/0_2 or DCI format 1_1/1_2 information fields, respectively - if the size of the information field is larger than the one required for the DCI format 0_1/0_2 or DCI format 1_1/1_2 interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, respectively, the UE uses a number of least significant bits of DCI format 0_1/0_2 or DCI format 1_1/1_2 equal to the one required for the UL BWP or DL BWP indicated by bandwidth part indicator prior to interpreting the DCI format 0_1/0_2 or DCI format 1_1/1_2 information fields, respectively - set the active UL BWP or DL BWP to the UL BWP or DL BWP indicated by the bandwidth part indicator in the DCI format 0_1 or DCI format 1_1 or DCI format 0_2 or DCI format 1_2, respectively A UE does not expect to detect a DCI format 1_1/1_2 or a DCI format 0_1/0_2 indicating respectively an active DL BWP or an active UL BWP change with the corresponding time domain resource assignment field providing a slot offset value for a PDSCH reception or PUSCH transmission that is smaller than a delay required by the UE for an active DL BWP change or UL BWP change [10, TS 38.133]. If a UE detects a DCI format 1_1 or a DCI format 1_2 indicating an active DL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format 1_1 or a DCI format 1_2 in a scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format 1_1 or a DCI format 1_2. If a UE detects a DCI format 0_1 or a DCI format 0_2 indicating an active UL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format 0_1 or a DCI format 0_2 in the scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format 0_1 or a DCI format 0_2. A UE does not expect to detect a DCI format 1_1 or a DCI format 1_2 indicating an active DL BWP change or a DCI format 0_1 or a DCI format 0_2 indicating an active UL BWP change for a scheduled cell within FR1 (or FR2) in a slot other than the first slot of a set of slots for the DL SCS of the scheduling cell that overlaps with a time duration where the UE is not required to receive or transmit for an active BWP change in a different cell from the scheduled cell within FR1 (or FR2). A UE expects to detect a DCI format 0_1 or a DCI format 0_2 indicating active UL BWP change, or a DCI format 1_1 or a DCI format 1_2 indicating active DL BWP change, only if a corresponding PDCCH is received within the first 3 symbols of a slot. *** Unchanged text is omitted ***

Feature lead view: The issue is valid. If time permits, it can be discussed.

Proposal 3-5: Adopt the above text proposal to capture BWP switching with DCI format 0_2 and DCI format 1_2 in TS 38.213.

Please provide your views and the corresponding reasons on the above proposal 3-5.

| Company | View |
| --- | --- |

## Issue A-13: Type2 HARQ-ACK codebook construction related to DAI bit width

The C-DAI field in DCI format 1_1 is 2 bits if only one serving cell is configured in the DL, and in DCI format 1_2 is configured as 0, 1, or 2 bit. The C-DAI is used to find HARQ-ACK bit position in Type 2 HARQ-ACK codebook. If the first HARQ-ACK bit corresponding to a first PDSCH scheduled by DCI format 1_1 and the second HARQ-ACK bit corresponding to a second PDSCH scheduled by DCI format 1_2 are multiplexed into a same dynamic HARQ-ACK codebook, and bit length of C-DAI in DCI format 1_1 and in DCI format 1_2 is different, according to current pseudo-code in TS38. 213, the gNB cannot correctly find the HARQ-ACK bit position for the first HARQ-ACK bit and the second HARQ-ACK bit.

This issues was discussed in RAN1#100bis-e under UCI enhancements but not be able to conclude. Companies provide further views in the contributions this meeting, the candidate options and positions are summarized as below:

- Option 1: If the size of counter DAI between DCI format 1_2 and 1_0/1_1 is different, a UE is not expected to multiplex HARQ-ACK for PDSCH scheduled by DCI format 1_2 and HARQ-ACK for PDCSH scheduled by DCI format 1_0/1_1 in the same HARQ-ACK codebook.

  - UE does not expect to multiplex HARQ-ACK information scheduled by a DCI format without counter DAI field in a same HARQ-ACK codebook

  - Support: Vivo, Huawei, CATT, NEC, Spreadtrum, OPPO, DCM, LGE, Qualcomm, LGE, Samsung, ZTE

  - Reasons

    - Enable smaller DCI size for high priority scheduling.

    - gNB can avoid multiplexing PDSCH scheduled by different DCI formats with different DAI bit width to the same HARQ-ACK codebook

- Option 2: The size of DAI field is the maximum of the bit widths for the DCI formats.

  - Support: LGE

- Option 3: When a UE is configured with DCI format 1_0 and/or with DCI format 1_1 and DCI format 1_2 for scheduling same priority traffic, the bit-width of the DAI fields is same.

  - Support: Ericsson, ZTE

  - Reasons

    - The overall flexibility of Option 1 is questionable since the reliance on gNB scheduling is via scheduling restrictions.

    - No need to complicate DAI calculation and tracking unnecessarily complicated. There is no incentive for the NW to configure different sizes of DAI field to gain 1 bit DCI saving at the cost of increased complexity. So, the reasonable configuration would be to have the same DAI field bit-sizes for the DCIs contributing to the same CB

- Option 4: Modify the procedure of generation of codebook. 2-bit DAI and 1-bit DAI is transformed to standard DAI value and the standard DAI value is used to determine HARQ-ACK bit location. For example, 2-bit DAI value {1, 2, 3, and 4} is transformed to {0, 1, 0, and 1} to align 1-bit DAI value. In general way, standard DAI value = mod (2-bit DAI value or 1-bit DAI value -1, 2) +1

  - Support: OPPO, Nokia

  - Reasons

    - Unnecessarily restrictive for option 1, 3 and 5

- Option 5: For type-2 HARQ-ACK codebook, if the size of DAI field between DCI format 1_2 and 1_0/1_1 is different, and they can schedule HARQ transmission using the same PUCCH-Config then the minimum DAI bit width over DCI_1_1 and DCI_1_2 is to be taken (Option 4 in R1-2002784)

  - Support: MTK, WILUS

  - Reasons

    - More flexibility than mandating that the same DAI bitwidth be configured

Feature lead view: Since it was already discussed in RAN1#100b-e meeting, I assumed that people already understand well the pros and cons of each option, but it seems still not much difference with the positions. In this case, I would like to make the tentative proposal below to follow the majority view first, though I do agree that we can support it better with some modification in the spec.

Proposal 3-6: If the size of counter DAI between DCI format 1_2 and 1_0/1_1 is different, a UE is not expected to multiplex HARQ-ACK for PDSCH scheduled by DCI format 1_2 and HARQ-ACK for PDCSH scheduled by DCI format 1_0/1_1 in the same HARQ-ACK codebook.

- UE does not expect to multiplex HARQ-ACK information scheduled by a DCI format without counter DAI field in a same HARQ-ACK codebook

Please provide your views on the above proposal 3-6.

| Company | View |
| --- | --- |

In addition, WILUS (R1-2004523) also provide some views on how to handle 1-bit counter DAI and 2 bits UL DAI as below.

WILUS R1-2004523 2-bit total-DAI and 2-bit UL DAI There exists 2-bit UL DAI for dynamic HARQ-ACK codebook in DCI format 0_1 and 0_2 and similarly, there are 2-bit total DAI field for dynamic HARQ-ACK codebook in DCI format 1_1 and 1_2. The possible value range of the 2-bit total DAI and UL DAI can be from 1 to 4. The total DAI and UL DAI can be used to determine HARQ-ACK codebook size. For example, by definition of UL DAI, the value of UL DAI, denoted by VDAIUL, is determined by VDAIUL=(X-1) mod 4 + 1 , where X is the number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release is present. In other words, if a UE detects a DCI format with VDAIUL, then the possible HARQ-ACK codebook size will be 4*n+VDAIUL, where n is non-negative integer. Consider the case where a UE receives 3 DCI formats having 1-bit counter-DAI among 3 monitoring occasions as shown in figure 3. The counter-DAI value is 1, 2, and 1 in DCI formats detected in MO#0, MO#1, and MO#2, respectively. In this case, 2-bit UL DAI in DCI format scheduling PUSCH indicates the UL-DAI = 3. Figure 3. 2-bit UL DAI and 1-bit counter-DAI Based on the current pseudo-code for Type-2 HARQ-ACK codebook, the HARQ-ACK codebook size is determine by OACK=TD⋅j+Vtemp2. And in this example, TD=2, j=1, Vtemp2=3 so that OACK= 2*1+3=5, which results in incorrect HARQ-ACK codebook size. To address this issue, the HARQ-ACK codebook size should be OACK=TD⋅j+Vtemp2-1mod TD+1. The same way can be also applied to the 2-bit total DAI case. - Proposal 5: To support 2-bit UL DAI, 2-bit total-DAI and 1-bit counter-DAI, the HARQ-ACK codebook size should be determined by OACK=TD⋅j+Vtemp2-1mod TD+1.

Feature lead view: The issue is valid and we need to discuss and fix it. Since there is not much inputs on this, more views are needed before we can make any proposal here.

Please provide your views on how to solve the above issue.

| Company | View |
| --- | --- |

- Issue A-14: Miscellaneous issues

In addition, DOCOMO (R1-2004389) also proposed to remove a bracket in DCI format 0_2, however it was already fixed in the editor CR after RAN1#100bis-e meeting since it is some editoral update.

- MTK (R1-2003684): Correction to DMRS transmission procedure for PUSCH scheduled by DCI format 0_2

## MTK (R1-2003684) The issue was raised when the UE is configured to monitor DCI format 0_2 but dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 are not configured to the UE. In this case, the UE should follow the configuration of the fall-back DCI format 0_0 otherwise the non-fall-back DCI 0_1. - If the UE is not RRC configured with a DMRS configuration for DCI format 0_2, we are in favour of following the DMRS configuration of the DCI format 0_0 otherwise the DCI format 0_1.

Feature lead view: The issue was discussed in RAN1#100bis-e under eCG agenda, and conclusion was made there.

- DOCOMO (R1-2004389): Remove a bracket in DCI format 0_2

Feature lead view: It was already fixed in the editor CR after RAN1#100bis-e meeting since it is some editorial update.

- OPPO (R1-2004114): A TP is provided for DCI format 0_2/1_2 to make DAI cover the case with 1 bit total DAI, assuming 1 bit total DAI is supported.

Feature lead view: According to TS 38.212 and TS 38.331, it is clear that 1 bit or 2 bits configured by RRC is only applied to single carrier case, and for CA case the DAI is always 4 bits. So the issue doesn’t exist.

Enhanced PDCCH monitoring capability

This section summarize the issues on enhanced PDCCH monitoring capability.

Remaining issues on reporting enhanced PDCCH monitoring capability

- Issue B-1: Minimum value of (pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16) for case 3

In the RAN1#100bis-e meeting, the following agreement was made:

Agreements: For one reported combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for CA:  The minimum value of pdcch-BlindDetectionCA-R15 is 1 and the minimum value of pdcch-BlindDetectionCA-R16 is 1  [3]<=pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 <=16  Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15  Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15

Some companies provide views on the above FFS and the position is summarized as below:

Option 1: Update the agreement from RAN1#100b-e as below by removing the bracket.

Agreements: For one reported combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for CA:  The minimum value of pdcch-BlindDetectionCA-R15 is 1 and the minimum value of pdcch-BlindDetectionCA-R16 is 1  [3]<=pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 <=16  Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15  Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15

- Support: Huawei/HiSilicon, Ericsson, Intel, NTT DOCOMO

- Reasons:

  - A UE supporting 4 Rel-15 CCs should be able to support 1 Rel-16 CC and 2 Rel-15 CC.

  - The BD/CCE limits are gated by the span durations and span gaps, implying that the peak loading from Rel-16 monitoring can be managed much better than that for Rel-15 monitoring wherein all the BD/CCEs may be located in a single span in a slot.

Feature lead view: The reasons given by the proponents of option 1 look reasonable.

Proposal 4.1-1: Update the agreement below by removing the bracket:

Agreements: For one reported combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for CA:  The minimum value of pdcch-BlindDetectionCA-R15 is 1 and the minimum value of pdcch-BlindDetectionCA-R16 is 1  [3]<=pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 <=16  Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15  Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15

Please provide your views and your reasons on the above proposal 4-1.

| Company | View |
| --- | --- |

- Issue B-2: Whether to adopt nested reporting structure for reporting the supported combination (X, Y) for Rel-16 PDCCH monitoring capability

Ericsson R1-2003439 In Rel-15, UE reports combination (X,Y) in form of nested set of combinations, i.e., {(7,3)}, {(4,3), (7,3)}, and {(2,2), (4,3), (7,3)}. However, for Rel-16 PDCCH monitoring capability, a UE can report combination (X,Y) separately, i.e., any subset of a set {(2,2), (4,3), (7,3)} is possible. This leads to a concern that the Rel-16 PDCCH monitoring capability can be lower than that of Rel-15. This is especially true since the agreed values for the per-CC limits for the maximum number of PDCCH candidates and non-overlapped CCEs for each combination (X,Y) are much smaller compared to the Rel-15 slot limits. For example, in Figure 2 below, for the Rel-16 scenario where the UE is configured with PDCCH monitoring occasions shown by the blue boxes in every slot, if UE reports only (2,2) according to the red text, the BD limit per span will be according to M(2,2) = 14 resulting in a total of 28 maximum PDCCH candidates monitored per slot. This is clearly much worse than the limit per slot in Rel-15 (M=44). Figure 2 Example of Rel-16 PDCCH blind decode capability being effectively worse than Rel-15 slot limit. The outcome illustrated in the above example does not satisfy the highlighted note agreed in RAN1#99 which states that the total number of monitored PDCCH candidates for Rel-16 monitoring capability is not smaller than the limit per slot in Rel-15. Although above we only described the issue regarding the maximum number of monitored PDCCH candidates or M values, the same applies to the maximum numbers of non-overlapped CCEs or C values as well. To prevent this from happening, we propose that for Rel-16 PDCCH monitoring capability, the UE only uses the nested reporting for combination (X,Y): {(2,2),(4,3),(7,3)} {(4,3),(7,3)} {(7,3)}. For the example in Figure 2, with the nested reporting of {(2,2),(4,3),(7,3)}, the limit per span would corresponds to the largest M(X,Y) possible with all the reported (X,Y) values which in this case is M(7,3) = 44. For Rel-16 PDCCH monitoring capability, UE reports combination (X,Y) using the nested reporting, i.e., the reporting candidates are {(2,2),(4,3),(7,3)}, {(4,3),(7,3)}, and {(7,3)}.

Feature lead view: The issue is valid. In RAN1#99 meeting, we agreed to remove the nested reporting structure, but there was no sufficient time to identify if any problem. Therefore, the following tentative proposal is made for further discussion, though whether to take it or not would depend on more views from companies.

Proposal 4.1-2: For the supported combination (X, Y) reported as UE capability, the candidate value sets are {(2,2),(4,3),(7,3)}, {(4,3),(7,3)} and {(7,3)}.

Please provide your views and your reasons on the above proposal 4-1.

| Company | View |
| --- | --- |

- Issue B-3: UE behavior if the obtained span monitoring configuration is invalid

MTK R1-2003684 After the UE has reported one or multiple (X, Y) span combinations, it is then configured by the gNB with a PDCCH configuration. If this configuration doesn’t comply with any of the reported (X,Y) combinations reported by the UE, then the UE behaviour in this case should be clarified. One possible option is to drop all the PDCCH monitoring configurations and consider it as an error case. The UE is not expected to be configured with a PDCCH monitoring configuration leading to invalid span monitoring configuration. The UE may fall-back to the monitoring configuration associated with CORESET 0

Feature lead view: An appropriate configuration should be able to avoid this kind of error case. It seems not really necessary to specify the behavior in the spec. However, if significant issue is identified, then we can further discuss.

Remaining issues on scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability

- Issue C-1: Corrections on span duration

The following text has been captured in section 10 of TS38.213 to define a combination (X, Y).

A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. A last span in a slot can have a shorter duration than other spans in the slot.

During the email discussion in RAN1#100b-e, it was observed that some further clarification are needed for the span duration in TS 38.213.

Question C-1-1: With the formula dspan=maxdCORESET,max,Ymin defined in TS 38.213, whether to allow the span duration larger than Y for a combination (X, Y).

Based on the discussion in RAN1#100b-e, it is common understanding that the span duration is not allowed to be larger than Y for a combination (X, Y), and it was observed that some further clarification is needed to reflect this in the spec. Samsung (R1-2003865), ZTE (R1-2003317), Huawei (R1-2003525) and Quectel (R1-2003942) provide some text proposal accordingly.

Text proposal 1 (R1-2003865):

If a UE monitors PDCCH on a cell according to combination X,Y, the duration of a span is Y dspan=maxdCORESET,max,YmindCORESET,maxYminYX,Y. A last span in a slot can have a shorter duration than other spans in the slot.

Feature lead view: As shown in R1-2003865, the assumption of the above TP is that UE will determine the combination (X, Y) for PDCCH monitoring first. Firstly, if we go this way, we may need some further clarification in the spec to define the order of determination of span duration and determination of combination (X, Y). Note that in the current TS 38.213, determination of combination (X, Y) is in the later section 10.1 than the definition of span duration in section 10. Secondly, according to the agreements we made, the logic is to determine the valid combinations (X, Y), and then pick one from all the valid combinations (X, Y).  Thirdly, in order to determine the separation of two consecutive spans, UE needs to know the span duration first.

Text proposal 2 (R1-2003317):

10 UE procedure for receiving control information <---------------------------Other parts are omitted -------------------------------> A UE reports one or more combinations of number of symbols, where , for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span is and is of length up to consecutive OFDM symbols, where is a maximum duration among durations of CORESETs that are configured to the UE and is a minimum value of in the combinations of that are reported by the UE. A last span in a slot can have a shorter duration than other spans in the slot. <---------------------------Other parts are omitted ------------------------------->

Feature lead view: When UE reports more than one combination (X, Y), it is not clear which Y to be used here.

Text proposal 3 (R1-2003525):

******************************** Start of TP **************************** 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot on a serving cell in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span other than the last span in a slot is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. The duration of a last span in a slot is mindspan,dboudary, where dboudary is the time separation between the start of the span and the end of the slot in number of symbols. When a UE reports in pdcch-MonitoringAnyOccasionsWithSpanGap combinations (X, Y) corresponding to value set 3 and is configured a CORESET with duration of 3 OFDM symbols, the UE is not expected to monitor PDCCH according to combination (2, 2). < Unchanged parts are omitted > ************************************* End of TP *********************************

Feature lead view: Value set 3 is not defined in the spec either. In addition, it cannot preclude the case of the configuration as shown in CC3 in the figure below.

Figure C-1-1 Whether three cells are aligned case for (2,2) or not

Text proposal 4 (R1-2003525):

*** Unchanged text is omitted *** A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. A last span in a slot can have a shorter duration than other spans in the slot. When dspan is equal to 3, a UE is not expected to be configured with PDCCH monitoring occasions resulting into a separation of the first symbol of two consecutive spans that is smaller than 4. *** Unchanged text is omitted ***

Feature lead view: The only case that may have misunderstanding is when UE reports the support of combination (2, 2) together with combination (4, 3) and/or combination (7, 3). In this case, if gNB decides to configure CORESET(s) with 3-symbol duration, then gNB needs to ensure that the separation X should not be smaller than 4. It can preclude the configuration similar as what for CC3 in the figure C-1-1. It seems text proposal 4 is more complete.

Proposal 4.2-1: Adopt the following text proposal for section 10 in TS 38.213:

*** Unchanged text is omitted *** A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. A last span in a slot can have a shorter duration than other spans in the slot. When dspan is equal to 3, a UE is not expected to be configured with PDCCH monitoring occasions resulting into a separation of the first symbol of two consecutive spans that is smaller than 4. *** Unchanged text is omitted ***

Please provide your views and your reasons on the above proposal 4.2-1.

| Company | View |
| --- | --- |

Question C-1-2: Whether to keep “A last span in a slot can have a shorter duration than other spans in the slot. ”, and if yes whether any further clarification needed.

Samsung R1-2003865 Last span in a slot can have shorter duration Another issue is the following statement that was inherited from FG 3-5b in Rel-15. A last span in a slot can have a shorter duration than other spans in the slot.. The above may be beneficial for example as it allows for X,Y=(4, 3) PDCCH monitoring to start at the beginning of a slot and have Y=1 at the 13th symbol of the slot. However, in that case, the following statement is not applicable for the “including across slots”. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots.. Proposal 4: RAN1 to clarify when and how both following statements are applicable. If no such case, remove the second statement from the specifications as otherwise the UE behavior is ambiguous. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots.. and A last span in a slot can have a shorter duration than other spans in the slot..

Quectel R1-2003942 Calculations of the duration of a last span in a slot should be specifically captured in the specification as well. Accordingly, the proposed text changes are as follows: ******************************** Start of TP **************************** 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot on a serving cell in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span other than the last span in a slot is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. The duration of a last span in a slot is mindspan,dboudary, where dboudary is the time separation between the start of the span and the end of the slot in number of symbols. When a UE reports in pdcch-MonitoringAnyOccasionsWithSpanGap combinations (X, Y) corresponding to value set 3 and is configured a CORESET with duration of 3 OFDM symbols, the UE is not expected to monitor PDCCH according to combination (2, 2). < Unchanged parts are omitted > ************************************* End of TP *********************************

Feature lead view: The text “A last span in a slot can have a shorter duration than other spans in the slot. ” can be kept, even consider the separation for cross slots, it can be valid when the first span in the slot doesn’t start at the beginning of the slot. The text proposal from R1-2003942 can make the specification clearer.

Proposal 4.2-2: Adopt the following text proposal for section 10 in TS 38.213:

******************************** Start of TP **************************** 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > A UE reports one or more combinations of X,Y number of symbols, where X≥Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. The duration of a span is dspan=maxdCORESET,max,Ymin, where dCORESET,max is a maximum duration among durations of CORESETs that are configured to the UE and Ymin is a minimum value of Y in the combinations of X,Y that are reported by the UE. The duration of a last span in a slot is mindspan,dboudary, where dboudary is the time separation between the start of the span and the end of the slot in number of symbols. < Unchanged parts are omitted > ************************************* End of TP *********************************

Please provide your views and your reasons on the above proposal 4.2-1.

| Company | View |
| --- | --- |

- Issue C-2: Corrections on “aligned spans” case

The following text has been captured in section 10.1 of TS38.213 for scaling PDCCH monitoring capability.

If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y, - TBD, otherwise where Ncells,r16DL,j is a number of configured cells using Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with downlink cells using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

Regarding the text for “aligned spans” case, the following updates were proposed by companies:

Proposed update #1 :

Ericsson R1-2003439 However, we found that the term “per span” used in the above TP is not clear. A span is defined for a cell only, not for multiple cells. For the CA case, there are multiple cells, and the total limits MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ should be applied to PDCCH monitoring occasions in multiple spans across the DL cells. Moreover, if a UE reports different sets of (X,Y) value(s) for each cell from the Ncells,r16DL,X,Y,μ Ncells,r16DL,X,Y,μ downlink cells, the span duration in different cells may not even be the same. This is illustrated in Figure 3 where for CC2, when max CORESET duration is not more than 2, the span duration according to the span definition in the specification is equal to 2. However, according to the specification, CC2 is considered as monitoring with the limit corresponding to (7,3) and therefore grouped together with CC1 for the purpose of CA limit scaling. Span with (7,3) Span with (7,3) Span with (7,3) Span with (7,3) Figure 3 Example of different DL cells from the Ncells,r16DL,X,Y,μ Ncells,r16DL,X,Y,μ downlink cells having different span durations. A UE repots (7,3) for CC1 and (2,2),(4,3),(7,3) for CC2. For each CC, spans pattern are determined according to the span definition in the specification where span duration = max(max CORESET duration, Ymin). Hence, the term “per span” should be changed or at least clarified. It is more appropriate, e.g., to refer instead to a “resulting span” derived from the union of PDCCH monitoring occasions which then includes all PDCCH monitoring occasions in the “aligned” spans across the DL cells. It is reasonable to base the definition of “aligned span” on PDCCH monitoring occasions on all the cells from the Ncells,r16DL,X,Y,μ Ncells,r16DL,X,Y,μ downlink cells. Span is defined only for a cell. It is not clear to use the term “per span” for the total limits MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ since they are applied to PDCCH monitoring occasions in multiple spans across the DL cells. Revise the TP above by clarifying that the span in “per span” refers to the resulting span derived from the union of PDCCH monitoring occasions on all scheduling cells.

From feature view: the above change is not needed. Following the definition of span pattern, combination (7, 3) should be used for CC2 shown in the above example, i.e. span duration is 3 not 2. In addition, according to the current specification, only the cells using the same combination (X, Y) will be grouped together, that is the span duration is aligned among all the serving cells. Therefore, similar as Rel-15, it is ok to use the terminology “per span” in this case.

Proposed update #2:

Intel R1-2003737 The above definition may cause ambiguity in identification of certain span combinations across cells as aligned or non-aligned. For instance, when UE is configured with 1-symbol CORESET(s), then the above text still satisfies the condition defined for “union of PDCCH monitoring occasions” for the example in Figure 1, while this case should be considered as unaligned. Figure 1: An example case of unaligned span combinations across CC1 and CC2 that may be identified as aligned spans per current definition of “aligned spans”. To address this, the following alternative characterization that was also discussed during RAN1 #100b-E meeting is proposed: < unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0> per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if any pair of spans on the active DL BWP(s) of any two scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells X,Y are within a same set of up to Y consecutive symbols, or have their first symbols separated by at least X symbols, < unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0>.

- Support: Intel, Apple

From feature view: It seems there is some issue at least when search space sets are configured with 1-symbol CORESET(s). The TP from Intel can work. More views are needed before making any proposal here.

Question C-2-1: Do you agree with the text proposal below to correct the definition of “aligned spans” case?

< unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0> per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if any pair of spans on the active DL BWP(s) of any two scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells X,Y are within a same set of up to Y consecutive symbols, or have their first symbols separated by at least X symbols, < unchanged parts omitted, TS 38.213, Subclause 10.1, v16.2.0>

Please provide your views and your reasons on the above text proposal for “aligned spans” case.

| Company | View |
| --- | --- |

- Issue C-3: Corrections on “unaligned spans” case

The following text has been captured in section 10.1 of TS38.213 for scaling PDCCH monitoring capability.

If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y, - TBD, otherwise where Ncells,r16DL,j is a number of configured cells using Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with downlink cells using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

As shown in the above text, the text for “unaligned spans” case is still TBD. Some companies provide views on this.

Option 1: Rel-15 slot-based PDCCH monitoring is applied for “unaligned spans”, i.e. if a configuration of search space sets in one or more cells would result to “unaligned” spans, the UE expects to be configured for Rel-15 PDCCH monitoring on those cells

- Support: Samsung

- Reasons

  - “Unaligned spans” is not a typical configuration

  - Maintain optional simplicity

- Cons

  - Revert the agreements

If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE - expects that the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,X,Y,μ downlink cells results to PDCCH monitoring according to the combination X,Y, and - is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cellsNcells,r16DL,(X,Y),μX,Y where Ncells,r16DL,j is a number of configured cells using Rel-16 PDCCH monitoring capability with SCS configuration j. If a UE is configured with downlink cells using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

Option 2: Adopt the following text proposal for section 10.1 in TS 38.213:

< Unchanged parts omitted, TS 38.213, Subclause 10.1, Editor’s CR with changes accepted> If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y, - for the sum of the number PDCCH candidates and corresponding number of non-overlapped CCEs across any set of spans on the active DL BWP(s) of different scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, with at most one span per scheduling cell for each set of spans, otherwise < Unchanged parts omitted, TS 38.213, Subclause 10.1, Editor’s CR with changes accepted>

- Support: Intel

Option 3 (original proposal in RAN1#100b-e): Adopt the following text proposal for section 10.1 in TS 38.213:

< Unchanged parts omitted, TS 38.213, Subclause 10.1, Editor’s CR with changes accepted> If a UE is configured only with Ncells,r16DL,μ downlink cells using Rel-16 PDCCH monitoring capability, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, and having active DL BWPs using SCS configuration μ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs - per span on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y, - per set of spans across the active DL BWP(s) of all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells, with at most one span per scheduling cell for each set of spans, otherwise < Unchanged parts omitted, TS 38.213, Subclause 10.1, Editor’s CR with changes accepted>

- Support:

From feature view: It seems we need more discussion on this issue. It would be good for us not to revert the agreement if possible. As to the wording, it looks to me that the original proposal in RAN1#100b-e is the best, but more views are welcome.

Question C-3-1: Which option do you prefer for defining “unaligned spans” case?

Please provide your views and your reasons on the above question C-3-1 for “unaligned spans” case. Any better idea are welcome also.

| Company | View |
| --- | --- |

- Issue C-4: Enhanced PDCCH monitoring capability for cross-carrier scheduling

Huawei (R1-2003525), Intel (R1-2001998) and Quectel (R1-2003942) discusses Rel-16 PDCCH monitoring capability for cross-carrier scheduling case. Two alternatives were discussed in the contributions:

- Option 1: Both the scheduling cell and scheduled cell for cross-carrier scheduling are restricted to be configured the same PDCCH monitoring capability (i.e., Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability) , and provided the same combination (X, Y) when Rel-16 PDCCH monitoring capability is configured,

  - Support: Intel, Huawei/HiSilicon, Spreadtrum,

  - Reasons:

    - To simplify the PDCCH BD/CCE dimensioning and overall operation considering typical use cases

- Option 2: There is no restriction on the PDCCH monitoring capabilities for scheduling cell and scheduled cell, but the calculations of MPDCCHtotal,(X,Y),μand CPDCCHtotal,(X,Y),μare based on the number of scheduled cells whose scheduling cells are configured with the same PDCCH monitoring capability and provided the same combination (X, Y) when Rel-16 PDCCH monitoring capability is configured.

  - Support: Quectel

  - Reasons:

    - Does not prohibit the use of cross-carrier scheduling with different PDCCH monitoring capabilities or with different combinations and maintains the scheduling flexibility with limited specification efforts

From feature view: It looks option 1 is a better choice for simplicity. However, more views are needed before making any proposal here.

Please provide your preference between option 1 and option 2 for enhanced PDCCH monitoring capability for cross-carrier scheduling.

| Company | View |
| --- | --- |

- Issue C-5: Whether/how to extend Rel-16 PDCCH monitoring capability to multi-TRP case

Some companies provide views on this issue, and there are two related questions as below:

Question C-5-1: Whether to apply multi-DCI based multiple TRP on the serving cell using Rel-16 PDCCH monitoring capability

Samsung (R1-2002131), Ericsson (R1-2003439) and Apple (R1-2004221) propose to extend the R16 PDCCH capability to multi-TRP case, while Quectel (R1-2003942) proposes that multiple DCIs scheduling multiple PDSCHs and span based PDCCH monitoring are not supported simultaneously in a serving cell.

Option 1: M-DCI Multi-TRP based on CA framework is extended to Rel-16 PDCCH monitoring capability

  - Support: Samsung, Ericsson, Apple, ZTE

Option 2: No need for combining of span-based PDCCH monitoring capability and multi-DCI based multi-TRP

  - Support: MTK, Quectel

  - Reasons:

    - Motivation is not clear. Multi-DCI based multi-TRP is aimed for throughput, i.e., for eMBB. In multi-TRP operation, all enhancements for reliability (URLLC) are through single-DCI based operations (assuming ideal backhaul), which does not require any modification of R15 spec on monitoring capability.

    - Better to discuss in the eMIMO WI.

From feature view: More views are needed before making any proposal, including whether it is appropriate to discuss in eURLLC WI.

Companies are encouraged to provide your views.

| Company | View |
| --- | --- |

Question C-5-2: Whether to apply multi-DCI based multiple TRP on the serving cell using Rel-15 PDCCH monitoring capability

In addition, Quectel also proposed that whether multiple DCIs scheduling multiple PDSCHs is applied for a serving cell using Rel-15 PDCCH monitoring capability is up to UE capability and network configuration, and a corresponding TP is provided.

***************************** Start of TP *********************************** 10 UE procedure for receiving control information < Unchanged parts are omitted > If a UE can support - a first set of Ncells,0DL serving cells with scheduling cell(s) where the UE is either not provided CORESETPoolIndex or is provided CORESETPoolIndex with a single value for all CORESETs on all DL BWPs of each scheduling of the first set of serving cells, and - a second set of Ncells,1DL serving cells with scheduling cell(s) where the UE is provided CORESETPoolIndex with a value 0 for a first CORESET and with a value 1 for a second CORESET on any DL BWP of each scheduling cell of the second set of serving cells the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is either a value reported by the UE or R=TBD if the UE does not report a value of R. If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than 4 serving cells and the UE is not provided PDCCHMonitoringCapabilityConfig for any downlink cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all downlink cells where the UE monitors PDCCH, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and for a maximum number of non-overlapped CCEs the UE can monitor per slot when the UE is configured for carrier aggregation operation over more than 4 cells. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to downlink cells, where - is Ncells,0DL+R⋅Ncells,1DL if the UE does not provide pdcch-BlindDetectionCA where Ncells,0DL+Ncells,1DL is the number of configured downlink serving cells - otherwise, is the value of pdcch-BlindDetectionCA < Unchanged parts are omitted > If a UE indicates in UE-NR-Capability-r16 a carrier aggregation capability larger than one downlink cell with PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring or larger than one downlink cell with PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring, the UE includes in UE-NR-Capability-r16 an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs the UE can monitor for downlink cells with PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability or for downlink cells with PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability when the UE is configured for carrier aggregation operation over more than two downlink cells with at least one downlink cell with PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring and at least one downlink cell with PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring . When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot or per span that corresponds to Ncells,r15cap-r16 downlink cells or to Ncells,r16cap-r16 downlink cells, respectively, where - Ncells,r15cap-r16 is Ncells,r15,0DL+R⋅Ncells,r15,1DL if the UE does not provide pdcch-BlindDetectionCA-r15 where Ncells,r15,0DL+Ncells,r15,1DL is the number of configured downlink serving cells with scheduling cell(s) using PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability - otherwise, - if the UE reports only one combination of (pdcch-BlindDetectionCA-r15, pdcch-BlindDetectionCA-r16), Ncells,r15cap-r16 is the value of pdcch-BlindDetectionCA-r15 - else, Ncells,r15cap-r16 is the value of pdcch-BlindDetectionCA-r15 from a combination of (pdcch-BlindDetectionCA-r15, pdcch-BlindDetectionCA-r16) that is provided by pdcch-BlindDetectionCAComb-indicator and - Ncells,r16cap-r16 is the number of configured downlink cells with scheduling cell(s) using PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability if the UE does not provide pdcch-BlindDetectionCA-r16 - otherwise, - if the UE reports only one combination of (pdcch-BlindDetectionCA-r15, pdcch-BlindDetectionCA-r16), Ncells,r16cap-r16 is the value of pdcch-BlindDetectionCA-r16 - else, Ncells,r16cap-r16 is the value of pdcch-BlindDetectionCA-r16 from a combination of (pdcch-BlindDetectionCA-r15, pdcch-BlindDetectionCA-r16) that is provided by pdcch-BlindDetectionCAComb-indicator the UE determines, for the purpose of reporting pdcch-BlindDetectionCA-r15, a number of serving cells as Ncells,r15,0DL+R⋅Ncells,r15,1DL. < Unchanged parts are omitted > ***************************** End of TP *******************************************

From feature view: For the cell using Rel-15 PDCCH monitoring capability, multi-DCI for multi-TRP can be applied.

Companies are encouraged to provide your views on the above TP from R1-2003942.

| Company | View |
| --- | --- |

- Issue C-6: Text proposal for Rel-16 PDCCH monitoring capability for NR-DC in section 10 in TS 38.213

In Rel-15, the PDCCH monitoring capability for CA was extended to Rel-15 NR-DC. In the RAN1#100bis-e meeting, it was agreed for carrier aggregation that the minimum number of cells to be supported with Rel-16 PDCCH monitoring capability is 2. For Case 3 (the mixed capability monitoring) the UE can report one or more combinations of Rel-15 #CC(s) and Rel-16 #CC(s). In the current section 10 in TS 38.213, the UE behavior for Rel-16 PDCCH monitoring in case of NR-DC is still missing. Some TPs are needed for section 10 in TS 38.213.

Companies are encouraged to provide your views on the TP for section 10 in TS 38.213.

| Company | View |
| --- | --- |

Miscellaneous corrections

- Issue D-1: Correction on determination of a combination (X, Y) for PDCCH monitoring in section 10.1 in TS 38.213

Samsung (R1-2002131) proposes to adopt the following TP, to cover the case that a configuration of search space sets for PDCCH monitoring results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for only one of the multiple combinations X,Y.

If the UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for one or more combinations X,Y of the multiple combinations X,Y, the UE is expected to monitor PDCCH on the cell according to the combination X,Y from the one or more combinations X,Y associated with the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ.

Feature lead view: the current spec should be clear even without the changes, but no harm to make it clear. If time permits, we can discuss and endorse the TP.

Proposal 4.3-1: Adopt the following TP on determination of a combination (X, Y) for PDCCH monitoring in section 10.1 in TS 38.213.

If the UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for one or more combinations X,Y of the multiple combinations X,Y, the UE is expected to monitor PDCCH on the cell according to the combination X,Y from the one or more combinations X,Y associated with the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ.

Please provide your views on the above proposal 4.3-1.

| Company | View |
| --- | --- |

- Issue D-2: PDCCH candidates allocation for primary cell in a slot or a span

Sharp R1-2004331 - Issue 3: PDCCH candidates allocation for primary cell in a slot or a span Given the PDCCH overbooking would happen in a primary cell, the eURLLC UEs need to allocate PDCCH candidates for monitoring to USS sets for the primary cell in a slot level or a span level according to the PDCCH monitoring capability configuration. The relevant description in TS 38.213[4] has been captured as below. - 10.1 UE procedure for determining physical downlink control channel assignment The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided CORESETPoolIndex with value 0 for first CORESETs, and is provided CORESETPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring.. The above cases that the UE allocates PDCCH candidates for monitoring to USS sets for the primary cell in a slot, correspond to the Rel-15 UE and the eURLLC UE being configured all serving cells with Rel-15 monitoring capability. However, for an eURLLC UE, who is configured a number of serving cells with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells, there is a case that the primary cell is configured with Rel-15 monitoring capability. The current description on the TS38.213 seems to miss capturing the case. Proposal 3: Adopt the following TP3 for section 10.1 in TS 38.213 to compensate for a missing case of a UE being configured with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving sells and the primary cell is configured with Rel-15 monitoring capability. TP3 TS 38.213 V16.1.0 (2020-03) - 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells the primary cell, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided CORESETPoolIndex with value 0 for first CORESETs, and is provided CORESETPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. < Unchanged parts are omitted >.

Feature lead view: The correction is needed. If time permits, we can discuss it.

Proposal 4.3-2: Adopt the following TP for section 10.1 in TS 38.213.

- 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells the primary cell, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided CORESETPoolIndex with value 0 for first CORESETs, and is provided CORESETPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. < Unchanged parts are omitted >

Please provide your views on the above proposal 4.3-2.

| Company | View |
| --- | --- |

- Issue D-3: Correction on higher layer parameter

Huawei (R1-2003525) proposed to adopt the following TP to align the RRC parameter between 38.213 and 38.331.

*** Unchanged text is omitted *** If a UE is provided monitoringCapabilityConfig-r16 for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig-r16 = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig-r16 = r15monitoringcapability If the UE is not provided PDCCHMonitoringCapabilityConfig, the UE monitors PDCCH on the serving cell per slot. *** Unchanged text is omitted ***

Feature lead view: The correction is needed but can be handled by editor during the editor CR phase.

- References

- RP-191584, “Revised WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)”, RAN#84, Newport Beach, CA, June 3-6, 2019.

- R1-2003317	Remaining issues on PDCCH enhancements for NR URLLC	ZTE

- R1-2003387	PDCCH enhancements for URLLC	vivo

- R1-2003439	Remaining Issue of PDCCH Enhancements for NR URLLC	Ericsson

- R1-2003525	Corrections on PDCCH enhancement for URLLC	Huawei, HiSilicon

- R1-2003577	Maintenance of Rel-16 URLLC PDCCH enhancements	Nokia, Nokia Shanghai Bell

- R1-2003620	Remaining issues on PDCCH enhancements	CATT

- R1-2003684	Remaining issues on PDCCH enhancements	MediaTek Inc.

- R1-2003702	Remaining issue for TCI field	ASUSTeK

- R1-2003722	DCI Size Alignment	Futurewei

- R1-2003737	Remaining details on PDCCH enhancements for eURLLC	Intel Corporation

- R1-2003865	Remaining issues for PDCCH enhancements	Samsung

- R1-2003942	Remaining Issues on PDCCH Enhancements for URLLC	Quectel

- R1-2003944	Remaining issues on PDCCH enhancements for NR URLLC	Panasonic Corporation

- R1-2003979	Remaining issues of PDCCH enhancements for URLLC	Spreadtrum Communications

- R1-2004029	Remaining issues of PDCCH enhancements for NR URLLC	LG Electronics

- R1-2004114	PDCCH enhancements for URLLC	OPPO

- R1-2004221	Remaining Issues on PDCCH Enhancements for eURLLC	Apple

- R1-2004331	Remaining issues on PDCCH enhancements for NR URLLC	Sharp

- R1-2004389	Remaining issues for PDCCH enhancements for Rel.16 URLLC	NTT DOCOMO, INC

- R1-2004457	Remaining issues on PDCCH Enhancements for URLLC	Qualcomm Incorporated

- R1-2004522	Remaining issues on PDCCH enhancement for NR URLLC	WILUS Inc.