3GPP TSG RAN WG1 #102-e		R1-2005387

e-Meeting, August 17th – 28th, 2020

Source:	vivo, Guangdong Genius

Title:	RRM relaxation for Reduced Capability NR devices

Agenda Item:	8.6.5

Document for:	Discussion and Decision

- Introductions

In 3GPP TSG RAN Meeting #86, a new SID on support of reduced capability NR devices (RedCap) was agreed [1].

Identify and study potential UE complexity reduction features, including [RAN1, RAN2]: - Reduced number of UE RX/TX antennas - UE Bandwidth reduction Note: Rel-15 SSB bandwidth should be reused and L1 changes minimized - Half-Duplex-FDD - Relaxed UE processing time - Relaxed UE processing capability Note1: The work defined above should not overlap with LPWA use cases. The lowest capability considered should be no less than an LTE Category 1bis modem. Study UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g. delay tolerant) [RAN2, RAN1]: - Reduced PDCCH monitoring by smaller numbers of blind decodes and CCE limits [RAN1]. - Extended DRX for RRC Inactive and/or Idle [RAN2] - RRM relaxation for stationary devices [RAN2] Study functionality that will enable the performance degradation of such complexity reduction to be mitigated or limited, including [RAN1]: - Coverage recovery to compensate for potential coverage reduction due to the device complexity reduction. Study standardization framework and principles for how to define and constrain such reduced capabilities – considering definition of a limited set of one or more device types and considering how to ensure those device types are only used for the intended use cases [RAN2, RAN1]. Study functionality that will allow devices with reduced capabilities to be explicitly identifiable to networks and network operators, and allow operators to restrict their access, if desired [RAN2, RAN1].

This contribution discusses the RRM relaxation for RedCap UEs.

- RRM relaxation for RedCap devices

  - Ratio of idle state for wearables

The user habits and services types for smart watches are different from smart phones. According to our statistics, the time ratio for UE in idle state is 80% and 90% for 4G smart phones and 4G smart watches, respectively, as shown in Figure 1. Therefore, to improve the battery life for wearables, idle-mode power savings are critical for RedCap UEs.

Observation 1: IDLE mode power saving is critical for wearable devices.

In Rel-16, RRM relaxation for neighboring cells (both inter-/intra-frequency) are specified, while no relaxation was allowed for serving cell RRM. Actually, the RRM measurement relaxation for serving cell has been discussed and studied [2] in Rel-16, but not been specified due to limited scope. However, in most of RedCap use case (e.g. industrial sensors, and video surveillance), the UEs are stationary. In these use cases, the measurement relaxation for serving cell can be further considered for power saving, especially for high SNR UEs. In the following, we present our views based on our evaluation of serving cell RRM relaxation in idle mode. The Rel-16 UE power models defined in TR38.840 are reused in the evaluation. Furthermore, high SINR scenario and stationary UEs are assumed, the RRM measurements for both intra-frequency and inter-frequency neighboring cells are not considered.

  - Power saving gain evaluation of serving cell RRM relaxation in idle mode

Our simulation assumptions and simulation cases are summarized in the following:

- Simulation Assumptions:

- Paging rate = wake-up signal/channel (WUS) paging rate = 10%, referring to that the probability for WUS indicating the UE to wake up and receive paging PDSCH is 10%.

- The time-domain gap of SSB relative to Paging Occasion (PO) is 10ms.

- The gap between WUS and SSB is 0 or 3ms.

- No time gap between WUS and SSB

- 𝑃WUS= 𝑃PO = 𝑃SSB =50 power units/slot, where 𝑃WUS, 𝑃PO and 𝑃SSB represent the relative power of WUS, PO, SSB respectively.

- Scaled to a 20MHz receiving bandwidth from the 100MHz power model in TR38.840

- We assume that in High SINR case, UE only needs to receive the best two POs associated to the corresponding SSBs from two beam directions. Hence, TPO = 1ms, where TPO denotes the duration of PO reception.

- TWUS= TSSB = 2ms, where TWUS and TSSB denote the duration of WUS and SSB.

- One SSB per DRX cycle (paging cycle) is processed in high SINR case

- Simulation cases:

- Rel-15 paging monitoring without WUS

- Case1: No serving cell RRM relaxation is assumed.

- Case2: 4x serving cell RRM relaxation, i.e. only one SSB is measured every four paging cycles.

- WUS for paging monitoring, no gap between WUS and SSB

- Case3: No serving cell RRM relaxation is assumed.

- Case4: 4x serving cell RRM relaxation, i.e. only one SSB is measured every four paging cycles.

- WUS for paging monitoring, the gap between WUS and SSB is 3ms

- Case5: No serving cell RRM relaxation is assumed.

- Case6: 4x serving cell RRM relaxation, i.e. only one SSB is measured every four paging cycles.

For Case 1, UE needs to receive SSB and monitor PO per paging cycle. For Case 2, with RRM relaxation, UE only perform SSB measurement every four paging cycles. As shown in Figure 2, in the last three paging cycles, power consumption is saved by skipping 75% of SSB processing and longer deep sleep duration can be obtained.

Case 3 and Case 4 take WUS into account, but there is no gap between WUS and SSB. Because the WUS paging rate is 10%, we only consider the case that WUS indicates UE not to wake up to receive Paging in the PO. With RRM relaxation, only one SSB is measured every four paging cycles for Case 4. Reduced SSB measurement leads to longer deep sleep.

Case 5 and Case 6 take WUS into account, and the gap between WUS and SSB is assumed as 3ms with considering the WUS processing time. The other assumptions are no changed compared to Case 3 and Case 4.

Figure 2 Illustration for four evaluation cases

According to the simulation assumptions, the simulation results are summarized in Table 1. Case 2 can achieve 13.4% power saving gain over Case 1 thanks to the RRM relaxation. When WUS is configured and no gap between WUS and SSB, Case 4 reduces 3.6% power consumption over Case 3. Besides, when WUS is configured and the gap between WUS and SSB is 3ms, Case 6 reduces 8.3% power consumption over Case 5. Therefore, it is beneficial to support serving cell RRM relaxation for high SINR RedCap UE in idle mode.

Table 1 Simulation results of RRM relax in high SINR case

| Cases |  | Average relative power per slot | Power saving gain |
| --- | --- | --- | --- |
| w/o WUS | Case 1 | 1.6975 | 13.4% (case 2 over case 1) |
|  | Case 2 | 1.4709 |  |
| w/ WUS No gap between WUS and SSB | Case 3 | 1.5367 | 3.6% (case 4 over case 3) |
|  | Case 4 | 1.4814 |  |
| w/ WUS The gap between WUS and SSB is 3ms | Case 5 | 1.627 | 8.3% (case 6 over case 5) |
|  | Case 6 | 1.4914 |  |

Observation 2: 13.4% power saving gain in IDLE mode can be observed if serving cell RRM relaxation is introduced for high SINR UE.

  - Evaluation of time drift caused by RRM relaxation

Compared with Case1, for case 2, UE does not need to receive SSB every paging cycle, and the power saving gain is achieved by skipping (N-1) SSB receptions every N paging cycle. Typically, UE receives SSB for AGC and synchronization to guarantee the reliable reception of paging PDCCH and PDSCH. However, the channel condition would be stable if UE is stationary, and AGC does not need to be performed every paging cycle. Besides, the time drift caused by local oscillator is quite slow so that UE may still maintain good timing even if SSB receptions are skipped in several paging cycles.

Based on the time drift model as agreed in LTE eMTC/NB-IOT [3], we evaluated the time drift caused by oscillator after a number of DRX cycles. As shown in Table 2 that it is feasible for UE to process SSB every 5.12 seconds in order to maintaining good time synchronization, assuming 1/3 CP timing drift is acceptable.

Table 2 Time drift due to oscillator with 5ppm frequency error

| Sync periodicity(s) | 0.32 | 0.64 | 1.28 | 2.56 | 5.12 | 1/3 CP_length (30kHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Time drift(s) | 2.56×10-9 | 1.02×10-8 | 4.10×10-8 | 1.64×10-7 | 6.55×10-7 | 7.81×10-7 |

To investigate the demodulation impacts due to 1/3 CP timing drift, we performed the link-level evaluation as the following.

In the evaluation, we assume the receiver intercepts the time domain signal 1/3 CP ahead of the OFDM signal, as shown in Figure 3. Thus, even if UE suffers from time drift up to 1/3 CP length, the starting time of intercepted signal is still within CP duration, UE can still decode the DL transmission, e.g. paging PDCCH and paging PDSCH. We evaluated the performance of PDCCH and PDSCH with different starting time of intercepted signal, the evaluation assumptions are given in Table A-1 and A-2 in appendix, and the evaluation results are shown in Figure 4 and Figure 5.The results prove that a 1/3 CP timing drift is acceptable for paging reception.

Figure 3 Illustration of time domain signal interception for FFT transform at receiver

Figure 4 Performance of PDCCH with different starting time of intercepted signal

Figure 5 Performance of PDSCH with different starting time of intercepted signal

Note that, since NB-IoT UEs are low cost devices, the time drift caused by oscillator using the model in NB-IoT can be considered as the worst cases for Redcap UEs. Even if considering the worst case, UE are still allowed to receive SSB for synchronization every 5.12s with limited performance loss. In addition, considering the RRM relaxation can be applied to high SINR UEs, the performance loss due to sub-optimal timing is negligible.

Observation 3: It is feasible for RedCap UEs to process SSBs once per multiple paging cycles.

Proposal 1: Serving cell RRM relaxation for high SINR UE in idle state should be supported for RedCap UE.

- Conclusion

In this contribution, we discuss the serving cell RRM relaxation for RedCap NR devices, and have the following observations and proposals:

Observation 1: IDLE mode power saving is critical for wearable devices.

Observation 2: 13.4% power saving gain in IDLE mode can be observed if serving cell RRM relaxation is introduced for high SINR UE.

Observation 3: It is feasible for RedCap UEs to process SSBs once per multiple paging cycles.

Proposal 1: Serving cell RRM relaxation for high SINR UE in idle state should be supported for RedCap UE.

References

- RP-193238, “New SID on support of reduced capability NR devices”, Ericsson, 3GPP TSG RAN Meeting #86.

- 3GPP TR 38.840, “Study on User Equipment (UE) power saving in NR (Release 16)”.

- R1-1714993, “Assumptions for NB-IoT power consumption for power saving signal or channel”, RAN1#90, Ericsson.

Appendix: Evaluation assumptions for paging PDCCH and PDSCH

Table A-1 Evaluation assumptions for paging PDCCH

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Channel model | TDL-C Low |
| Delay scaling | 100ns |
| UE speed | 3km/h |
| Subcarrier spacing | 30 kHz |
| Antenna configuration | 2T,2R |
| Channel estimation | Real MMSE |
| DCI size | 39 bits |
| CORESET resource | 48 RBs and 2 symbols |
| REG bundle size | 6 |
| Interleaver size | 2 |

Table A-2 Evaluation assumptions for paging PDSCH

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Channel model | TDL-C Low |
| Delay scaling | 100ns |
| UE speed | 3km/h |
| Subcarrier spacing | 30 kHz |
| Antenna configuration | 2T,2R |
| Channel estimation | Real MMSE |
| Modulation | QPSK |
| Code rate | 1/3 or 2/3 |
| Data allocation | 12 symbols per slot, 12 RBs allocated |
| DMRS configuration | single symbol, 1 front-loaded+1 additional |