3GPP TSG RAN WG1 Meeting #102-e                                                R1-2005481

e-Meeting, August 17th – 28th, 2020

Source:              ZTE

Title:                   DL TBS increase for eMTC

Agenda Item:     8.9.3

Document for:   Discussion

- Introduction

In RAN #88 e-meeting, the revised WID on additional enhancements for NB-IoT and LTE-MTC was agreed [1]. One of the objectives is to add a Rel-17 optional UE capability to support a maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only:

  - Determine soft buffer size [RAN1]

  - Capability signaling without introducing a new UE category [RAN2]

  - There shall be no changes to: DCI formats, TBS tables, CQI tables

In this contribution, we provide the relevant analysis and proposal.

- Discussion

In CE mode A, for the BL/CE UE configured with up to 16QAM, a 4-bit "modulation and coding scheme" field is used in DCI for PDSCH scheduling and the MCS index indicated by this field is less than or equal to 15. For the BL/CE UE configured with up to 64QAM, a 5-bit "modulation and coding scheme" field is used in DCI for PDSCH scheduling. Then, the MCS for PDSCH is indicated based on Table 7.1.7.1-1 regardless of 4-bit field or 5-field in DCI.

Table 7.1.7.1-1: Modulation and TBS index table for PDSCH

| MCS Index | Modulation Order | Modulation Order | TBS Index |
| --- | --- | --- | --- |
| 0 | 2 | 2 | 0 |
| 1 | 2 | 2 | 1 |
| 2 | 2 | 2 | 2 |
| 3 | 2 | 2 | 3 |
| 4 | 2 | 2 | 4 |
| 5 | 2 | 4 | 5 |
| 6 | 2 | 4 | 6 |
| 7 | 2 | 4 | 7 |
| 8 | 2 | 4 | 8 |
| 9 | 2 | 4 | 9 |
| 10 | 4 | 6 | 9 |
| 11 | 4 | 6 | 10 |
| 12 | 4 | 6 | 11 |
| 13 | 4 | 6 | 12 |
| 14 | 4 | 6 | 13 |
| 15 | 4 | 6 | 14 |
| 16 | 4 | 6 | 15 |
| 17 | 6 | 6 | 15 |
| 18 | 6 | 6 | 16 |
| 19 | 6 | 6 | 17 |
| 20 | 6 | 6 | 18 |
| 21 | 6 | 6 | 19 |
| 22 | 6 | 6 | 20 |
| 23 | 6 | 6 | 21 |
| 24 | 6 | 6 | 22 |
| 25 | 6 | 6 | 23 |
| 26 | 6 | 6 | 24 |
| 27 | 6 | 6 | 25 |
| 28 | 6 | 6 | 26/26A |
| 29 | 2 | 2 | reserved |
| 30 | 4 | 4 |  |
| 31 | 6 | 6 |  |

Thus, it can be observed that the TBS range for the BL/CE UE configured with up to 16QAM is TBS 0 to 14 which are shown in Table 7.1.7.2.1-1.

Table 7.1.7.2.1-1: Transport block size table (6 PRBs)

|  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 |
| 6 | 328 | 176 | 256 | 392 | 504 | 600 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 |
| 13 | 224 | 488 | 744 | 1000 | 1256 | 1544 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 |

However, the transport block size lager than 1000 bits in Table 7.1.7.2.1-1 cannot be used for UE category M1 since Table 4.1A-1 restricts the DL maximum TBS of UE category M1 to 1000 bits in existing TS36.306.

# Table 4.1A-1: Downlink physical layer parameter values set by the field ue-CategoryDL

| UE DL Category | Maximum number of DL-SCH transport block bits received within a TTI (Note 1) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| DL Category M1 | 1000 | 1000 | 25344 | 1 |

If DL TBS is extended to 1736 bits, i.e. TBS 14, the current MCS table, CQI table and DCI indication will not be affected. But some descriptions in TS36.213 and UE soft buffer size in TS36.306 need to be modified. In particular, for the UE configured with 6PRB PDSCH bandwidth, TS36.213 has following specifications:

-	if the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config-r15 and the MPDCCH DCI format 6-1A is mapped onto the UE specific search space and the repetition number field in the DCI indicates PDSCH repetition level 1,

-	set  to the TBS determined by the procedure in Subclause 7.1.7.2.1,

-	TBS=minTBS',1000

-	otherwise

-	TBS is determined by the procedure in Subclause 7.1.7.2.1

Wherein, the description of “TBS=minTBS',1000” should be removed and ITBS≤14 is added when DL TBS is extended to 1736 bits.

Moreover, the downlink physical layer parameter values of Rel-17 UE category M1 shall be added in Table 4.1A-1. When DL TBS is extended to 1736 bits, UE soft buffer size with 16QAM is different from that with 64QAM for 6PRB bandwidth. A common soft buffer size should be defined for Rel-17 UE category M1. Considering that the UE with increased DL TBS may not have 64QAM capability, the soft buffer size should be defined based on 16QAM modulation.

Proposal: DL TBS for UE Cat-M1 can be extended to 1736 bits.

- Modify existing TBS restriction in TS36.213.

- Define new soft buffer size based on 16QAM in TS36.306

- Introduce the capability parameter in TS36.306

- Conclusion

In this contribution, we discuss DL TBS increase for UE Cat-M1 in CE mode A. And the proposal is given as below:

Proposal: DL TBS for UE Cat-M1 can be extended to 1736 bits.

- Modify existing TBS restriction in TS36.213.

- Define new soft buffer size in TS36.306

- Introduce the capability parameter in TS36.306

- Reference

[1] 3GPP, RP-201306, WID revision: Additional enhancements for NB-IoT and LTE-MTC, Huawei, HiSilicon, RAN #88e

[2] 3GPP, TS36.213

[3] 3GPP, TS36.306