3GPP TSG RAN WG1 #102e		                               R1-2005583

e-Meeting, Aug 17th – Aug 28th, 2020

Source: 	Sony

Title:	On TRS/CSI-RS occasion(s) for idle/inactive UEs

Agenda Item:	8.7.1.2

Document for:	Discussion

# Introduction

In the 3GPP TSG RAN#86 meeting, a new WID on power saving enhancement was approved [1]. One of the objectives is to specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects. This includes an objective for RAN1 to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact. NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required. In this contribution, we discuss aspects related to power saving enhancement for idle/inactive-mode, particularly on the possibility of using TRS/CSI-RS occasion(s) when the UE is in idle/inactive mode.

# Discussions

During idle/in-active mode, a UE is configured with a DRX operation, in which during the ON duration, the gNB transmits a paging indication in case there is downlink data for that UE or system information updates. During the DRX opportunity (OFF period), the UE basically can sleep (i.e. expect no data) to save energy. However, the UE needs to be ready / synchronized during ON period. Hence, it is expected that the UE performs gain control, synchronization and measurement prior to the ON duration. When a legacy NR UE is in idle/inactive mode, as opposed to LTE, no cell specific reference signals are available and the UE, as shown in Figure 1, can only rely on synchronization signal block (SSB) reception for the automatic gain control (AGC), synchronization, beam selection and measurement operations. This can lead to an increase in UE power consumption.

Figure 1: Idle mode with DRX operation and SSBs reception.

The legacy operation in NR can be described below:

- In IDLE mode, the UE monitors the initial bandwidth part (BWP) to receive SSB for synchronization purposes and cell detection and also monitors for potential paging following DRX configuration.

- During the initial access phase, the UE performs a random access procedure, where the UE gets attached to the network, receives RRC messages and enters the connected mode state. When in connected mode, the UE can move to another BWP that may have a wider bandwidth than the initial BWP (Here we call it active BWP)

- One of the RRC messages contains the reference signal configuration (CSI-RS, TRS) that the UE needs for measurement and further time/frequency synchronization.

- TRS is configured to have an association with the SSB. This association is described by the quasi-colocation (QCL) information. The UE can assume the same QCL configuration for the TRS reception and the associated SSB. Multiple TRS (CSI-RS) resources can be mapped to an SSB.

- Once the UE has finished data communication, the UE goes back to idle mode and only monitors the initial BWP, and uses SSB to measure and monitor for potential paging and cell re-selection.

In idle mode legacy NR operation, the UE is not configured to receive CSI-RS/TRS. The UE monitors the initial BWP and can only rely on SSB reception for the automatic gain control (AGC), synchronization, beam selection and measurement operations. Furthermore, the SSB bandwidth is limited to the initial BWP bandwidth (e.g. 5 MHz for 15 kHz subcarrier spacing). while TRS can have much wider bandwidth. Hence, TRS can provide finer measurement estimation results than SSB.

During IDLE mode, the UE monitors one PO per paging DRX cycle for potential paging and SI update. In multi beam operation (or in FR2) the UE needs to wake up much earlier than the paging occasion in order to establish a suitable beam to be used during the paging occasion. SSB availability can be sparse since the lowest periodicity can be 20 msec. UEs at the cell edge or when operating in FR2 may need to utilize multiple SSBs prior to the on-duration of the paging DRX cycle, when it receives control and data channels from gNB. The time needed for the UE to wake-up is highly determined by the location of SSB and PO. This means that if the SSB and POs are not aligned, the UE needs to wake-up multiple times or cannot go to deep sleep between SSB and PO.

The UE should perform serving cell measurements and evaluate the cell selection criterion based on SSB according to the requirements in 38.133. Decoding SSB can become costly as the SSB burst can be up to 5 msec long. as it also includes MIB necessary information, primarily used for the UE attempting to find the cell. This information is not necessarily needed when the UE loses its synchronization while it is still within the same cell. Furthermore, the UE may need to process multiple SSBs, depending on its operational frequency or SNR, to perform the above operations.

Observation 1 – Providing synchronization signal block (SSB) as the only reference signal can result in high power consumption at the UE.

Introducing additional reference signals can be useful to assist the UE in performing the above operations. The UE may skip some of the SSBs and hence reduced power consumption. Furthermore, the additional reference signals can improve the demodulation performance of the paging indication during the on-duration. One of the potential reference signals is the tracking reference signal (TRS). In legacy NR, TRS is a UE specific signal which the UE is configured to use during connected mode. A gNB is designed to serve many UEs. Once a UE goes to idle mode, the other UEs can still be in connected mode. Hence, the TRS of other UE(s) in connected mode are essentially still available and ideally they can be used by the idle mode UE. As stated in WID [1], it would be beneficial for UEs in IDLE/INACTIVE mode to be able to monitor these reference signals when they are available. TRS is typically transmitted by the gNB with relatively wider bandwidth than SSB. Hence, TRS may provide fine time/frequency synchronization, assist AGC operation, and any other UE operations as described above. The usage of TRS is mainly for power saving purpose. The UE can combine the TRS and SSB reception (hence, requiring a smaller number of SSBs) or even replace the SSB reception with TRS reception (typically when the UE is static and has a strong received signal).

Observation 2 – Using reference signals available in connected mode for idle/inactive-mode UE can be beneficial and results in UE power saving.

To provide the reference signals (TRS) available in connected mode UEs to idle/inactive mode UEs, we need to consider the following,

- What information has to be provided in order that idle/inactive mode UEs can utilize the TRS of other UEs and how to provide this information with minimized signaling. We consider that the UE needs to be configured on the possibility of using TRS during idle mode by the gNB while the UE is in connected mode.

- There should be some criteria/validation on whether the UE can use TRS in idle mode. For example, once the UE in idle mode changes its cell then we consider the UE cannot use TRS until it has been configured by the new gNB.

Proposal 1 – Study the information/configuration to enable the idle/inactive mode UE to use TRS and also the validation mechanism on whether the UE can use TRS of other UEs.

# Conclusion

In this contribution, we discuss power saving enhancement for UEs in IDLE or INACTIVE mode.

Observation 1 – Providing synchronization signal block (SSB) as the only reference signal can result in high power consumption at the UE.

Observation 2 – Using reference signals available in connected mode to idle/inactive-mode UE can be beneficial and results in UE power saving.

Proposal 1 – Study the information/configuration to enable the idle/inactive mode UE to use TRS and also validation mechanism whether the UE can use TRS of other UEs.

# References

- RP-193239, New WID: UE Power Saving Enhancements.

- TR 38.840, “Study on UE power saving”, RAN1.