3GPP TSG RAN WG1 #102-e	R1-2005609

e-Meeting, August 17th – 21th, 2020

Source:            ZTE, Sanechips

Title:                 Preliminary simulation results for above 52.6GHz

Agenda Item:   8.2.3

Document for: Discussion and Decision

# Introduction

In RAN1 101 e-meeting and post meeting email discussion, the proposals in Tables 2, 3, 4, 5, 7, 8, 9, 10, 11, 12 of R1-2005185 are agreed, which includes the LLS and SLS simulation assumption and objectives.

In this contribution, we provide some primary simulation results of LLS and SLS based on the agreed simulation assumption in R1-2005185 for above 52.6GHz.

# Link level simulation

## PUSCH

### TDL-A 5ns

- 16QAM

Figure 2.1-1 TDL-A 16QAM

It is shown from Figure 2.1-1, for 16QAM TDL-A channel:

- Without phase noise, different SCS shows similar performance.

- With phase noise and without PN compensation, phase noise will cause about 1 dB performance degradation on each SCS. Larger SCS shows better performance against PN, 120kHz is 0.5dB worse than 960kHz.

- With phase noise and with PTRS PN compensation, the performance is almost the same as the performance without phase noise.

- 64QAM

Figure 2.1-2 TDL-A 64QAM

It is shown from Figure 2.1-2, for 64QAM TDL-A channel:

- Without phase noise, different SCS shows similar performance.

- With phase noise and without PN compensation, phase noise will cause significant performance degradation on each SCS, larger SCS shows better resistance against the impairment caused by PN.

- With phase noise and with PTRS PN compensation, the performance is much better than without PN compensation. Although there is still some gap from the reference performance without PN, larger SCS shows better performance with PTRS PN compensation.

## PDSCH

### CDL-B 20ns

Considering 16QAM TDL-A performance of different SCS is similar, for DL evaluation, we only consider 64QAM in order to show the difference between different SCS.

Figure 2.2-1 CDL-B 64QAM

Figure 2.2-1 shows the evaluation results for 64QAM and CDL-B channel model with 20ns delay spread. Generally, similar observation as UL can be found as follows:

- Without phase noise, different SCS shows similar performance, 960kHz performance is slightly worse than others, probably due to the shortened CP length.

- With phase noise and if we do not do the PN compensation, significant performance loss can be observed on each curve with different SCS.

- Larger SCS shows better performance with PTRS PN compensation.

Observation 1: Phase noise has limited impact on 16QAM modulation, and with PTRS PN compensation, different SCS (120kHz, 240kHz, 480kHz, 960kHz) shows similar performance.

Observation 2: Phase noise has significant impact on 64QAM modulation, and with PTRS PN compensation, larger SCS shows better performance.

# System level simulation

## Coexistence interference analysis

In this section, some simulation is performed to study the interference of different LBT schemes. As we know, in 60 GHz frequency, omni-directional LBT is used in Wi-Fi system such as IEEE 802.11 ad/ay.

Figure 3.3-1 Indoor scenario C

In this part, we provide system-level simulations to evaluate the impact of LBT schemes on the coexistence between NR-U and Wi-Fi systems, where one operator using omni-directional LBT mimics the behavior of Wi-Fi system and the other operator is assumed as NR-U can choose between omni-directional LBT and directional LBT. The detailed simulation assumptions are listed in Appendix.

- Case 1 corresponds to omni-directional LBT for Operator 1 (mimic an existed Wi-Fi operator) vs omni-directional LBT for Operator 2 (mimic a new NR-U operator) ;

- Case 2 corresponds to omni-directional LBT for Operator 1(mimic an existed Wi-Fi operator) vs directional LBT for Operator 2 (mimic a new NR-U operator).

In Case 1, gNBs are randomly dropped in a 10m*10m box, UEs are randomly dropped in the 120m*50m room. In Case 2, the randomization is totally the same with Case 1 in order to show a compatible performance between 2 cases. Simulation assumptions and detailed statistic can be found in Appendix.

Figure 3.1-2 Coexistence interference analysis

It can be shown in Figure 3.1-2:

- In mean and 5% and 95% UPT, it is shown from Case1 that Operator 1 has a relatively high throughput with omni-directional LBT compared to the same LBT mechanism used for Operator 2. The performance difference between the two operators is mainly due to the random distribution of the nodes in this simulation drop which is not friendly to operator 2. However, we can see from Case 2 that such phenomenon is mitigated when directional LBT is used for Operator 2 instead of omni-directional LBT.

- In 50% UPT, it can be observed that NR-U and Operator 1 show very similar performance and means that the performance of operator 1 will not be significantly affected even if directional LBT is used for Operator 2.

Observation 3: Compared to omni-directional LBT, directional LBT is beneficial to increase the probability of channel access and the spatial reuse efficiency for NR-U, and the impact on the performance of the existed Wi-Fi system is negligible.

## CCA threshold and LBT schemes

In this section, the deployment scenario is Indoor scenario A, 2 operators use the same LBT mechanism, directional LBT or Omnidirectional LBT. Different CCA threshold such as [-82dBm -62dBm -42dBm] are compared to evaluate the total performance of different LBT schemes.  Medium traffic load and high traffic load performance is shown in Figure 3.2-1~3.2-2.

Simulation assumptions and detailed statistic can be found in Appendix.

- Medium traffic load

Figure 3.2-1 Medium traffic load mean UPT

- High traffic load

Figure 3.2-2 High traffic load mean UPT

Observation 4: As the CCA threshold increases, the LBT failure probability gradually decreases and different LBT schemes converge to a similar performance.

Observation 5: With appropriate CCA threshold, Directional LBT shows better performance than Omnidirectional LBT in NRU-NRU coexistence scheme.

# Conclusion

Observation 1: Phase noise has limited impact on 16QAM modulation, and with PTRS PN compensation, different SCS (120kHz, 240kHz, 480kHz, 960kHz) shows similar performance.

Observation 2: Phase noise has significant impact on 64QAM modulation, and with PTRS PN compensation, larger SCS shows better performance.

Observation 3: Compared to omni-directional LBT, directional LBT is beneficial to increase the probability of channel access and the spatial reuse efficiency for NR-U, and the impact on the performance of the existed Wi-Fi system is negligible.

Observation 4: As the CCA threshold increases, the LBT failure probability gradually decreases and different LBT schemes converge to a similar performance.

Observation 5: With appropriate CCA threshold, Directional LBT shows better performance than Omnidirectional LBT in NRU-NRU coexistence scheme.

# Reference

- Chairman's Notes RAN1 92 final, RAN1 #92, Feb. 2018.

- ETSI EN 301 893 V2.1.1 (2017-05)

- TR 36.889, Study on Licensed-Assisted Access to Unlicensed Spectrum, V13.0.0, Jul. 2015.

- TR 36.802, Study on New Radio (NR) Access Technology Physical Layer Aspects, V1.1.0, Jan. 2017.

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V14.2.0, Sep. 2017.

# Appendix

## A1. LLS Simulation assumptions

Table A1-1 LLS simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 60GHz |
| Waveform | CP-OFDM |
| System Bandwidth | 400MHz |
| Subcarrier spacing | 120kHz/240kHz/480kHz/960kHz |
| RB number | 8/4/2/1 |
| CP type | Normal CP |
| Channel Model | TDL-A 5ns CDL-B 20ns |
| PN model | 3GPP TR38.803 example 2 |
| DMRS Configuration | 2 DMRS symbols per slot at (2,11) |
| PTRS Configuration | (K = 2, L = 1) |
| SLIV | (S=0, L=14) |
| Channel Estimation | Realistic |
| PN Estimation | Realistic |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | For TDL model: 1*2 For CDL model: (M, N, P, Mg, Ng) = (8, 16, 1, 1, 1), dH = dV = 0.5 λ |
| UE antenna Array configuration | For TDL model: 1*2 For CDL model: (M, N, P, Mg, Ng) = (4, 4, 1, 1, 1), dH = dV = 0.5 λ |

## A2. LLS Simulation assumptions

Table A2-1 SLS simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 60GHz |
| Channel Bandwidth | 2GHz |
| Subcarrier spacing | 960kHz |
| Scenario | Indoor A |
| LBT schemes | Omnidirectional LBT Directional LBT |
| CCA threshold | [-92dBm -82dBm -62dBm -42dBm] |
| Channel Model | InH Open office model in TR 38.901 Chapter 7.4.1 |
| BS Tx Power | 14dBm |
| UE Tx Power | 21dBm |
| BS Antenna gain | 5dBi |
| UE Antenna gain | 5dBi |
| BS Noise Figure | 7dB |
| UE Receiver Noise Figure | 10dB |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna Array configuration | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | FTP Model 3 (27Mbyte file) |

## A3. SLS simulation results

Table A3-1 Coexistence interference analysis

| Reported parameter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Case1: Omni vs Omni |  | Case2: Omni vs Directional |  |
|  |  | Operator1 | Operator2 | Operator1 （Omni） | Operator2 |
| DL： UPT CDF [Mbps] | 5% | 330.6076 | 145.3680 | 342.1996 | 213.1892 |
|  | 20% | 698.4680 | 718.9111 | 646.5258 | 812.3870 |
|  | 50% | 1899.3197 | 2012.3595 | 1864.7524 | 2303.1646 |
|  | 95% | 13254.0068 | 7434.4336 | 11698.5234 | 10241.9492 |
|  | Mean | 3927.3369 | 2864.0049 | 3742.8137 | 3825.9131 |
| DL： Delay CDF [s] | 5% | 0.013 | 0.021 | 0.013 | 0.018 |
|  | 50% | 0.132 | 0.133 | 0.135 | 0.090 |
|  | 95% | 0.975 | 1.836 | 1.100 | 1.268 |
|  | Mean | 0.266 | 0.403 | 0.318 | 0.267 |
| BO（%） |  | 51.958 | 52.811 | 55.795 | 40.687 |
| RU（%） |  | 28.716 | 35.741 | 30.941 | 27.287 |
| lamuda |  | 1.25/4files/s |  |  |  |

Table A3-2 High traffic load omnidirectional LBT

| Reported parameter |  | NRU +NRU(omni) |  |  |
| --- | --- | --- | --- | --- |
|  |  | CCA=-82dBm | CCA=-62dBm | CCA=-42dBm |
| DL： UPT CDF [Mbps] | 5% | 11.5132 | 258.4258 | 300.1205 |
|  | 10% | 20.0545 | 406.6419 | 517.3281 |
|  | 50% | 714.9429 | 2119.0698 | 2273.7708 |
|  | 95% | 6843.7842 | 13586.9219 | 14820.4902 |
|  | Mean | 1813.1741 | 3828.8721 | 4150.6094 |
| DL： Delay CDF [s] | 5% | 0.019 | 0.013 | 0.013 |
|  | 50% | 0.334 | 0.116 | 0.106 |
|  | 95% | 27.013 | 2.321 | 1.191 |
|  | Mean | 4.346 | 0.711 | 0.461 |
| 𝜌（%) |  | 65.2185 | 95.0905 | 98.5297 |
| BO（%） |  | 77.811 | 57.496 | 54.842 |
| RU（%） |  | 77.341 | 57.139 | 55.170 |
| lamuda |  | 1.25files/s |  |  |

Table A3-3 High traffic load directional LBT

| Reported parameter |  | NRU +NRU(direcional) |  |  |
| --- | --- | --- | --- | --- |
|  |  | CCA=-82dBm | CCA=-62dBm | CCA=-42dBm |
| DL： UPT CDF [Mbps] | 5% | 18.4151 | 322.0743 | 300.1205 |
|  | 10% | 26.1314 | 506.8828 | 517.3281 |
|  | 50% | 1398.0867 | 2259.9055 | 2273.7708 |
|  | 95% | 9657.4678 | 14573.1670 | 14820.4902 |
|  | Mean | 2599.9751 | 4128.9458 | 4150.6094 |
| DL： Delay CDF [s] | 5% | 0.016 | 0.013 | 0.013 |
|  | 50% | 0.160 | 0.106 | 0.106 |
|  | 95% | 17.101 | 1.201 | 1.191 |
|  | Mean | 2.800 | 0.453 | 0.461 |
| 𝜌（%) |  | 83.4192 | 98.4964 | 98.5297 |
| BO（%） |  | 64.738 | 55.493 | 55.170 |
| RU（%） |  | 64.343 | 55.167 | 54.842 |
| lamuda |  | 1.25files/s |  |  |

Table A3-4 Medium traffic load omnidirectional LBT

| Reported parameter |  | NRU +NRU(omni) |  |  |
| --- | --- | --- | --- | --- |
|  |  | CCA=-82dBm | CCA=-62dBm | CCA=-42dBm |
| DL： UPT CDF [Mbps] | 5% | 498.9044 | 824.0012 | 792.1085 |
|  | 10% | 687.4977 | 1246.5151 | 1277.9742 |
|  | 50% | 2560.2195 | 3645.6738 | 3650.9165 |
|  | 95% | 13730.7559 | 19198.8438 | 19525.3438 |
|  | Mean | 4095.3210 | 5635.7500 | 5704.1040 |
| DL： Delay CDF [s] | 5% | 0.015 | 0.012 | 0.012 |
|  | 50% | 0.092 | 0.059 | 0.058 |
|  | 95% | 0.660 | 0.281 | 0.269 |
|  | Mean | 0.194 | 0.096 | 0.096 |
| 𝜌（%) |  | 98.8948 | 98.0911 | 98.3255 |
| BO（%） |  | 37.093 | 50.142 | 49.284 |
| RU（%） |  | 20.710 | 13.147 | 12.998 |
| lamuda |  | 1.25/4files/s |  |  |

Table A3-5 Medium traffic load directional LBT

| Reported parameter |  | NRU +NRU(direcional) |  |  |
| --- | --- | --- | --- | --- |
|  |  | CCA=-82dBm | CCA=-62dBm | CCA=-42dBm |
| DL： UPT CDF [Mbps] | 5% | 848.8726 | 812.2195 | 792.1085 |
|  | 10% | 1237.0601 | 1264.5615 | 1277.9742 |
|  | 50% | 3329.4951 | 3631.0200 | 3650.9165 |
|  | 95% | 16873.7363 | 19125.8789 | 19525.3438 |
|  | Mean | 5413.4297 | 5691.3403 | 5704.1040 |
| DL： Delay CDF [s] | 5% | 0.012 | 0.012 | 0.012 |
|  | 50% | 0.061 | 0.058 | 0.058 |
|  | 95% | 0.312 | 0.276 | 0.269 |
|  | Mean | 0.103 | 0.097 | 0.096 |
| 𝜌（%) |  | 98.0576 | 98.4260 | 98.3255 |
| BO（%） |  | 53.080 | 53.518 | 49.284 |
| RU（%） |  | 13.932 | 13.005 | 12.998 |
| lamuda |  | 1.25/4files/s |  |  |