3GPP TSG RAN WG1 Meeting #102-e                                    R1-2005679

e-Meeting, August 17th – 28th, 2020

Source:	CATT

Title:	Remaining Issues on Multi-beam Operation Enhancement

Agenda Item:	7.2.6.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we deal with remaining issues on multi-beam operation in NR Rel-16 and provide our TPs.

- TPs for specifications

2.1  TPs for 38.213

In section 6 of TS38.213 it is specified that periodic CSI-RS can be explicitly or implicitly provided as the reference signal of Beam Failure Detection (BFD). SS/PBCH blocks are not included in the BFD reference signal.

TS38.213: 6 Link recovery procedures A UE can be provided, for each BWP of a serving cell, a set of periodic CSI-RS resource configuration indexes by failureDetectionResources or beamFailureDetectionResourceList …….. If the UE is not provided by failureDetectionResources or beamFailureDetectionResourceList for a BWP of the serving cell, the UE determines the set to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH.

At the same time, in section 6 of TS38.213 it is said that in RAN1 when UE assess the quality of the beam, both CSI-RS and SS/PBCH blocks can be used.

TS38.213: 6 Link recovery procedures The physical layer in the UE assesses the radio link quality according to the set of resource configurations against the threshold Qout,LR. For the set , the UE assesses the radio link quality only according to periodic CSI-RS resource configurations, or SS/PBCH blocks on the PCell or the PSCell, that are quasi co-located, as described in [6, TS 38.214]

These two paragraphs are contradictory to each other as to whether SS/PBCH block are allowed to be used for beam failure detection. A clarification is needed. Depending on the outcome, one of these two paragraphs needs to be updated.

Proposal : Clarify whether SS/PBCH blocks can be provided for BFD.

- Conclusion

In this contribution, the following proposal is given:

Proposal : Clarify whether SS/PBCH blocks can be provided for BFD.

- References

- 3GPP RAN1, TS 38.213 V16.1.0