3GPP TSG RAN WG1 #102-e					  	R1-2005759

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.8.2.2

Source:	NEC

Title:              	Discussion on PUCCH coverage enhancement

Document for:	Discussion and Decision

- Introduction

One of the objective in the SID [1] of coverage enhancement is,

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services - The target channels include at least PUSCH/PUCCH - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions) - Study the additional enhanced solutions for FR2 if any - Evaluate the performance of the potential solutions based on link level simulation.

In this contribution, we provide our view on the PUCCH coverage enhancement and potential solution.

- Discussion

  - Cross channel estimation for PUCCH

In our relevant discussion paper [2], we provide the benefits to support cross channel estimation on PUSCH. The PUCCH has the same reason to support cross channel estimation when PUCCH repetition is supported. In coverage limited case, PUCCH repetition is the most effective method to increase the coverage and should be adopted. The cross channel estimation at least within all the PUCCH repetitions of one UCI feedback should be consider. The PUSCH cross channel estimation and PUCCH cross channel estimation could be configured separately.

Proposal 1: If PUSCH cross channel estimation is adopted for coverage enhancement, PUCCH cross channel estimation should also be supported for PUCCH repetition case.

  - Enhanced frequency hopping for PUCCH

In our relevant discussion paper [2], we provide the benefits to enhance the frequency hopping on PUSCH. The PUCCH has the same reason to enhance the frequency hopping even more as PUCCH is not dynamic scheduling hence no scheduling gain for PUCCH. In current NR, only two RB positions are supported by PUCCH frequency hopping. Only inter slot frequency hopping is supported for PUCCH repetition case. If increasing the RB positions for frequency hopping in PUSCH is supported for coverage enhancement, PUCCH should also increase the RB positions for frequency hopping. The additional RB positions, e.g. totally 4 position, can be transmitted on the every 3rd and 4th PUCCH slot or on the second hop within the every 1st and 2nd PUCCH slot, i.e. support intra inter combined PUCCH frequency hopping.

Proposal 2: If PUSCH enhanced frequency hopping is adopted for coverage enhancement, PUCCH enhanced frequency hopping should also be supported.

  - Sequenced based transmission of PUCCH

Sequenced based transmission and non-coherent detection at receiver is adopted in Rel-15 NR for PUCCH format 0. Sequence based transmission without DMRS can provide less required demodulation SNR which could increase the coverage. However, sequence based transmission may have higher detection complexity than coherent detection by channel estimation and decoding. To increase the PUCCH coverage for small UCI payload should be studied. For coverage limited scenario, power accumulation in time domain is very important. Even for UCI 1~2 bits, PUCCH format 1 is adopted. Sequence based transmission (format 0 like) can be used to enhance PUCCH format 1 for coverage enhancement, the drawback to adopt the sequence based transmission to enhance format 1 is the number of multiplexing UE will reduce. It’s a balance to consider the enhanced format 1 and coverage, i.e. only coverage limited UE is configured enhanced format 1 and other UEs are configured legacy format 1 which gives large multiplexing capacity.

Proposal 3: Sequence based transmission (PUCCH format 0 like) could be studied to enhance PUCCH format 1 for UCI up to 2 bits.

To support sequence based transmission for UCI bits larger than 2 bits and up to X bits should also be consider. First of all, we should decide the value of X. It depends on the orthogonal or non-orthogonal sequence we will use. PUCCH format 0 use the orthogonal DFT sequence. In coverage/power limited case, the larger number of transmission RB will not increase the coverage obviously, one RB allocation is enough in such case. For one RB and one slot allocation, there are at most 168 orthogonal sequences which could support X = 7.

Proposal 4: Sequence based transmission could be studied to enhance PUCCH transmission for UCI larger than 2 bits and up to X bits.

- Conclusion

In this contribution, we provide our view on the PUCCH coverage enhancement and we propose that,

Proposal 1: If PUSCH cross channel estimation is adopted for coverage enhancement, PUCCH cross channel estimation should also be supported for PUCCH repetition case.

Proposal 2: If PUSCH enhanced frequency hopping is adopted for coverage enhancement, PUCCH enhanced frequency hopping should also be supported.

Proposal 3: Sequence based transmission (PUCCH format 0 like) could be studied to enhance PUCCH format 1 for UCI up to 2 bits.

Proposal 4: Sequence based transmission could be studied to enhance PUCCH transmission for UCI larger than 2 bits and up to X bits.

- References

[1] RP-193240, New SID on NR coverage enhancement, TSG RAN Meeting #86, China Telecom.

[2] R1-2005758, Discussion on PUSCH coverage enhancement, TSG RAN WG1 #102-e, NEC.