3GPP TSG RAN WG1 Meeting #102-e	R1-2005815

E-meeting, August 17–28, 2020

Agenda Item:	6.2.1

Source:	Huawei, HiSilicon

Title:	Corrections regarding RAN2 LS reply on PUR

Document for:	Discussion and Decision

Introduction

This paper discusses the issues of PUR regarding the RAN2 reply LS in [1].

L1 adjustment on the PUSCH repetition number

In Rel-16 PUR, dedicated PUR ACK DCI is newly introduced, and a 3-bit “PUSCH repetition adjustment” filed is included in this DCI. One remaining issue is whether the L1 adjustment on the PUSCH repetition number is intended to update the higher layer (i.e. RRC) configuration or not.

RAN1 has discussed this issue in RAN1#100bis-e and RAN1#101-e, and the following conclusion was made in RAN1#101-e:

(copied from RAN1#101-e chairman notes) Conclusion Regarding L1 adjustment on the NPUSCH repetition number, one of the following TPs is the starting point for discussion for TS 36.213 based on the outcome of RAN2’s discussion - If the L1 adjustment on the (N)PUSCH repetition number is intended to update the higher layer (i.e. RRC) configuration, then it is RAN1 understanding the text proposal in R1-2004897 will be endorsed - If the L1 adjustment on the (N)PUSCH repetition number is to be used instead of the configuration provided by higher layers, then it is RAN1 understanding the text proposal in R1-2004898 will be endorsed

In RAN2#110-e, RAN2 sent a reply LS [1] to RAN1, and RAN2’s response is copied below for reference:

(copied from RAN2’s reply LS [2]) RAN2 response: RAN2 would like to inform RAN1 that RAN2 will update their specifications so that the adjustment on the (N)PUSCH repetition number provided with L1 ACK / fallback indicator updates the repetition number configuration in PUR configuration in RRC layer. RAN2 expects that PHY layer will provide a 3-bit repetition adjustment index to higher layers so that the value can be stored in the PUR configuration in RRC and expects that the format of the 3-bit information is same as RRC parameter numRepetitions-r16 for eMTC CE Mode A and CE Mode B and npusch-NumRepetitionsIndex-r16 for NB-IoT. Furthermore, RAN2 expects PHY layer to provide L1 ACK / fallback indication to higher layers.

It can be observed that the first bullet in the RAN1 conclusion is aligned with RAN2’s reply LS, i.e., L1 adjustment on the (N)PUSCH repetition number is intended to update the higher layer (i.e. RRC) configuration. Therefore, the TP in Annex I can be endorsed.

Proposal 1: Endorse the text proposal in Annex I and capture its contents in TS 36.213.

Moreover, as mentioned in the RAN2’s reply LS, RAN2 expects that PHY layer will provide a 3-bit repetition adjustment index to higher layers. Therefore, the following TP is proposed.

Proposal 2: Endorse the text proposal in Annex II and capture its contents in TS 36.213.

Collision handling between PUR transmission/ PUR SS monitoring and Paging CSS/ WUS

Regarding collision handling between PUR transmission/ PUR SS monitoring and Paging CSS/ WUS, the following WA was made in RAN1#100bis-e, and an LS [2] was sent to RAN2 for confirmation.

(copied from RAN1#100bis-e chairman notes) Working Assumption - When PUR transmission overlaps with WUS, PUR transmission is prioritized - For eMTC, this applies only to HD-FDD UEs - When PUR SS monitoring overlaps with Paging CSS, PUR SS monitoring is prioritized - When PUR SS monitoring overlaps with WUS, PUR SS monitoring is prioritized If it is concluded by RAN2 that the working assumption is feasible, the working assumption will be automatically confirmed. R1-2002944 LS on PUR working assumption for NB-IoT and eMTC RAN1, Huawei Agreement The LS to RAN2 on PUR working assumption for NB-IoT and eMTC is approved.

RAN1 has further discussed this issue in RAN1#101-e, and the following conclusion was made in RAN1#101-e:

(copied from RAN1#101-e chairman notes) Conclusion The text proposal in R1-2004896 is the starting point for discussion for TS 36.213 based on the outcome of RAN2’s discussion and it is RAN1 understanding it will be endorsed unless it is concluded by RAN2 that the working assumption in R1-2002944 is infeasible.

In RAN2#110-e, RAN2 sent a reply LS [1] to RAN1, and RAN2’s response is copied below for reference:

(copied from RAN2’s reply LS [1]) RAN2 response: RAN2 would like to inform RAN1 that the working assumption is feasible from RAN2 point of view and can be confirmed.

Based on RAN2’s reply LS, the working assumption above is feasible and can be automatically confirmed, and the text proposal in Annex III can be endorsed according to the RAN1 conclusion.

Proposal 3: Confirm the Working Assumption in RAN1#100bis-e

- When PUR transmission overlaps with WUS, PUR transmission is prioritized

  - For eMTC, this applies only to HD-FDD UEs

- When PUR SS monitoring overlaps with Paging CSS, PUR SS monitoring is prioritized

- When PUR SS monitoring overlaps with WUS, PUR SS monitoring is prioritized

Proposal 4: Endorse the text proposal in Annex III and capture its contents in TS 36.213.

Conclusion

The contribution discusses some remaining issues of PUR. The proposals based on the analysis in the contribution are as following:

Proposal 1: Endorse the text proposal in Annex I and capture its contents in TS 36.213.

Proposal 2: Endorse the text proposal in Annex II and capture its contents in TS 36.213.

Proposal 3: Confirm the Working Assumption in RAN1#100bis-e

- When PUR transmission overlaps with WUS, PUR transmission is prioritized

  - For eMTC, this applies only to HD-FDD UEs

- When PUR SS monitoring overlaps with Paging CSS, PUR SS monitoring is prioritized

- When PUR SS monitoring overlaps with WUS, PUR SS monitoring is prioritized

Proposal 4: Endorse the text proposal in Annex III and capture its contents in TS 36.213.

- References

- R2-2005946, “LS reply on PUR transmission for NB-IoT/eMTC”, Ericsson, RAN2#110-e, 1 – 12 June 2020

- R1-2002944, “LS on PUR working assumption for NB-IoT and eMTC”, Huawei, RAN1#100bis-e, April 20th – 30th, 2020

- Annex I

------------------------------------- Start of Text Proposal for TS 36.213-------------------------------

-------------------------------------------- Unchanged parts omitted -------------------------------------

## 8.0	UE procedure for transmitting the physical uplink shared channel

-------------------------------------------- Unchanged parts omitted -------------------------------------

For a PUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number configured by higher layers.

-------------------------------------------- Unchanged parts omitted -------------------------------------

- ---------------------------------------------- End of Text Proposal ----------------------------------------

- Annex II

------------------------------------- Start of Text Proposal for TS 36.213-------------------------------

------------------------------------------- Unchanged parts omitted --------------------------------------

#### 9.1.5.3	Preconfigured Uplink Resource ACK/fallback procedure

If a UE has initiated a PUSCH transmission using preconfigured uplink resource on a given serving cell, and upon detection of a MPDCCH with DCI format 6-0A/6-0B with CRC scrambled by PUR C-RNTI intended for the UE within the PUR search space window as defined in Subclause 9.1.5, and the corresponding DCI is for PUR ACK/fallback indication (as defined in [4]), the UE shall deliver the PUR ACK/fallback indication and PUSCH repetition adjustment, as signalled on the MPDCCH, to the higher layers.

-------------------------------------------- Unchanged parts omitted -------------------------------------

- ---------------------------------------------- End of Text Proposal ----------------------------------------\

- Annex III

---------------------------Start of Text Proposal for 36.213(Editor CR, R1-2002962) -------------------------

9.1.5	MPDCCH assignment procedure

<Unchanged parts are omitted>

The BL/CE UE is not required to monitor Type2A-MPDCCH common search space if the set of subframes comprising the search space include any subframes in which it monitors Type1A-MPDCCH common search space or any subframes in which the UE receives PDSCH assigned by MPDCCH with DCI CRC scrambled by SC-RNTI.

A BL/CE UE is not required to monitor Type1-MPDCCH common search space or MWUS if the set of subframes comprising the search space or the set of subframes where MWUS may be received include any subframes in which the UE has initiated a PUSCH transmission using preconfigured uplink resource on a given serving cell.

<Unchanged parts are omitted>

If the UE has initiated a PUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the MPDCCH UE-specific search space in a search space window starting in subframe n+4 with duration given by higher layer parameter pur-MPDCCH-SS-window-duration. Upon detection of a MPDCCH with DCI format 6-0A/6-0B with CRC scrambled by PUR C-RNTI intended for the UE within the search space window and the corresponding DCI is for PUR ACK/fallback indication (as defined in [4]), the UE is not required to monitor the MPDCCH UE-specific search space for the remaining search space window duration. The UE is not required to monitor Type1-MPDCCH common search space or MWUS on the set of subframes where it may be received, in case any of these overlaps with any of the subframes comprising the PDCCH UE-specific search space within the search space window.

- -----------------------------------------------------End of Text Proposal---------------------------------------------