3GPP TSG RAN WG1 #102-e Meeting	    			             R1-2006175

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.12.3

Source: 	Samsung

Title: 		On Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

Document for:	Discussion and Decision

Introduction

In order to support multicast transmission in NR, the work item was approved for Rel-17 [1]. There would be many applications of multicast data transmission, which was not supported by NR in Rel-15 and Rel-16, where the use cases include mission critical, V2X applications, multicast delivery, IPTV, software delivery over wireless, group communications and IoT applications.

In this contribution, the basic mechanism to support RRC_idle and RRC_inactive UEs for multicast data transmission is discussed.

Discussion

In LTE, two types of broadcast and multicast transmission were supported: 1) eMBMS and 2) SC-PTM. The former is by configuring specific subframes and using the entire system bandwidth while the latter is by using PDSCH like unicast transmission with group RNTI delivered by SIB.

Similar to the case for RRC_connected UEs, BWP, CORESET, and multicast data transmission should be discussed also for RRC_idle/inactive UEs.

Whether to configure a separate BWP for multicast

After a UE detects SSB/PBCH, it monitors CORESET#0 and receives SIB. Then, the UE can be aware of initial BWP.

Figure 1. Procedure to obtain information on initial BWP.

In order to support multicast, that initial BWP can be utilized. However, for the purpose of reducing the required bandwidth to be monitored for UEs, the initial BWP may not have large number of PRBs. This may limit the max data rate and scheduling capability of multicast services. Therefore, another BWP can be considered for supporting multicast transmission.

In Figure 2, one example of separate BWP configured for multicast is shown. To allow enough resources for multicast, larger bandwidth is allocated for multicast BWP. Not to have BWP switching between multicast BWP and initial BWP, it is better to have CORESET#0 included in multicast BWP.  Also, frequency resources for DCI, i.e., CORESET, to schedule multicast PDSCH, needs to be configured within a BWP for UEs. To properly decode the PDSCH, the related parameters such as which MCS table is used, and how many layers are used should be configured.

Figure 2. Illustration of initial BWP and multicast BWP in frequency domain

Proposal 1. A separate BWP is configured for multicast by SIB.

Proposal 2. SIB also delivers CORESET for multicast and group-specific RNTI.

Proposal 3: MCS table for multicast and number of layers for multicast are configured to the multicast BWP.

Conclusions

This contribution considered basic aspects to support multicast transmission in NR and provided the following proposals.

Proposal 1. A separate BWP is configured for multicast by SIB.

Proposal 2. SIB also delivers CORESET for multicast and group-specific RNTI.

Proposal 3: MCS table for multicast and number of layers for multicast are configured to the multicast BWP.

- References:

[1] RP‑201038, “Revised Work Item on NR Multicast and Broadcast Services,” June 2020.