3GPP TSG RAN WG1 Meeting #102-e 		 		R1-2006384

e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.10.3

Source: 	LG Electronics

Title: 	Discussion on RACH procedure for IAB enhancement

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss on RACH procedure for IAB enhancement.

- Discussion

In Rel-16 IAB WI, Type-1 random access procedure (a.k.a. 4-step RACH) for IAB has been specified. Especially, PRACH configuration for IAB and additional configuration to indicate longer periodicity (i.e. up to 640ms) were adopted [1]. However, Type-2 random access procedure (a.k.a. 2-step RACH) was not considered for IAB.

In Rel-17, ‘reduction of service interruption time caused by IAB-node migration’ for improving network performance is included as a new function of IAB enhancement. 2-step RACH can be a good solution for achieving the reduction of service interruption time [2]. The simple way for supporting 2-step RACH for IAB is to define an IAB specific new RACH configuration of 2-step RACH in Rel-17.

Proposal: Type-2 random access procedure is adopted for IAB enhancement in Rel-17.

Conclusion

In this contribution, we discuss on RACH procedure for IAB enhancement. To achieve the reduction of service interruption time, we propose to adopt Type-2 random access procedure for IAB enhancement in Rel-17.

Reference

[1] 3GPP TS 38.331 V16.0.0 (2020-3): "NR; Radio Resource Control (RRC) protocol specification"

[2] Qualcomm, “New WID on Enhancements to Integrated Access and Backhaul,” RP-201293, Electronic Meeting, June 29 – July 3, 2020

- Annex: Radio resource control information elements [1]

BWP-UplinkCommon

The IE BWP-UplinkCommon is used to configure the common parameters of an uplink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP-UplinkCommon information element

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters    BWP,

rach-ConfigCommon	SetupRelease { RACH-ConfigCommon }	OPTIONAL,   -- Need M

pusch-ConfigCommon	SetupRelease { PUSCH-ConfigCommon } OPTIONAL,   -- Need M

pucch-ConfigCommon	SetupRelease { PUCCH-ConfigCommon }	OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16	SetupRelease { RACH-ConfigCommonIAB-r16 }	OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16	ENUMERATED	{enabled}	OPTIONAL,   -- Need M

rach-ConfigCommonTwoStepRA-r16	SetupRelease	{ RACH-ConfigCommonTwoStepRA-r16 }	OPTIONAL,   -- Need M

msgA-PUSCH-Config-r16	SetupRelease { MsgA-PUSCH-Config-r16 }	OPTIONAL    -- Need M

]]

}

RACH-ConfigCommonIAB

The IE RACH-ConfigCommonIAB is used to specify the cell specific random-access parameters for IAB-MT.

RACH-ConfigCommonIAB information element

RACH-ConfigCommonIAB-r16 ::=            SEQUENCE {

rach-configIAB-r16	RACH-ConfigCommon	OPTIONAL,   -- Need S

prach-ConfigurationPeriodScaling-r16	ENUMERATED {scf1,scf2,scf4,scf16,scf32,scf64}	OPTIONAL,   -- Need M

prach-ConfigurationFrameOffset-r16	INTEGER (0..63)	OPTIONAL,   -- Need M

prach-ConfigurationSOffset-r16	INTEGER (0..39)	 OPTIONAL,   -- Need M

...

}