3GPP TSG RAN WG1 Meeting #102-e	R1-2006404

E-meeting, August 17-28, 2020

Agenda Item:	8.7.3

Source:	Huawei, HiSilicon

Title:	Other considerations on power saving in Rel-17

Document for:	Discussion and Decision

# Introduction

In this contribution, we share our view on the power saving related work on power saving WI and REDCAP SI. Furthermore, the RAN4 lead topic of RLM and BFD relaxation is also discussed from RAN1 perspective.

# Power saving for REDCAP UEs

In both Rel-17 power saving enhancements WI and REDCAP SI, power saving is one of the objectives. The objective related with power saving in REDCAP SI is as following:

Study UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g. delay tolerant) [RAN2, RAN1]: - Reduced PDCCH monitoring by smaller numbers of blind decodes and CCE limits [RAN1]. - Extended DRX for RRC Inactive and/or Idle [RAN2] - RRM relaxation for stationary devices [RAN2]

However, we need to emphasize that Rel-16 has already introduced power saving mechanisms including WUS indication, cross-slot scheduling based power saving, and BWP-based maximum MIMO layer(s) adaptation. As analyzed in our companion paper, the following Rel-16 features can be utilized by the REDCAP UE:

Table 1 Analysis for R16 power saving techniques

| Power saving techniques | Can be utilized by RedCap UEs | Detailed analysis |
| --- | --- | --- |
| PDCCH based wake-up indication | YES | - When a RedCap UE is in CONNECTED mode, it can be configured with C-DRX to save power. Wake-up indication bit before the ON duration of C-DRX cycle can be used to skip PDCCH monitoring in the C-DRX cycle to save power. - Dormancy indication is not needed considering RedCap UE are not likely to support CA. |
| Cross-slot scheduling | YES | - Cross-slot scheduling based power saving can provide the power saving gain by avoiding unnecessary DL OFDM symbol buffering and by relaxing PDCCH processing. - Cross-slot scheduling may introduce little latency increment. However, RedCap UEs with power saving requirements are usually not latency tolerant. |
| BWP based maximum MIMO layer adaptation | YES | - For 2Rx UE, gNB can switch UE to a BWP with maximum MIMO layer of 1 for power saving purpose. In this case, UE can utilize one single Rx chain for power saving. |
| SCell dormancy like behaviour | NO | - Dormancy like behaviour is a scheme to reduce PDCCH monitoring on SCell(s). - Due to the relatively low data rate requirement, there is no need to support CA in RedCap, thus dormancy like behaviour is not needed by RedCap UEs. |
| RRM relaxation for neighbour cell | YES | - RRM relaxation is already part of the RedCap SID, thus the RRM relaxation schemes in Rel-16 should be utilized as baseline for RedCap. |
| UE assistance information | YES | In Rel-16, the following assistance information are specified: - RRC Release Request, to avoid waiting for RRC release message from gNB for a long time - Preferred C-DRX configuration, to make C-DRX configuration more suitable - Preferred max aggregated Bandwidth, to make BWP parameter more suitable - Preferred max number of MIMO layers, to make MIMO parameter more suitable for UE’s traffic - Preferred K0min/K2min value, to make cross-slot scheduling parameter more suitable It is useful to let UE report necessary information to gNB, thus the above assistance information can be utilized for RedCap UE. |

For wearable devices, which are one important application of RedCap, they are expected to stay in IDLE/INACTIVE time for long time. The wearable devices usually have VoIP, instant message, and heart beat traffic. The power consumption in IDLE/INACTIVE mode would significantly impact the battery life of the wearable device. Furthermore, UE supports the 20MHz bandwidth for FR1, and on the other hand, eMBB in IDLE/INACTIVE state usually just needs to use 20MHz bandwidth to cover the initial DL BWP. Therefore, the power saving for IDLE /INACTIVE mode UE in NR power saving WI is expected to be also utilized by REDCAP UE.

Observation 1: The power saving mechanisms for IDLE/INACTIVE mode UE specified in power saving WI can be utilized by REDCAP UE.

Regarding the connected mode UE, extension of DCI based power saving adaptation, including PDCCH skipping, monitoring periodicity adaptation, would be specified in power saving WI. The Rel-17 connected mode extension of DCI based power saving is also expected to be utilized by REDCAP UE

Observation 2: The Rel-17 extension of DCI based power saving is expected to be utilized by REDCAP UE.

# RLM and BFD relaxation

In RANP #86, it is agreed to further specify enhancements for UE power saving in Rel-17 on the top of Rel-16 schemes [1]. For CONNECTED mode, RLM/BFD can be relaxed.

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4] - NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

In this Section, the power consumption impact due to RLM and BFD are analyzed.

## Power consumption due to RLM and BFR

The UE procedures for RLM are specified in [2], which are summarized as following

- UE receives the RS used for RLM, where the RS can be SSB or CSI-RS.

- For each RS, UE processes it to get the radio link qualities, the metric of which can be RSRP, RSRQ, SNR, and etc. To get the qualities, the samples of the RS within a period of TEvaluate are filtered and processed. Depending on the RS is SSB or CSI-RS, and depending on the quality is used for OOS evaluation or IS evaluation, the value of TEvaluate can be different.

- The qualities of each RS is compared with two thresholds Qout and Qin, where for SSB and CSI-RS the value of Qout and Qin can be different.

- Based on the result of the comparisons of all RS for RLM, PHY layer may send L1 indication to higher layer to indicate OOS or IS. PHY layer get the result of the comparisons once for every TIndication_interval.

Figure 1 The UE procedures for RLM.

The above procedures are also shown in Figure 1, where each block is a sample of SSB or CSI-RS, each red arrow is the timing for PHY layer to get the qualities and compare the qualities with thresholds. According to [2], Tevaluate is usually much longer than Tindication, this relationship is also shwon in Figure 1.

The UE procedures for BFD are similar with those of RLM.

From the above procedures, it can be observed that the power consumption of RLM/BFD is impacted mainly by two parameters, TEvaluate and TIndication_interval. Currently, the values of the two parameters are determined by the configuration of RS and DRX [2].

Observation 3: The power consumption of RLM/BFD is impacted mainly by TEvaluate and TIndication_interval, which is determined by the configuration of the RS and DRX.

If UE can reduce the number of RS samples for RLM/BFD, the power consumption of RLM/BFD can be reduced accordingly. However, just like the RRM relaxation in Rel-16, the power consumption reduction may be with the cost of performance loss. For example, if RLM/BFD is relaxed, the radio link/beam quality may be already very bad, but UE does not realize it in time. So the potential relaxation can only be used under certain condition, and should be under gNB control.

Observation 4: RLM/BFD relaxation may cause performance loss, which should be evaluated when specifying RLM/BFD relaxation.

To evaluate the power saving gain and performance loss when RLM/BFD is relaxed, system level simulation may be needed.

Proposal 1: System level evaluation is needed to evaluate the power saving gain and performance loss when RLM/BFD is relaxed.

# Conclusions

In this contribution, we share our view on the power saving related work on power saving WI and REDCAP SI. The RAN4 lead topic of RLM and BFD relaxation is also discussed from RAN1 perspective. Based on the analysis, we have the following observations and proposals:

Observation 1: The power saving mechanisms for IDLE/INACTIVE mode UE specified in power saving WI can be utilized by REDCAP UE.

Observation 2: The Rel-17 extension of DCI based power saving is expected to be utilized by REDCAP UE.

Observation 3: The power consumption of RLM/BFD is impacted mainly by TEvaluate and TIndication_interval, which is determined by the configuration of the RS and DRX.

Observation 4: RLM/BFD relaxation may cause performance loss, which should be evaluated when specifying RLM/BFD relaxation.

Proposal 1: System level evaluation is needed to evaluate the power saving gain and performance loss when RLM/BFD is relaxed.

# References

RP-193239, “New WID: UE Power Saving Enhancements”, MediaTek

TS 38.133, Requirements for support of radio resource management.