3GPP TSG RAN WG1 #102-e				R1-2006882

e-Meeting, August 17th – 28th, 2020

Source:	WILUS Inc.

Title:	Remaining issues on PDCCH for NR URLLC

Agenda item:	7.2.5.1

Document for:	Discussion/Decision

- Introduction

In this contribution, we discuss remaining issues on PDCCH for NR URLLC.

Note that the issues discussed in this contribution include type-2 HARQ-ACK codebook issue due to configurable counter-DAI field size, which is offloaded to PDCCH agenda from UCI agenda in the last RAN1#101-e meeting.

- Remaining issues on Type-2 HARQ-ACK Codebook

In the last RAN1#101-e meeting, it was agreed to re-interpret the UL DAI per configurable counter-DAI field size in order to determine type-2 HARQ-ACK codebook size when the HARQ-ACKs are multiplexed on PUSCH as follows:

Agreement at RAN#101-e meeting If UE is configured to monitor DCI format 1_2/0_2, the HARQ-ACK codebook size for type-2 HARQ-ACK codebook is determined by

Further, the pseudo-code related to the agreement was also specified in section 9.1.3 of TS38.213 v16.2.0[2]:

9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel […] If the UE transmits HARQ-ACK information in a PUCCH in slot n and for any PUCCH format, the UE determines the , for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code: Set – PDCCH with DCI format scheduling PDSCH reception or SPS PDSCH release monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion Set Set Set Set Set to the number of serving cells configured by higher layers for the UE […] Set to the number of PDCCH monitoring occasion(s) while […] end while if end if if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACK=2⋅TD⋅j+Vtemp2-1mod TD+1 else OACK=TD⋅j+Vtemp2-1mod TD+1 end if for any

In the revised pseudo-code, the type-2 HARQ-ACK codebook size is determined based on Vtemp2 as well as the value of j. Also, the value of j is incremented when Vtemp2 is less than Vtemp (as shown in the yellow part in the pseudo-code). Note that Vtemp2 is coming from UL DAI value, which is one of {1, 2, 3, 4}, and Vtemp is the last counter-DAI value, which is one of {1, 2, … 2TD}, among counter-DAI values included in received PDCCHs. The range of two values such as Vtemp and Vtemp2 is not aligned due to configurable counter-DAI field size, TD. Therefore, it is necessary to align the range of two values by comparing the two values in the pseudo-code. Similarly, as in the modifications by the agreement at RAN1#101-e meeting, since the range of Vtemp2 can be re-interpreted to that of Vtemp, the comparison should be performed by the re-interpreted value of Vtemp2, i.e., Vtemp2-1mod TD+1.

  - Proposal 1: In case of HARQ-ACK multiplexing on PUSCH, the value of j is determined by comparing between the re-interpreted value of Vtemp2, i.e., Vtemp2-1mod TD+1, and Vtemp.

  - Proposal 2: Adopt the following text proposal for TS38.213

9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel =============================Unchanged part is omitted=================================== If the UE transmits HARQ-ACK information in a PUCCH in slot n and for any PUCCH format, the UE determines the , for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code: Set – PDCCH with DCI format scheduling PDSCH reception or SPS PDSCH release monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion Set Set Set Set Set to the number of serving cells configured by higher layers for the UE - if, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the serving cell is counted two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs - if the UE indicates PDSCH-Number-perMOperCell, a serving cell is counted NPDSCHMO times where NPDSCHMO is the number of PDSCH receptions that can be scheduled for the serving cell by DCI formats in PDCCH receptions at a same PDCCH monitoring occasion based on the reported value of PDSCH-Number-perMOperCell Set to the number of PDCCH monitoring occasion(s) while Set – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while if PDCCH monitoring occasion is before an active DL BWP change on serving cell or an active UL BWP change on the PCell and an active DL BWP change is not triggered in PDCCH monitoring occasion ; else if there is a PDSCH on serving cell associated with PDCCH in PDCCH monitoring occasion , or there is a PDCCH indicating SPS PDSCH release on serving cell if end if if else end if if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell, o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1 elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell, oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 else oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 end if end if end if end while end while if Vtemp2-1mod TD+1<Vtemp end if if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACK=2⋅TD⋅j+Vtemp2-1mod TD+1 else OACK=TD⋅j+Vtemp2-1mod TD+1 end if for any =============================Unchanged part is omitted===================================

- Conclusion

In this contribution, remaining issues on type-2 HARQ-ACK codebook were discussed, and the following was proposed.

  - Proposal 1: In case of HARQ-ACK multiplexing on PUSCH, the value of j is determined by comparing between the re-interpreted value of Vtemp2, i.e. Vtemp2-1mod TD+1, and Vtemp.

  - Proposal 2: Adopt the following text proposal for TS38.213

9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel =============================Unchanged part is omitted=================================== If the UE transmits HARQ-ACK information in a PUCCH in slot n and for any PUCCH format, the UE determines the , for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code: Set – PDCCH with DCI format scheduling PDSCH reception or SPS PDSCH release monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion Set Set Set Set Set to the number of serving cells configured by higher layers for the UE - if, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the serving cell is counted two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs - if the UE indicates PDSCH-Number-perMOperCell, a serving cell is counted NPDSCHMO times where NPDSCHMO is the number of PDSCH receptions that can be scheduled for the serving cell by DCI formats in PDCCH receptions at a same PDCCH monitoring occasion based on the reported value of PDSCH-Number-perMOperCell Set to the number of PDCCH monitoring occasion(s) while Set – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while if PDCCH monitoring occasion is before an active DL BWP change on serving cell or an active UL BWP change on the PCell and an active DL BWP change is not triggered in PDCCH monitoring occasion ; else if there is a PDSCH on serving cell associated with PDCCH in PDCCH monitoring occasion , or there is a PDCCH indicating SPS PDSCH release on serving cell if end if if else end if if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell, o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1 elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell, oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 else oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 end if end if end if end while end while if Vtemp2-1mod TD+1<Vtemp end if if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACK=2⋅TD⋅j+Vtemp2-1mod TD+1 else OACK=TD⋅j+Vtemp2-1mod TD+1 end if for any =============================Unchanged part is omitted===================================

References

- Draft Report of 3GPP TSG RAN WG1 #101e v0.2.0

- 3GPP TS38.213 v16.2.0