3GPP TSG RAN WG1 #102-e	R1-2007218

e-Meeting, August 17th – 28th, 2020

Source:	moderator (vivo)

Title:	Text proposal for TS 38.214 based on [102-e-NR-L1enh-URLLC-eCG-01]

Agenda Item:	7.2.5.6

Document for:	Discussion and Decision

Introduction

The document captures text proposal agreed from email discussion [102-e-NR-L1enh-URLLC-eCG-01].

Reasons for change

It was clarified that for CG PUSCH with repetitions, the determination of its initial transmission occasion is not affected by UL CI, dynamic SFI and intra-UE multiplexing/prioritization.

Summary of changes

Adding Clause 9 for intra-UE cancellation, and 11.2A for inter-UE cancellation, to clarify that the cancellation occurs after the determination the initial transmission occasion for CG repetitions described in 6.1.2.3.1.

Consequences if not approved

gNB and UE may have different understanding about the initial transmission occasion for CG PUSCH.

Text Proposal

---------------------------------------- Start of Text Proposal for TS 38.214 --------------------------------------

#### 6.1.2.1	Resource allocation in time domain

<Unchanged parts are omitted>

When transmitting MsgA PUSCH, if the UE is not configured with startSymbolAndLengthMsgA-PO, and if the TDRA list PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon, the UE shall use msgA-PUSCH-TimeDomainAllocation to indicate which values are used in the list. If PUSCH-TimeDomainResourceAllocationList is not provided in PUSCH-ConfigCommon, the UE shall use parameters S and L from table 6.1.2.1.1-2 where msgA-PUSCH-TimeDomainAllocation indicates which values are used in the list. The time offset for PUSCH transmission is described in [6, TS38.213].

For PUSCH repetition Type A, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in in Clause 9, Clause 11.1, Clause 11.2A of [6, TS38.213].

<Unchanged parts are omitted>

For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A of [6, TS38.213].

<Unchanged parts are omitted>

For PUSCH repetition Type B, when a UE receives a DCI that schedules aperiodic CSI report(s) or activates semi-persistent CSI report(s) on PUSCH with no transport block by a CSI request field on a DCI, the number of nominal repetitions is always assumed to be 1, regardless of the value of numberofrepetitions. When the UE is scheduled to transmit a PUSCH repetition Type B with no transport block and with aperiodic or semi-persistent CSI report(s) by a CSI request field on a DCI, the first nominal repetition is expected to be the same as the first actual repetition. For PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a CSI request field on a DCI, if the first nominal repetition is not the same as the first actual repetition, the first nominal repetition is omitted; otherwise, the first nominal repetition is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A of [6, TS38.213].

<Unchanged parts are omitted>

6.1.2.3.1	Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant

<Unchanged parts are omitted>

A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A of [6, TS38.213].

<Unchanged parts are omitted>

----------------------------------------End of Text Proposal for TS 38.214 --------------------------------------