3GPP TSG RAN WG1 #102	DRAFT R1-20xxxx

e-Meeting, August 17th – 28th, 2020

Agenda item:	7.2.10

Source:	Moderator (Nokia)

Title:	FL summary on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

Document for:	Discussion and Decision

# 1	Introduction

This contribution is a pre-meeting summary of the documents submitted for the AI7.2.10 that discussed the aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH. One document was identified with proposals related to the functionality [1].

# 2	Proposal 3, section 2.2 [1]

Section 2.2 of [1] makes the following point:

The following agreement was achieved last meeting. A new RRC parameter was added to control the newly defined beam switching behavior in Rel-16.

Agreements: - Introduce a new capability signaling for aperiodic CSI-RS triggering with beam switching timing (FG14-7 in UE features list) - Followings are clarified for FG14-7 (beamSwitchTiming-r16) in the UE features list - Candidate values of beamSwitchTiming-r16 include {224, 336} - An RRC configuration parameter is added to indicate the UE behavior for AP-CSI-RS beam switching in Rel-16 - When provided, the UE behavior agreed in Rel-16 TEI is performed, with beamSwitchTiming-r16 as input - Otherwise, the UE behavior specified in Rel-15 is performed, with beamSwitchTiming as input - Adopt following TP to TS38.214

However, above behavior is only clarified for the same numerology case. For X-numerology CSI-RS triggering the related UE behavior should also be updated based on the latest agreement.

Moreover, the RRC parameter used in current TS 38.213 (aperiodicTriggeringOffsetExt-r16) is not aligned with that defined in TS 38.331: aperiodicTriggeringOffset-r16, which is updated together in the following TP.

[detailed TP to TS38.214 provided in [1] copied in the Annex]

Proposal 3: Agree the TP above for clarification on default beam switching behavior and alignement with RRC spec.

# 3	FL proposal on the scope of the RAN1#102

Feature lead proposal: Consider section 2.2 and proposal 3 of [1] and the relatd TP to TS38.214

Comments on the FL proposal

| Company | Comments |
| --- | --- |

# References

- R1-2005360 Remaining issues on MR-DC, vivo

# Annex – TP to TS38.214, section 2.2 of [1]

##### 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology <Unchanged part ommited> When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16 aperiodicTriggeringOffset-r16. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} slots. If the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP or minimumSchedulingOffsetK2 for any UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states , the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement. <Unchanged part ommited> ##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions: Beam switch timing: - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided,, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided, where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero - if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD', - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS; - else, - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - else if the UE is configured with [enableDefaultBeamForCCS], when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, when the reported value is one of the values of {14,28,48} and enableBeamSwitchTiming-r16 is not provided, or is equal to or greater than 48+d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. Table 5.2.1.5.1a-1: Additional beam switching timing delay d µPDCCH; d [PDCCH symbols]. 0; 8. 1; 8. 2; 14. Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16 aperiodicTriggeringOffset-r16, including the case that the UE is not configured with minimumSchedulingOffsetK0-r16 for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1, …, 31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where - n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16 aperiodicTriggeringOffset-r16, - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively, - Nslot, offset, PDCCHCA and μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH respectively, Nslot, offset, CSIRSCA and μoffset,CSIRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] clause 4.5 - If the µPDCCH < µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than the first symbol of the CSI-RS carrier's slot that starts at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS. - If the µPDCCH > µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS. Table 5.2.1.5.1a: Ncsirs as a function of the subcarrier spacing of the triggering PDCCH µPDCCH; Ncsirs [symbols]. 0; 4. 1; 5. 2; 10. 3; [14]. <Unchanged part ommited>