3GPP TSG RAN WG1 #102-e			R1-200xxxx

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.1

Source: 	Moderator (Samsung)

Title: 	Revised moderator summary for Rel.16 NR eMIMO maintenance

Document for:	Discussion and Decision

- Introduction

The moderator summary of the maintenance-related issues raised in the submitted contributions for Rel.16 NR_eMIMO maintenance is given below. The listed maintenance issues are under the usual designations:

- LP: low-PAPR RS

- MB: Multi-beam operation

- MT: Multi-TRP

- MU: Type-II enhancement for MU-CSI

- UL: UL full power transmission

An initial assessment on each of the issues is given (but can be revised based on the outcome of the discussion during the preparation week). The assessment will be used as a basis to select six issues (per chairman instruction) for further discussion in the upcoming weeks.

- High priority (H): this includes high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

  - H2: The proposal can be endorsed without discussion in the upcoming weeks (i.e. unless pointed out otherwise, the moderator will propose to the chair that the proposal be endorsed by Aug 17th thereby not counted toward the six-thread quota)

- Non-essential (N): this includes all other purposes such as spec optimization and low priority issues

- Maintenance issues

The issues are summarized in the following table:

Table 1 Summary of issues

| # | Issue (summary) | Companies | Initial assessment |
| --- | --- | --- | --- |
| LP.1 | When Pi/2 BPSK is used for the PUSCH transmission, i.e., DMRSuplink-r16 in the DMRS-UplinkConfig IE is provided, different DMRS sequence is generated on different DMRS ports associated with different CDM group λ. The specification in clause 6.4.1.2.1.1 of TS 38.211 does not clearly specify for a PTRS port or is based on which CDM group. | Qualcomm | N |
| MB.1 | Specify the pathloss RS for PUSCH scheduled by DCI format 0_2 Note: Missing piece to enable the default PLRS for PUSCH scheduled by DCI 0_2 | OPPO | H |
| MB.2 | Support enabling default PL RS in FR1 Note: Proposed in multiple meetings but couldn’t have chance to be discussed | Huawei/HiSi | H |
| MB.3 | [Editorial] Align the RRC parameter names with the latest TS 38.331. TP2-TP8 from ZTE Note: Obviously needed, not controversial | ZTE | H2 |
| MB.4 | Support the feature of simultaneous spatial relation update across multiple BWPs/CCs when default spatial relation for SRS is enabled Note: Unresolved issue from the last meeting, priority is unclear | Support: Vivo, Sony. Not support: LGE | N |
| MB.5 | [Editorial] Clarify that RRC can provide two CC lists and the applied CC list is determined by the MAC-CE. TP1 from ZTE Note: The current spec seems clear | ZTE | N |
| MB.6 | Exclude multi-TRP CC from the simultaneous TCI state activation Note: Motivation seems unclear | OPPO | N |
| MB.7 | Support the feature of default spatial relation/PL RS for multi-DCI based MTRP Note: late optimization for Rel-16, many impacts on spec as well as UE features | Apple | N |
| MB.8 | Clarification on measurement restriction Define the measurement restriction for L1-SINR, where - For two report settings, timeRestrictionForChannelMeasurements defines the measurement restriction for CMR, and timeRestrictionForInterferenceMeasurements defines the measurement restriction for dedicated IMR. For one report setting, timeRestrictionForChannelMeasurements defines the measurement restriction for CMR, and timeRestrictionForInterferenceMeasurements defines the measurement restriction for interference measurement based on the same resources for CMR Note: Based on previous conclusion and should not take too much effort, also have impact on RAN4’s work | vivo | H FUTUREWEI: we assume here “report setting” should be “resource setting”. Ok to discuss but assume the behavior should be the same for other type of CSI report. |
| MB.9 | QCL assumption for CMR without QCL indication One of the following changes can avoid the ambiguity. - UE considers the case as an error case - One way to clarify the ambiguity is considering it as an error case. - Extending the current UE behavior - According to the current specification, when a UE is configured with two Resource Settings for L1-SINR, the UE can apply same QCL-TypeD assumption for CMR resource and the corresponding IMR resource. We may extend the current UE behavior to this case. Note: UE behavior is unclear if QCL-TypeD for CSI-RS is not provided for both CMR and IMR, but this looks to be a general issue for P-CSI-RS | Interdigital, Huawei/HiSi, OPPO | H FUTUREWEI: This is related to the RAN4 LS R1-2006952 on scenarios of P1 CSI-RS without QCL. Also not clear whether the issue is only for QCL-TypeD or more general |
| MB.10 | Various editorial issues (TP): See Appendix A Note: Obviously needed, not controversial | Apple. MTK, Sony, ZTE, APT | H2 |
| MB.11 | L1-SINR report without dedicated IMR For CMR without IMR, define the interference should be measured based on total received power on dedicated CSI-RS resource for channel measurement excluding the power of the NZP CSI-RS corresponds to interference and noise. Note: There should be legacy UE behavior for SINR measured with CMR only. In Rel-15, SS-SINR/CSI-SINR is defined with CMR only. | MTK | N |
| MB.12 | QCL for IMR: The agreed behavior that the QCL for IMR is based on that of CMR should exclude the case when aperiodic L1-SINR is triggered and the report setting is associated with periodic/semi-persistent CSI-RS Note: According to previous agreement, gNB should avoid the case that one CSI-RS would be with two different QCL assumption | vivo | N |
| MB.13 | NZP+ZP IMR: There are two alternatives: - Alt1: Add the restriction that all CMR should be QCLed w.r.t ‘QCL-TypeD’ if both NZP IMRs and ZP IMR are configured. - Alt2: Remove NZP+ZP based interference measurement Note: Already discussed for a few meetings without much consensus | Huawei/HiSi, LGE | N |
| MB.14 | Scheduling restriction for NZP IMR: In L1-SINR measurement, if the CMR is associated with a NZP-CSI-RS-ResourceSet with parameter repetition set to 'on', both the CMR and the associated IMR cannot be configured over the symbols during which the UE is also configured to monitor the CORESET. Note: According to some comments in last meeting, this seems to be a RAN4 issue. | Huawei/HiSi | N |
| MB.15 | EPRE for L1-SINR: When L1-RSRP is measured by NZP CSI-RS, UE should not compensate the L1-RSRP by the associated EPRE parameters. Note: Out of scope | Qualcomm | N |
| MB.16 | BWP for CORESET #0 beam reset by RACH: In case of CORESET 0 beam updated by RACH, the updated beam is applied to CORESET 0 in all BWPs Note: Out of scope | Qualcomm | N |
| MB.17 | PUCCH/PDCCH Beam after CBRA-BFR for PCell/PSCell BFR - During contention-based BFR, if Msg.3 or Msg.A of CBRA-BFR contains BFR MAC CE, the UE shall transmit PUCCH using the spatial filter which is used for the PRACH transmission, until reconfigured/reactivated by the gNB. - During contention-based BFR, if Msg.3 or Msg.A of CBRA-BFR contains BFR MAC CE, the UE shall monitor PDCCH in all CORESETs using QCL assumption of SSB index selected for the PRACH transmission. Note: Out of scope | Docomo, Ericsson, Nokia/NSB, Qualcomm | N |
| MB.18 | BFD RS monitoring behavior: If explicit BFD is configured before SCell fails, UE should stop monitoring the previously configured explicit BFD RS(s) after receiving the response for the step-2 MAC-CE - UE can start implicit BFD after receiving TCI activation/reconfiguration for failed SCell Note: RAN2 issue per comments from several companies in the last meeting | Qualcomm | N |
| MB.19 | Clarification of SSB for BFD: Clarify whether SS/PBCH blocks can be provided for BFD Note: According to a previous agreement, SSB cannot be used for SCell BFD. Rel-15 CR seems to be a better place to clarify SSB for PCell/PSCell BFD | CATT | N |
| MT.1 | Closed-loop Power control for multi-DCI based related with Out-of-order operation + QCL-TypeD for PDCCH in Multi-DCI based transmission Note: Uplink out-of-order cannot be supported due to the note added in FG 16-2a-3 and the current specification on closed loop power control index. | ZTE, Intel, Ericsson, Qualcomm, Nokia/NSB, OPPO | H FUTUREWEI: it is not clear what exact the issue intended here. It may help to list the related contributions. |
| MT.2 | PDSCH processing time for URLLCScheme 3 Note: missing for scheme 3 in the spec | Huawei/HiSi, Qualcomm | H |
| MT.3 | Various editorial changes (TP): see Appendix B Note: still need discussion | Vivo, Spreadtrum, Sharp, ZTE, OPPO, CATT, LGE, Huawei/HiSi | H |
| MT.4 | Default QCL for AP CSI-RS in multi-DCI based M-TRP Note: discussed in previous meeting, no conclusion, can be considered optimization | vivo, ZTE, Apple, NTT DOCOMO, Qualcomm, Nokia | N |
| MT.5 | Default TCI-state for PDSCH of cross-carrier scheduling in multi-DCI based M-TRP Note: optimization | vivo, Qualcomm | N |
| MT.6 | CSI/SR UCI overlapping with two HARQ-ACK PUCCH /PUSCH of two different TRPs Note: can be resolved via implementation | Vivo | N |
| MT.7 | Sub-slot based HARQ-ACK feedback in multi-DCI based multi-TRP Note: current spec is clear | vivo | N |
| MT.8 | PDSCH overlapping with PDCCH from different TRP Note: current spec is clear | vivo | N |
| MT.9 | Active BWP operation in multi-DCI based M-TRP system Note: optimization | ZTE, Lenovo/MotM, NTT DOCOMO | N |
| MT.10 | SPS transmission in multi-DCI based M-TRP Note: optimization | Samsung, Qualcomm | N |
| MT.11 | PDCCH BD/CCEs in Multi-DCI based system Note: can be resolved via implementation | Spreadtrum, Qualcomm | N |
| MT.12 | Radio link monitoring in multi-DCI based M-TRP Note: optimization | Apple, NTT DOCOMO | N |
| MT.13 | Collision between QCL-typeD of PDCCH and default QCL of PDSCH Note: optimization | Apple | N |
| MT.14 | Out-of-order operation of DL shall be allowed within a slot Note: optimization | CATT | N |
| MT.15 | Default TCI-state for PDSCH in Single-DCI based transmission Note: discussed in previous meeting, no conclusion | Vivo, ZTE, LGE | N |
| MT.16 | Default QCL for AP CSI-RS in single-DCI based M-TRP Note: discussed in previous meeting, no conclusion | vivo, ZTE, Apple, Ericsson, NTT DOCOMO, Qualcomm, Nokia/NSB | N |
| MT.17 | Default TCI-state for PDSCH of cross-carrier scheduling in single-DCI M-TRP Note: optimization | Vivo, Samsung | N |
| MT.18 | Determining RV values for Scheme 4 Note: current spec is clear | ZTE, CATT | N |
| MT.19 | 3 CDM groups vs 2 TCI states in Single-DCI based multi-TRP Note: Current specification suggests that when 2 TCI states are indicated, 3 CDM groups cannot be indicated. | Apple | N |
| MT.20 | SPS transmission in Single-DCI based multi-TRP Note: optimization | Ericsson and Qualcomm | N |
| MT.21 | Time restriction on S and L for TDM scheme A Note: current spec is clear | Ericsson | N |
| MT.22 | Type-1 HARQ-ACK codebook determination for Scheme 3 Note: current spec is clear | NTT DOCOMO, Nokia/NSB | N |
| MU.1 | Two clarifications for TS38.214 5.2.2.2.5 Enhanced Type II Codebook <Unchanged part omitted> The parameter R is configured with the higher-layer parameter numberOfPMISubbandsPerCQISubband-r16. This parameter controls the total number of precoding matrices N3 indicated by the PMI as a function of the number of configured subbands in csi-ReportingBand,... Add the text “(if reported)” after the FD basis indicator, i1,6,l, in the description of Group 1 in Sec. 5.2.3 to include the case in which this indicator is not reported, for Mν=1. Note: Obviously needed, not controversial | Nokia/NSB, Spreadtrum, vivo | H2 |
| MU.2 | Refinement on CBSR equation Note: previously proposed for a few meetings | MotM/Lenovo | N |
| MU.3 | When N3>19, the size of the intermediate set is give by N3'=2M1 for RI={1,2,3,4}, where M1 is the number of FD bases selected for RI={1,2}. Note: previously proposed for a few meetings | Qualcomm | N |
| UL.1 | Finalizing TPMI grouping indication for Mode 2 Alt1. Add new TPMI group(s) Alt2. No change in TPMI group, and UE can report >1 TPMI groups Alt3. Revise at least one TPMI group Note: discussed over several meetings, but needed for RAN2 to proceed | Alt1. Huawei/HiSi. ZTE, vivo, OPPO, Apple, LGE, Samsung (2nd pref) Alt2. Samsung (1st pref), OPPO, Apple Alt3. Qualcomm | H |
| UL.2 | 2-port PTRS for mode 1 6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled … - PUSCH antenna port 1000 and 1002 in indicated TPMI share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI share PT-RS port 1 except for the cases that ul-FullPowerTransmission is configured to fullpowerMode1, and TPMI=2 in Table 6.3.1.5-1, or one of the TPMI 12-15 in Table 6.3.1.5-2 and Table 6.3.1.5-3 in [4, TS 38.211] is indicated. - UL PT-RS port 0 is associated with the UL layer [x] of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI, and UL PT-RS port 1 is associated with the UL layer [y] of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI, where [x] and/or [y] are given by DCI parameter PTRS-DMRS association as shown in DCI format 0_1 described in Clause 7.3.1 of [5, TS38.212]. - For the cases that ul-FullPowerTransmission is configured to fullpowerMode1, and TPMI=2 in Table 6.3.1.5-1, or one of the TPMI 12-15 in Table 6.3.1.5-2 and Table 6.3.1.5-3 in [4, TS 38.211] is indicated, PUSCH antenna port 1000, 1001, 1002 and 1003 in the indicated TPMI share PT-RS port 0. | Huawei/HiSi | H |
| UL.3 | Align the RRC parameter ul-FullPowerTransmission-r16 across specifications TS 38.212, 213, 214 Note: Obviously needed, not controversial | Spreadtrum | H2 |
| UL.4 | Support for single-port SRS in mode 1 Alt1 (OPPO). Add “if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has one SRS port, s=1” Alt2 (Spreadtrum). Add “A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission-r16 set to ‘fullpowerMode1’ and SRS-ResourceSet with usage set to ‘codebook’ with single port SRS resource simultaneously.” Note: Discussed last meeting, common understanding seems to indicate that it’s not needed | Spreadtrum, OPPO | N |
| UL.5 | Clarification on port coherence for 2Tx and 4Tx partially coherent UE Proposal: - As a first priority, specify that a UE configured for partially coherent operation in full power Mode 1 is not expected to maintain relative phase between ports 0 and 2 nor between ports 1 and 3 - As a second priority, specify that a UE configured for noncoherent operation in full power Mode 1 is not expected to maintain phase coherence among any of its ports. Note: previously proposed common understanding seems to indicate that it’s not needed | Ericsson | N |
| UL.6 | Clarification on 2Tx codebook subset for 2-port SRS (TS 38.214) 6.1.1.1 Codebook based UL transmission <Unchanged part omitted> A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by codebookSubset with 'fullyAndPartialAndNonCoherent' and when higher layer parameter nrofSRS-Ports in an SRS-ResourceSet with usage set to 'codebook' indicates that two SRS antenna ports are configured, the UE shall not expect to be configured by codebookSubset with 'fullyAndPartialAndNonCoherent'. A UE reporting its UE capability of 'nonCoherent' transmission shall not expect to be configured by codebookSubset with 'fullyAndPartialAndNonCoherent' or with 'partialAndNonCoherent'. A UE shall not expect to be configured with the higher layer parameter codebookSubset set to 'partialAndNonCoherent' when higher layer parameter nrofSRS-Ports in an SRS-ResourceSet with usage set to 'codebook' indicates that two SRS antenna ports are configured. Note: Strictly speaking, not ULFPTX issue, the need is unclear (redundant relative to the next two sentences) | vivo | N |

- Conclusion

The above summary will be used as a starting point of the discussion during the preparation week to select up to six (per chairman instruction) email threads for the coming weeks.

# Appendix A: TP for MB.10

TP for 38.212

##### 6.3.2.1.2 CSI <unchanged part omitted> The mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for CRI/SINR or SSBRI/SINR reporting is provided in Table 6.3.1.1.2-8A.The procedure in clause 6.3.2 described for CSI part 1 is also applicable for one report for CRI/RSRP, SSBRI/RSRP, CRI/SINR, or SSBRI/SINR reporting. Table 6.3.2.1.2-3: Mapping order of CSI fields of one CSI report, CSI part 1 CSI report number; CSI fields. CSI report #n CSI part 1; CRI as in Tables 6.3.1.1.2-3/4/6, if reported. Rank Indicator as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8, if reported. Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8, if reported. Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8, if reported. Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported. Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported. Indicator of the total number of non-zero coefficients summed across all layers KNZ as in Table 6.3.2.1.2-8, if reported. Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0.. <unchanged part omitted> Table 6.3.2.1.2-6: Mapping order of CSI reports to UCI bit sequence , with two-part CSI report(s) UCI bit sequence; CSI report number. CSI part 1 of CSI report #1 as in Table 6.3.2.1.2-3 or Table 6.3.1.1.2-8 or Table 6.3.1.1.2-8A. CSI part 1 of CSI report #2 as in Table 6.3.2.1.2-3 or Table 6.3.1.1.2-8 or Table 6.3.1.1.2-8A. …. CSI part 1 of CSI report #n as in Table 6.3.2.1.2-3 or Table 6.3.1.1.2-8 or Table 6.3.1.1.2-8A. <unchanged part omitted>

TP for 38.213

6 Link Reconfiguration A UE can be provided, for each BWP of a serving cell, a set of periodic CSI-RS resource configuration indexes by failureDetectionResources and a set of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt-r16 or candidateBeamRSSCellList-r16 for radio link quality measurements on the BWP of the serving cell. If the UE is not provided by failureDetectionResources for a BWP of the serving cell, the UE determines the set to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH and, if there are two RS indexes in a TCI state, the set includes RS indexes with QCL-TypeD configuration for the corresponding TCI states. The UE expects the set to include up to two RS indexes. The UE expects single port RS in the set . The UE expects single-port or two-port CSI-RS with frequency density equal to 1 or 3 REs per RB in the set .

TS 38.213 Section 9.2.4

-------- unchanged part omitted ---------------

A UE can be configured by SchedulingRequestResourceConfig a set of configurations for SR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1. A UE can be configured by schedulingRequestID-BFR-SCell-r16 a configuration for LRR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1. The UE can be provided, by phy-PriorityIndex-r16 in SchedulingRequestResourceConfig, a priority index 0 or a priority index 1 for the SR. If the UE is not provided a priority index for SR, the priority index is 0.

The UE is configured a PUCCH resource by SchedulingRequestResourceId, or by schedulingRequestID-BFR-SCell-r16, providing a PUCCH format 0 resource or a PUCCH format 1 resource as described in Clause 9.2.1. The UE is also configured a periodicity  in symbols or slots and an offset  in slots by periodicityAndOffset for a PUCCH transmission conveying SR. If  is larger than one slot, the UE determines a SR transmission occasion in a PUCCH to be in a slot with number  [4, TS 38.211] in a frame with number  if .

-------- unchanged part omitted ---------------

TS 38.213 Section 9.2.5.1 UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH

In the following, a UE is configured to transmit  PUCCHs for respective  SRs in a slot, as determined by a set of schedulingRequestResourceId and a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell-r16, with SR transmission occasions that would overlap with a transmission of a PUCCH with HARQ-ACK information from the UE in the slot or with a transmission of a PUCCH with CSI report(s) from the UE in the slot.

-------- unchanged part omitted ---------------

If a UE would transmit a PUCCH with  HARQ-ACK information bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, as described in Clauses 9.2.1 and 9.2.3,  bits representing a negative or positive SR, in ascending order of the values of schedulingRequestResourceId and a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell-r16, are appended to the HARQ-ACK information bits and the UE transmits the combined  UCI bits in a PUCCH using a resource with PUCCH format 2 or PUCCH format 3 or PUCCH format 4 that the UE determines as described in Clauses 9.2.1 and 9.2.3. If one of the SRs is a positive LRR, the value of the  bits indicates the positive LRR. An all-zero value for the  bits represents a negative SR value across all  SRs.

If a UE would transmit a PUCCH with  CSI report bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot,  bits representing corresponding negative or positive SR, in ascending order of the values of schedulingRequestResourceId and a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell-r16, are prepended to the CSI information bits as described in Clause 9.2.5.2 and the UE transmits a PUCCH with the combined  UCI bits in a resource using the PUCCH format 2 or PUCCH format 3 or PUCCH format 4 for CSI reporting. If one of the SRs is a positive LRR, the value of the  bits indicates the positive LRR. An all-zero value for the  bits represents a negative SR value across all  SRs.

TP for 38.214

< Start of the text proposal on TS 38.214 v16.2.0 section 5.6.1.6> #### < Unchanged parts are omitted > #### 5.1.6.1 CSI-RS reception procedure #### The CSI-RS defined in Clause 7.4.1.5 of [4, TS 38.211], may be used for time/frequency tracking, CSI computation, L1-RSRP computation, L1-SINR computation and mobility. < Unchanged parts are omitted > < End of the text proposal on TS 38.214 v16.2.0 section 5.6.1.6>

5.2.1.4.1 Resource Setting configuration < Unchanged parts are omitted > For semi-persistent or periodic CSI, each CSI-ReportConfig is linked to periodic or semi-persistent Resource Setting(s): - When one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is configured, the Resource Setting is for channel measurement for L1-RSRP or for channel and interference measurement for L1-SINR computation. - When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference) is used for interference measurement performed on CSI-IM. For L1-SINR computation, the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourceForInterference) is used for interference measurement performed on CSI-IM or on NZP CSI-RS. < Unchanged parts are omitted >

TP for 38.215

5.1.6 CSI signal-to-noise and interference ratio (CSI-SINR) < Unchanged parts are omitted > For CSI-SINR determination CSI reference signals transmitted on antenna port 3000 according to TS 38.211 [4] shall be used. If CSI-SINR is used for L1-SINR, CSI reference signals transmitted on ports 3000, 3001 can be used for CSI-SINR determination. < Unchanged parts are omitted >

# Appendix B: TP for MT.3

| MT.3 index | Summary of TP | Proposing companies |
| --- | --- | --- |
| MT.3.1 | Align the RRC parameter names | Vivo, Spreadtrum, Sharp |
| MT.3.2 | Default QCL for DCI format 1_2 | vivo |
| MT.3.3 | TCI states for inter-slot PDSCH repetition | vivo |
| MT.3.4 | Clarify UL transmission from different TRP shall be TDMed | ZTE |
| MT.3.5 | Type 1 HARQ codebook | ZTE |
| MT.3.6 | TBS determination for Scheme 3 | ZTE |
| MT.3.7 | Wideband PRG for Scheme 2a/2b | ZTE |
| MT.3.8 | A missing condition for two default TCI state for Multi-DCI based system in TS 38.214 | OPPO |
| MT.3.9 | One part in the agreement made in RAN1#101e was not captured in TP | OPPO |
| MT.3.10 | TP to capture the case “is not provided with CORESETPoolIndex” in TS 38.213 | Spreadtrum |
| MT.3.11 | One Typo correction | CATT |
| MT.3.12 | TP for the case “startingSymbolOffset is not configured” | LGE |
| MT.3.13 | determine nHARQ-ACK for PUCCH transmission in M-DCI M-TRP | Huawei/HiSi |

# References

- R1-2005353	Remaining issues on Multi beam operation	vivo

- R1-2005354	Remaining issues on Multi TRP operation	vivo

- R1-2005355	Miscellaneous corrections on Rel-16  MIMO	vivo

- R1-2005449	Maintenance of multi-beam operation	ZTE

- R1-2005450	Maintenance of full power UL transmission	ZTE

- R1-2005451	Maintenance of Multi-TRP enhancements	ZTE

- R1-2005554	Remaining issues on NR MIMO	Interdigital Inc.

- R1-2005559	Remaining issues on multi-beam operation	Sony

- R1-2005624	Remaining issues on multi-beam operation	Mediatek Inc.

- R1-2005679	Remaining issues on multi-beam operation enhancement	CATT

- R1-2006588	Discussion on remaining issues of multi-TRP/panel transmission	CATT

- R1-2005819	Maintenance on multi-TRP operation	Lenovo, Motorola Mobility

- R1-2005924	Maintenance on MU-CSI Enhancements	Lenovo, Motorola Mobility

- R1-2005853	Corrections to multi TRP	Intel Corporation

- R1-2005975	Text proposals for enhancements on multi-TRP and panel Transmission	OPPO

- R1-2005976	Text proposals for Multi-beam Operation Enhancement	OPPO

- R1-2005977	Discussion and Text proposals for full TX power UL transmission	OPPO

- R1-2006115	On maintenance of Rel.16 MU CSI enhancements and multi-beam operation	Samsung

- R1-2006117	On Rel.16 multi-TRP/panel transmission		Samsung

- R1-2006118	On UL full power transmission	Samsung

- R1-2006249	Remaining issues on CSI enhancement for MU-MIMO	Spreadtrum Communications

- R1-2006256	Discussion on remaining issues on UL full power transmission		Spreadtrum Communications

- R1-2006257	Discussion on remaining issues for multi-TRP operation	Spreadtrum Communications

- R1-2006395	Remaining issues for Multi-TRP in Rel-16	Huawei, HiSilicon

- R1-2006396	Remaining issues on multi-beam enhancements in R16	Huawei, HiSilicon

- R1-2006397	Remaining issues on UL full power transmission in R16	Huawei, HiSilicon

- R1-2006494	Remaining issues on Multi-TRP enhancement	Apple

- R1-2006495	Remaining issues on Beam Management	Apple

- R1-2006496	Remaining issues on Rel-16 full power transmission	Apple

- R1-2006551	Corrections for enhancements on MIMO for NR	Sharp

- R1-2006593	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

- R1-2006594	Remaining issues on multi beam operation	LG Electronics

- R1-2006595	Discussions on full Tx power UL transmission	LG Electronics

- R1-2006608	Corrections for Full Power UL Transmission		Ericsson

- R1-2006688	Remaining issues on single-DCI based Multi-TRP	Ericsson

- R1-2006689	Remaining issues on multi-DCI based Multi-TRP	Ericsson

- R1-2006635	Maintenance of SCell Beam Failure Recovery	Asia Pacific Telecom co. Ltd

- R1-2006700	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC.

- R1-2006701	PUCCH/PDCCH beam after CBRA-BFR in Rel.16	NTT DOCOMO, INC., Nokia, Nokia Shanghai Bell, Ericsson

- R1-2006780	Remaining issues on MU-CSI enhancement, UL full power, and low PAPR RS	Qualcomm Incorporated

- R1-2006781	Multi-TRP Enhancements	Qualcomm Incorporated

- R1-2006782	Enhancements on Multi-beam Operation		Qualcomm Incorporated

- R1-2006840	Correction of RRC parameter name for AP-TRS		Nokia, Nokia Shanghai Bell

- R1-2006841	Maintenance of Rel-16 CSI enhancement	Nokia, Nokia Shanghai Bell

- R1-2006842	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell