3GPP TSG RAN WG1 #102-e			R1-2006984

e-Meeting, August 17th – 28th, 2020

Agenda item:	7.2.6

Source: 	Moderator (Samsung)

Title: 	Text proposal on MU.1 issue for Rel.16 NR_eMIMO MU CSI

Document for:	Discussion and Decision

- Analysis

| # Reasons for change | # Current version of TS 38.214 V16.2.0: - Sec 5.2.2.2.5: ambiguity in the term ‘number of subbands’ - Sec 5.2.3: basis indicator i1,6,l may not be reported, but the description doesn’t indicate so - Table 5.2.2.2.5-4: Typo on a combinatorial coefficient value |
| --- | --- |
| # Summary of changes | # TS 38.214 V16.2.0: - Sec 5.2.2.2.5: added “configured” on ‘number of subbands’ - Sec 5.2.3: added “if reported” after the basis indicator i1,6,l - Table 5.2.2.2.5-4: Change C(14,6) = 4004 to 3003 |
| # Specs/Sections impacted | # TS 38.214 V16.1.0, section 5.2.2.2.5 and 5.2.3 |
| # Consequences if not approved | # The spec is either ambiguous or erroneous (incorrect implementation) |

- Text proposal

TP for TS 38.214 V16.2.0

< Start TP for TS 38.214 V16.2.0> ##### 5.2.2.2.5 Enhanced Type II Codebook < Unchanged parts are omitted > - The parameter R is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-r16. This parameter controls the total number of precoding matrices N3 indicated by the PMI as a function of the number of configured subbands in csi-ReportingBand, the subband size configured by the higher-level parameter subbandSize and of the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2, as follows: < Unchanged parts are omitted > Table 5.2.2.2.5-4: Combinatorial coefficients Cx,y 1; 2; 3; 4; 5; 6; 7; 8; 9. 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 1; 1; 0; 0; 0; 0; 0; 0; 0; 0. 2; 2; 1; 0; 0; 0; 0; 0; 0; 0. 3; 3; 3; 1; 0; 0; 0; 0; 0; 0. 4; 4; 6; 4; 1; 0; 0; 0; 0; 0. 5; 5; 10; 10; 5; 1; 0; 0; 0; 0. 6; 6; 15; 20; 15; 6; 1; 0; 0; 0. 7; 7; 21; 35; 35; 21; 7; 1; 0; 0. 8; 8; 28; 56; 70; 56; 28; 8; 1; 0. 9; 9; 36; 84; 126; 126; 84; 36; 9; 1. 10; 10; 45; 120; 210; 252; 210; 120; 45; 10. 11; 11; 55; 165; 330; 462; 462; 330; 165; 55. 12; 12; 66; 220; 495; 792; 924; 792; 495; 220. 13; 13; 78; 286; 715; 1287; 1716; 1716; 1287; 715. 14; 14; 91; 364; 1001; 2002; 3003; 3432; 3003; 2002. 15; 15; 105; 455; 1365; 3003; 5005; 6435; 6435; 5005. 16; 16; 120; 560; 1820; 4368; 8008; 11440; 12870; 11440. 17; 17; 136; 680; 2380; 6188; 12376; 19448; 24310; 24310. 18; 18; 153; 816; 3060; 8568; 18564; 31824; 43758; 48620. < Unchanged parts are omitted > ### 5.2.3 CSI reporting using PUSCH < Unchanged parts are omitted > - Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυ-KNZ/2 highest priority elements of i1,7,l, i2,3,l, the KNZ/2-υ highest priority elements of i2,4,l and the KNZ/2-υ highest priority elements of i2,5,l (l=1,…,υ). < End TP for TS 38.214 V16.2.0>