3GPP TSG RAN WG1 #102-e		R1-200xxxx

e-Meeting, Aug 17th – 28nd, 2020

Title:	[draft] LS on evaluation methodology for UE power saving enhancements

Response to:

Release:	Release 17

Work Item:	NR_UE_pow_sav_enh

Source:	MediaTek, RAN1

To:	RAN2

Cc:	RAN4

Contact Person:

#### Name: Wei-De Wu

#### E-mail Address:	Weide.Wu@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

In response to WID of Rel-17 UE power saving enhancements, RAN1 is expected to inform RAN2 about the evaluation methodology updates related to paging enhancement(s) as well as the evaluation assumptions related to connected-mode power saving enhancements.

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting - (Omitted) - Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1] - NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized. - (Omitted)

Accordingly, RAN1 would like to inform RAN2 the following related agreements:

- Evaluation methodology updates for paging enhancements:

Agreements:

For study of Rel-17 paging enhancement, the following are assumed as a baseline for FR1 and FR2:

- Reference configuration for FR1/FR2 as specified in Section 8.1.1/8.1.2 of TR 38.840

  - Note: the setting for some PDSCH parameters may not be applicable for RedCap UEs

- Baseline paging cycle length: [1.28] second

- SS burst related assumptions:

  - 20 ms periodicity

  - 2 ms duration for serving cell RRM measurement, which can overlap with the one for synchronization before PO

  - FFS time/frequency tracking

- Measurement related assumptions:

  - 20 ms SMTC periodicity

  - 2 ms SMTC window for intra-frequency RRM measurement, assuming synchronized deployment

  - [5 ms SMTC window and 6 ms measurement gap for inter-frequency RRM measurement]

    - Note: RAN4 requirement assumes one frequency layer per measurement gap, and 0.5 ms is assumed for switch in/out a frequency layer

- Note: the inclusion of potential TRS/CSI-RS occasions can be considered

Agreements:

The following power consumption model for FR1 is utilized for the evaluations of Rel-17 UE power saving enhancements in idle/inactive mode

- FFS: FR2 power consumption model for idle/inactive mode operations

| Power State | Relative Power (FR1 reference from TR 84.840) | Relative Power (Idle/inactive-mode operation with reception bandwidth 20 MHz) |
| --- | --- | --- |
| Deep Sleep (PDS) | 1 | 1 |
| Light Sleep (PLS) | 20 | 20 |
| Micro sleep (PMS) | 45 | 45 |
| PDCCH-only (PPDCCH) | 100 | 50Note |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 300 | 120 |
| PDSCH-only (PPDSCH) | 280 | 112 |
| SSB/CSI-RS proc. (PSSB) | 100 (synchronization or serving cell measurement) | 50 |
| Intra-frequency RRM measurement (Pintra) | · 150 (synchronous case, N=8, measurement only; Pintra, meas-only) · 200 (combined search and measurement; Pintra, search+meas) | · [60] (synchronous case, N=8, measurement only; Pintra, meas-only) · [80] (combined search and measurement; Pintra, search+meas) |
| Inter-frequency RRM measurement (Pinter) | · 150 (measurement only per freq. layer; Pinter, meas-only) · 150 (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer | · [60] (measurement only per freq. layer; Pinter, meas-only) · [150] (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer |
| Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}. |  |  |

Agreements:

Group paging rate of 10% is assumed for the evaluation of Rel-17 paging enhancement

- FFS: Another group paging rate > 10%

- Note: If UE sub-grouping is applied, the sub-group paging rate can be reduced w.r.t. the total sub-group number for a PO

Agreements:

For the study on paging enhancements to reduce unnecessary paging reception, the following metrics are considered:

- UE power saving gain (relative to a given feature or overall)

- Impact to UE paging detection probability

  - FFS: Link level simulation assumptions

- System impact, including

  - Additional resource overhead and its implications

  - Impact to Rel-15/Rel-16 idle/inactive-mode UEs and connected-mode UEs

  - Impact to other legacy functionalities, including SI change and ETWS indication

  - [Note: NW energy consumption evaluation is not precluded]

Agreements:

- For the study of paging enhancement, 1, 2, or 3 SS burst processing is assumed before PO

  - Note: in choosing one or more values (1, 2, or 3) for the evaluations, companies to provide justification

Proposed Agreements:

For the estimation of average power consumption for idle/inactive mode UE over a paging cycle, the following example calculations for FR1 can be referenced. Companies to report the assumed UE processing timeline(s) with justification for the estimation of average power consumption.

- Example 1: 3 SS bursts are utilized before PO (e.g. for low SNR)

| UE operations in a paging cycle | Time duration (ms) | Energy contribution (relative power * ms) |
| --- | --- | --- |
| SSB processing | 2 | PSSB * 2 |
| Light sleep | 18 | PLS * 18 + 100Note1 |
| SSB processing | 2 | PSSB * 2 |
| Light sleep | 18 | PLS * 18 + 100 Note1 |
| SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4]Note2 + Pintra, meas-only * (1 – [1/4] Note2) ) ) * 0.85Note3 * 2 |
| Light sleep | 8 | PLS * 8 + 100 Note1 |
| PO reception | 4 | (PPDCCH * (1 – RG) + PPDCCH+PDSCH * RGNote4) * 4 |
| Light sleep | 6 | PLS * 6 + 100 |
| Switch to another frequency layer | 0.5 | PMS * 0.5 |
| Inter-freq. RRM measurement | 5 | (Pinter, search-only * [1/4]Note2 + Pinter, meas-only * (1 – [1/4] Note2) ) * 5 |
| Switch back to serving frequency | 0.5 | PMS * 0.5 |
| Deep sleep | 1214 | PDS * 1214 + 450Note5 |
| (Total) | 1280 | Total Energy |
| Average power consumption = Total Energy / 1280 |  |  |
| Note 1: Additional transition energy (relative power * ms) for light sleep as specified in Table 19 of TR 38.840 Note 2: Cell search rate for intra/inter-frequency RRM measurement Note 3: Scaling convention to combine two different types of UE operations as in Section 8.1.3 of TR 38.840 Note 4: RG is the paging rate to the UE group Note 5: Additional transition energy (relative power * ms) for deep sleep as specified in Table 19 of TR 38.840 |  |  |

- Example 2: 2 SS bursts are utilized before PO (e.g. for medium SNR)

| UE operations in a paging cycle | Time duration (ms) | Energy contribution (relative power * ms) |
| --- | --- | --- |
| SSB processing | 2 | PSSB * 2 |
| Light sleep | 18 | PLS * 18 + 100 Note1 |
| SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4]Note2 + Pintra, meas-only * (1 – [1/4] Note2) ) ) * 0.85Note3 * 2 |
| Light sleep | 8 | PLS * 8 + 100 Note1 |
| PO reception | 4 | (PPDCCH * (1 – RG) + PPDCCH+PDSCH * RGNote4) * 4 |
| Light sleep | 6 | PLS * 6 + 100 |
| Switch to another frequency layer | 0.5 | PMS * 0.5 |
| Inter-freq. RRM measurement | 5 | (Pinter, search-only * [1/4]Note2 + Pinter, meas-only * (1 – [1/4] Note2) ) * 5 |
| Switch back to serving frequency | 0.5 | PMS * 0.5 |
| Deep sleep | 1254 | PDS * 1254 + 450Note5 |
| (Total) | 1280 | Total Energy |
| Average power consumption = Total Energy / 1280 |  |  |
| Note 1: Additional transition energy (relative power * ms) for light sleep as specified in Table 19 of TR 38.840 Note 2: Cell search rate for intra/inter-frequency RRM measurement Note 3: Scaling convention to combine two different types of UE operations as in Section 8.1.3 of TR 38.840 Note 4: RG is the paging rate to the UE group Note 5: Additional transition energy (relative power * ms) for deep sleep as specified in Table 19 of TR 38.840 |  |  |

- Example 2: 1 SS burst is utilized before PO (e.g. for high SNR)

| UE operations in a paging cycle | Time duration (ms) | Energy contribution (relative power * ms) |
| --- | --- | --- |
| SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4]Note1 + Pintra, meas-only * (1 – [1/4] Note1) ) ) * 0.85Note2 * 2 |
| Light sleep | 8 | PLS * 8 + 100 Note3 |
| PO reception | 4 | (PPDCCH * (1 – RG) + PPDCCH+PDSCH * RGNote4) * 4 |
| Deep sleep | 1266 | PDS * 1266 + 450Note5 |
| (Total) | 1280 | Total Energy |
| Average power consumption = Total Energy / 1280 |  |  |
| Note 1: Cell search rate for intra/inter-frequency RRM measurement Note 2: Scaling convention to combine two different types of UE operations as in Section 8.1.3 of TR 38.840 Note 3: Additional transition energy (relative power * ms) for light sleep as specified in Table 19 of TR 38.840 Note 4: RG is the paging rate to the UE group Note 5: Additional transition energy (relative power * ms) for deep sleep as specified in Table 19 of TR 38.840 |  |  |

- Evaluation assumptions for connected-mode power saving enhancements:

Agreements:

- Reusing power model in TR38.840 for evaluation of DCI-based power saving adaptation schemes.

  - Note: company reporting additional power model for missing state or update is not precluded.

Agreements:

- Company should report assumptions used for periodic measurement activities for the Rel-17 DCI-based power saving adaptation evaluation.

  - The periodic activities defined in TR38.840 can be reused.

  - Measurement for RLM/BFD every C-DRX cycle can be optionally modelled

Proposed Agreements:

- The performance metrics described in TR38.840 section 8.2 is reused for power saving evaluation of Rel-17 DCI-based power saving adaptation during ActiveTime.

- The following Rel-15 / 16 features is recommended of the power consumption as reference for baseline. Company can report the feature combinations being used in the analysis.

  - DRX

    - C-DRX cycle 40msec for VoIP

      - 10ms IAT, 8ms On-duration

      - Assume max two packets bundled

    - C-DRX cycle 160msec for FTP

      - Alt 1: 20 msec IAT, 8ms On-duration

      - Alt 2: short DRX

        - 20 ms IAT, 8ms On-duration

        - 20 ms for short DRX cycle, 4 cycles

    - C-DRX cycle 40ms for [‘frequent data traffic’ / ’optional traffic model’] if defined in Rel-17

      - 10ms IAT, 8ms On-duration

    - Note: Other settings are not precluded and up to company to report

  - DCP for DRX adaptation,

    - DCP offset  to DRX ON = 2 ms, other values are not precluded

  - Cross-slot scheduling adaptation

    - Minimum K0 can be adapted from 0 to 1 for FR1, 0 to [4] for FR2

  - BWP switching, including

    - MIMO layer adaptation,

      - Max # of MIMO layer can be adapted from 4 layer to 2 layer for FR1, 2 layer to 1 layer for FR2

    - PDCCH monitoring period adaptation

      - PDCCH monitoring period can be adapted from per slot monitoring to X slot monitoring

        - X = [2] for FR1 and [8] for FR2

    - Bandwidth adaptation

      - Bandwidth can be adapted from 100MHz to 20MHz for FR1, FFS for FR2

    - Note:

      - BWP transition time type 2 is assumed, BWP transition duration is

        - 5 slot @ 30kHz SCS for FR1,

        - 18 slot@120kHz SCS for FR2

        - the slot-average power level for BWP transition duration is according to TR38.840

        - BWP transition time type 1 can be optional modelled

      - Whether BWP switching is modeled depends on the assumed UE capability and evaluated schemes.

  - Scell dormancy assumption for CA capable UEs

    - FR1 & FR2: SCell dormancy with [160 ms] periodic CSI measurement and reporting

- Other settings

  - CA assumption if configured for CA capable UEs

    - For FR1, FFS

    - For FR2, 4*100MHz can be considered.

  - Assumptions for scheduler

    - For FR1, no restriction on the beam assumptions being used in each slot

    - For FR2, gNB equally schedule the slots for UEs targeting to different beams.

    - Baseline indication delay from last data scheduling to power saving indication sent by the gNodeB

      - BWP switching

        - BWP switching indication is Y (ms) after last PDSCH of a packet/data burst

          - Y = [8], other values are not precluded

    - Note: the assumptions does not necessary mean to restrict or precluded any implementation. Other assumptions are not precluded and can be reported by companies.

2. Actions:

To RAN2:

RAN1 respectfully asks RAN2 to take the evaluation methodology for paging enhancements into account in the corresponding RAN2 study. For the connected-mode evaluation assumptions, if there is significant concern, please reply RAN1 with the suggested change(s).

3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting 103-e 	26 October – 13 November 2020		e-Meeting

TSG-RAN WG1 Meeting 104	01 – 05 March 2021 	 		Athens, Greece