3GPP TSG-RAN WG1 #103-e	R1-2007797

e-Meeting, October 26th – November 13th, 2020

Title:	Draft Reply LS on Intra UE Prioritization Scenario

Response to:	R1-2007523 (R2-2008599)

Release:	Rel-16

Work Item:	NR_IIOT-Core

Source:	CATT [RAN WG1]

To:	RAN WG2

Contact Person:

#### Name:	Lei Zhou

E-mail Address:	zhoulei@catt.cn

- Overall Description:

RAN1 thanks RAN2 for the LS [1] and would like to provide the following responses.

PHY behaviour in the collision scenario between CG and DG with same/different PHY-priority index, and only one transport block is delivered to PHY is consistent with RAN2’s understanding, i.e. PHY transmit on the grant for which a transport block is delivered and skip the transmission on the other grant, at least when there is no UCI to be multiplexed in the PUSCH.

If there is UCI to be multiplexed in a PUSCH in the overlapping PUSCHs, RAN2 needs to consider the potential problem of UCI dropping if MAC does not deliver a MAC PDU to PHY for the PUSCH which is used for UCI multiplexing. To be more specific, for DG and CG with same L1 priority, if MAC delivers CG PDU to PHY when UCI is supposed to be transmitted on DG PDU, UCI is dropped. For CG-CG/DG with different L1 priorities, if UCI is supposed to be multiplexed in the HP CG/DG but there is no data to be transmitted on the HP CG/DG (i.e. CG/DG is skipped), the HP UCI is dropped.

2. Actions:

To RAN2

ACTION: RAN1 respectfully asks RAN2 to take the above potential problem into account on Intra UE Prioritization Scenario.

3. Date of Next TSG WG RAN1 Meetings:

TSG-RAN WG1 Meeting #104e	                                      25th January – 05th February  2021 e-meeting.

TSG-RAN WG1 Meeting #104bis-e 	                         12th April – 20th April  2021  e-meeting.

4. Reference:

[1] R2-2008599, LS on Intra UE Prioritization Scenario